3GPP TSG-RAN WG4 Meeting #92bis	R4-1912600

Chongqing, China, 14 October – 18 October 2019

Source:	Ericsson, T-Mobile US

Title:	TP to TR 37.716.21-21: Addition of DC configurations for DC_2_n41-n71

Agenda item:	9.8.2

Document for:	Approval

# Background

This text proposal for TR 37.716-21-21 v0.7.0 to add DC_2_n41-n71 as defined in WID [1].

# Text Proposal

##### ---Start of changes---

## 6.x	DC_2_n41-n71

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n41-n71 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n41A-n71A | DC_2A_n41A DC_2A_n71A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 140 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n71 | 30 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |

### 6.x.3	Co-existence studies

As can be seen in the co-existence analysis for DC_2_n41 in TR [2] shows that there are IMD2 and IMD5 impact from DC_2_n41 UL to Band n71 DL.

As can be seen in the co-existence analysis for DC_2_n71 in TR [3] shows that there IMD2 impact from DC_2_n71 UL to Band n41 DL.

### 6.x.4	∆TIB and ∆RIB values

For DC_2_n41-n71 the TIB,c and RIB,c values are derived from LTE combinations CA_2-7-12, CA_2-7-28, CA_3-7-28 and DC_3-7_n28 and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n41-n71 | 2 | 0.5 |
|  | n41 | 0.5 |
|  | n71 | 0.3 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n41-n71 | 2 | 0 |
|  | n41 | 0 |
|  | n71 | 0 |

### 6.x.5	MSD

MSD values for UL 2_n41 into DL n71 and UL 2_n71 into DL n41 are needed. Below table is to be captured in Table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.x.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n41A-n71A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | N/A |
|  | n71 | 676 | 5 | 50 | 630 | 28.7 | IMD2 |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | N/A |
|  | n71 | 686 | 5 | 50 | 640 | 3.8 | IMD5 |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2586 | 10 | 50 | 2586 | 29.2 | IMD2 |
|  | n71 | 686 | 5 | 50 | 640 | N/A | N/A |

##### ---End of changes---

# Reference

[1]		RP-191744, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics

[2]		TR 37.716-11-11, “Dual Connectivity (EN-DC) of 1 band LTE (1 Down Link (DL) / 1 Up Link (UL)) and 1 NR band (1 DL / 1 UL)(1DL/1UL) Rel-16”

[3]		TR 37.863-01-01 “Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band Rel-15”