3GPP TSG-RAN WG4 Meeting #92BIS                                                        R4-1912873

Chongqing, China, 14th – 18th Oct, 2019

Title:	Reply LS on UL-SL prioritization for NR V2X operation

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

Name:	Suhwan Lim

E-mail Address:	suhwan.lim@lge.com

Attachments:	None

1. Overall Description:

In RAN2 #107 meeting, RAN2 send LS to RAN4 on the UL-SL prioritization for NR V2X service. They are agreements and questions are captured as follow

1: 	(To be confirmed by RAN1/4) RAN2 work on NR-UL/NR-SL prioritization at least for two scenarios: 1) when UL TX overlaps in time domain with SL TX in the shared/same carrier frequency, and 2) when UL TX and SL TX (in different carrier frequency) share TX chains and power budget.

2:	(To be confirmed by RAN1/4) RAN2 work on LTE-UL/NR-SL and LTE-SL/NR-UL prioritization at least for scenario when UL TX and SL TX (in different carrier frequency) share TX chains and power budget.

Furthermore, RAN2 would like to ask RAN1/RAN4 the following questions:

Q1: For the two scenarios agreed by RAN2 for NR-UL/NR-SL prioritization (i.e., 1) when UL TX overlaps in time domain with SL TX in the shared/same carrier frequency, and 2) when UL TX and SL TX (in different carrier frequency) share TX chains and power budget), are they valid scenarios for prioritization from RAN1/4 perspective?

Q2: For the second scenario agreed by RAN2 for LTE-UL/NR-SL and LTE-SL/NR-UL prioritization, (i.e., when UL TX and SL TX (in different carrier frequency) share TX chains and power budget), is it a valid scenario for prioritization from RAN1/4 perspective?

Q3: Additionally, for LTE-UL/NR-SL and LTE-SL/NR-UL prioritization, is the scenario of “UL TX overlaps in time domain with SL TX in the shared/same carrier frequency” valid or not from RAN1/4 perspective? Please note that RAN2 raise a similar question in R2-1911680, but for another issue, i.e., cross-RAT sidelink configuration.

Q4: Till now, the RAN2 conclusion on UL/SL prioritization is limited to the prioritization between MCG UL and MCG SL. Besides that, from RAN1/4 perspective, is there a need to separately consider SCG UL and MCG SL prioritization, e.g., for the scenario of “when UL TX overlaps in time domain with SL TX in the shared/same carrier frequency” and/or “when UL TX and SL TX (in different carrier frequency) share TX chains and power budget”? Q4 includes the following scenarios:

- SCG NR-UL and NR-SL under control of MCG;

- SCG NR-UL and LTE-SL under control of MCG;

- SCG LTE-UL and NR-SL under control of MCG;

In RAN4 #92BIS meeting, RAN4 discussed the coexistence evaluation results for V2X SL operation in licensed band (e.g. 3.5GHz for TDD, 2GHz for FDD). Also RAN4 addressed candidate RF architecture for con-current operation between Uu uplink transmission and V2X SL operation.

According to the RAN4 discussion, RAN4 would like to answer to RAN2 as follow;

Q1: For the two scenarios agreed by RAN2 for NR-UL/NR-SL prioritization (i.e., 1) when UL TX overlaps in time domain with SL TX in the shared/same carrier frequency, and 2) when UL TX and SL TX (in different carrier frequency) share TX chains and power budget), are they valid scenarios for prioritization from RAN1/4 perspective?

A1: Currently RAN4 has not concluded the coexistence evaluation results in SL operation using licensed bands. If the coexistence evaluation results indicate feasibility, then both scenarios for UL/SL prioritization are not precluded.

Q2: For the second scenario agreed by RAN2 for LTE-UL/NR-SL and LTE-SL/NR-UL prioritization, (i.e., when UL TX and SL TX (in different carrier frequency) share TX chains and power budget), is it a valid scenario for prioritization from RAN1/4 perspective?

A2: Scenarios with UL TX and SL TX in different carrier frequency would be feasible that LTE-UL/NR-SL is deployed in adjacent carrier, if the coexistence evaluation results indicate feasibility. In case, RAN4 assume the shared Tx chains but power budget is independently operated. Also LTE-SL/NR-UL is not valid under licensed band since LTE-SL is only allowed in ITS band in LTE V2X.

Q3: Additionally, for LTE-UL/NR-SL and LTE-SL/NR-UL prioritization, is the scenario of “UL TX overlaps in time domain with SL TX in the shared/same carrier frequency” valid or not from RAN1/4 perspective? Please note that RAN2 raise a similar question in R2-1911680, but for another issue, i.e., cross-RAT sidelink configuration.

A3: Scenarios with UL TX and SL TX in the shared/same carrier frequency is one of use case in licensed band since operator has restricted operating frequency. Also RAN4 still ongoing discussion the coexistence evaluation of NR SL operation in licensed bands. The scenario of LTE-SL/NR-UL prioritization in the shared/same carrier frequency can be excluded, while LTE-UL/NR-SL prioritization is still a valid scenario if RAN4 allow the NR SL operation in licensed bands.

Q4: Till now, the RAN2 conclusion on UL/SL prioritization is limited to the prioritization between MCG UL and MCG SL. Besides that, from RAN1/4 perspective, is there a need to separately consider SCG UL and MCG SL prioritization, e.g., for the scenario of “when UL TX overlaps in time domain with SL TX in the shared/same carrier frequency” and/or “when UL TX and SL TX (in different carrier frequency) share TX chains and power budget”? Q4 includes the following scenarios:

- SCG NR-UL and NR-SL under control of MCG;

- SCG NR-UL and LTE-SL under control of MCG;

- SCG LTE-UL and NR-SL under control of MCG;

A4: The 1st and 3rd cell group combination are valid. The 2nd combination is invalid since LTE-SL is not permitted in licensed band in LTE V2X. Regarding MR-DC scenarios with UL/SL, the answers for Q1 to Q3 are applicable as well.

2. Actions:

To RAN WG42

RAN4 respectfully requests RAN2 to take the above information into consideration in their future work.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #93                      November 18 – 22, 2019    		Reno, USA

TSG-RAN WG4 Meeting #94                      February   24 – 28, 2020      	             Athens, GREECE