3GPP TSG RAN WG4 Meeting #93	R4-1913106

Reno, USA, November 18th – 22nd, 2019

3GPP TSG RAN WG1 Meeting #98bis						R1-1911718

Chongqing, China, October 14th – 20th, 2019

Title:	LS on NR V2X synchronization procedures

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN1

To:	RAN2

Cc:	RAN4

Contact Person:

#### Name:	Teng Ma

#### E-mail Address:	mateng@catt.cn

Attachments:	None

1. Overall Description:

In NR V2X synchronization discussion, RAN1 reached the following agreements:

Agreements: - When two or more UE synchronization sources have same priority, S-SSB RSRP is used for the selection of the synchronization source. Agreements: - The procedure for signalling, identifying priority for one or more synchronization references and selecting the synchronization reference from the LTE is re-used (as a working assumption) for NR SL - FFS SSIDs used for each priority - FFS other potential impacts due to P3/P4/P5 - FFS whether there is an issue with prioritization among references of the same priority Agreements: 672 SL-SSIDs are divided into 2 sets to indicate different synchronization priorities following a similar approach as in LTE-V2X: - Set id_net {0, 1, …, 335} - Set id_oon{336, 337, 338, …, 671} - The usage of 0 is the same as 0 as in LTE - The usage of 336 is the same as 168 as in LTE - The usage of 337 is the same as 169 as in LTE Agreements: - Do not support 2/4/8 as the number of S-SSB transmissions within one S-SSB period for 15/30/60 KHz SCS for FR1, respectively.

2. Actions:

To RAN WG2:

RAN1 respectfully requests RAN2 to take the above agreement into consideration in their future work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #99                      November 18 – 22, 2019    		Reno, USA

TSG-RAN WG1 Meeting #100                    February 24 – 28, 2020    		Athens, Greece