3GPP TSG-RAN WG4 Meeting #93                                                                                             R4-1913268

Reno, Nevada, USA, 18th – 22nd Nov, 2019

Agenda item:            9.17.2.2.4

Source:	MediaTek Inc.

Title:	PDSCH simulation results for high-speed TDL channels

Document for:          Discussion

# 1	Introduction

It was agreed in RAN4#92bis to further study the options regarding to following aspects:

- Maximum Doppler frequency;

- DMRS configuration between 1+2 and 1+3 for multipath fading channel

In this paper, we present the simulation results of PDSCH for NR high-speed TDL channels and provide our observations and proposals for the test case setting.

# 2	Discussion on NR high-speed TDL test case setting

2.1 Maximum Doppler frequency

Based on the simulation assumption in [1], in the following we provide the corresponding simulation results. Table 2.1 and Table 2.2 present the required SNR (in dB) at 70% throughput in TDL-C channel for FDD and TDD cases, respectively. Table A.1 (in Appendix) contains detailed parameter setting in the simulation. From our results, 70% throughput can be achieved with a reasonable SNR under the assumption that maximum Doppler frequency is 600Hz and 1200Hz for 15kHz SCS and 30kHz SCS, respectively. Therefore, we agree to support these Doppler options in the TDL-C channel.

Proposal #1: Agree to assume 600Hz maximum Doppler frequency for 15kHz SCS and 1200Hz maximum Doppler frequency for 30kHz SCS.

Table 2.1 Required SNR (dB) @ 70% Tput for TDL-C FDD cases

| Doppler shift | MCS | 2x2 |  | 2x4 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | AddPos2 | AddPos3 | AddPos2 | AddPos3 |
| Rank 1 | 4 | -1.25 | -1.4 | -4.21 | -4.39 |
|  | 13 | 7.04 | 7.08 | 3.33 | 3.37 |
|  | 17 | 10.74 | 10.19 | 6.79 | 6.38 |
| Rank 2 | 4 | 3.18 | 2.97 | -0.54 | -0.78 |
|  | 13 | 14.18 | 14.07 | 8.2 | 8.13 |
|  | 17 | 20.01 | 19.6 | 12.19 | 11.95 |

Table 2.2 Required SNR (dB) @ 70% Tput for TDL-C TDD cases

| Doppler shift | MCS | 2x2 |  | 2x4 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | AddPos2 | AddPos3 | AddPos2 | AddPos3 |
| Rank 1 | 4 | -1.04 | -1.16 | -4.02 | -4.17 |
|  | 13 | 7.62 | 7.41 | 3.77 | 3.62 |
|  | 17 | 10.85 | 10.86 | 6.85 | 6.9 |
| Rank 2 | 4 | 3.98 | 3.81 | 0.09 | -0.09 |
|  | 13 | 15.13 | 14.83 | 8.88 | 8.67 |
|  | 17 | 20.15 | 20.07 | 12.51 | 12.46 |

2.2 DMRS symbol number

We analyze the impact of different DMRS symbol numbers on demodulation performance. Figure 2.1 shows the BLER and Tput curve with different DMRS number under TDD cases with maximum Doppler shift = 1200Hz, 2x4 antenna configuration and rank 2. From the BLER curves, 3 additional DMRS has 0.25dB SNR gain in BLER compared to 2 additional DMRS due to more accurate channel estimation results. However, 3 additional DMRS configuration increases the pilot overhead and thus reduce the throughput accordingly. Considering the complexity of UE demodulation, 2 additional DMRS is enough to handle the high Doppler spread and should be more practical for HST scenario.

Proposal #2: Use 2 additional PDSCH DMRS symbols (Option 1: 1+1+1) for high-speed TDL channel test

Figure 2.1 TDL-C performance with different DMRS number

# 3	Conclusions

In the contribution, we provide our simulation results of PDSCH HST tests and following proposals:

Proposal #1: Agree to assume 600Hz maximum Doppler frequency for 15kHz SCS and 1200Hz maximum Doppler frequency for 30kHz SCS.

Proposal #2: Use 2 additional PDSCH DMRS symbols (Option 1: 1+1+1) for multipath fading channel test

# 4	Reference

[1] R4-1912808, “WF on UE demodulation for NR HST”, CMCC.

Appendix

Table A.1 Simulation assumption for multi-path fading channel

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 2x2; 2x4 |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | Option1: DMRS 1+1+1 Option2: DMRS 1+1+1+1 |  |
| MCS | MCS 4; MCS 13; MCS 17 based on 64QAM table |  |
| TDD pattern: | 7D1S2U, S: 6D 4G 4U |  |
| Propagation condition | TDL-C 300ns |  |
| TRS periodicity | 10ms, 2 slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds= 300m, Dmin=2m |  |
| Rank | Rank = 1 and 2 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | 600Hz | 1200Hz |
| Testing metric | SNR @70% of maximum throughput |  |