3GPP TSG-RAN WG4 Meeting #93                                                              R4-1913328

Reno, USA, 18th-22th Nov, 2019

Source:	Samsung, Verizon

Title:	TP for TR 36.716-03-02: CA_3BDL_46-48-66_2UL_48A-66A

Agenda item:			8.6.3

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 36.716-03-02 to include CA_3BDL_46-48-66_2UL_48A-66A according to the request in [1].

# 2. Reference

- RP-1911428, Revised WID on Rel-16 LTE inter-band Carrier Aggregation for x bands DL (x= 3, 4, 5) with 2 bands UL.

# 3. Text Proposal

<Start of Text Proposal>

## 6.x LTE-A inter-band CA: Band 46 and Band 48 and Band 66 DL with 2 bands UL

### 6.x.1	List of specific combination issues

#### 6.x.1.1	Channel bandwidth per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_46A-48A-66A | CA_48A-66A | 46 |  |  |  |  |  | Yes | 60 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_46A-48C-66A | CA_48A-66A | 46 |  |  |  |  |  | Yes | 80 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_46C-48A-66A | CA_48A-66A | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_46C-48C-66A | CA_48A-66A | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_46D-48A-66A | CA_48A-66A | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 100 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_46D-48C-66A | CA_48A-66A | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 120 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |

#### 6.x.1.2	Co-existence studies for LTE-A inter-band UL CA_48A-66A and DL CA_46-48-66

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1.

Table 6.x.1.2-1: Co-existence study for UL CA_48A-66A and DL CA_46-48-66

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3550 | 3700 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 | 5130 | 5340 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1770 | 1990 | 5260 | 5480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5320 | 5690 | 10 | 280 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8810 | 9180 | 6970 | 7260 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 8870 | 9390 | 1430 | 1790 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3540 | 3980 | 10520 | 10960 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 12360 | 12880 | 8680 | 9040 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 3140 | 3570 | 12420 | 13090 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1760 | 2270 | 7090 | 7680 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10390 | 10820 | 15910 | 16580 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12230 | 12740 | 14070 | 14660 |

Table 6.x.1.2-1shows that:

The impact of the harmonic and the IMD of UL CA_48A-66A to

Band46:

3rd harmonic from the band 66 can impact the band 46 and this band is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. Therefore, there is no need to study for this harmonic problem.

2nd and 3rd order IMD products by band 48 and band 66 falls into the own Rx frequency band 46, and it need to define exclusion zone.

Band 48:

2nd harmonic from band 66 can impact band 48 and its Reference sensitivity exception has been specified in Table 7.3.1A-0a of TS36.101.

No IMD impact, since band 48 is a TDD band.

Band 66:

No harmonic issue.

5th order IMD product by band 48 and band 66 falls into the own Rx frequency band 66, and the MSD has been defined in the case of 2DL/2UL CA_48A-66A.

#### 6.x.1.3	MSD

For band combinations including band 46, there is no need to study for MSD since the requirements are already specified in Table 7.3.1A-0eA: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49) and when band 46 have self-interference problems by operating dual uplink CA, then these requirements do not need to apply in exclusion zone which is frequency range within (harmonics frequency region + delta FHD in Table 7.3.1A-0eC) and IMD frequency region as below.

6.x.1.3-1: IMD frequency range for UL CA_48A-66A and DL CA_46-48-66

| DL_CA configuration | UL_CA configuration | Exclusion zone center frequency | Exclusion zone BW |
| --- | --- | --- | --- |
| CA_46A-48A-66A CA_46A-48C-66A CA_46C-48A-66A CA_46C-48C-66A CA_46D-48A-66A CA_46D-48C-66A | CA_48A-66A | fc_48A + fc_66A | BW_48A + BW_66A |
| CA_46A-48A-66A CA_46A-48C-66A CA_46C-48A-66A CA_46C-48C-66A CA_46D-48A-66A CA_46D-48C-66A | CA_48A-66A | 2*fc_48A - fc_66A | 2*BW_48A - BW_66A |

#### 6.x.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 can be applied.

<End of Text Proposal>