3GPP TSG-RAN WG4 Meeting #93                                            R4-1913336

Reno,USA, 18th – 22th Nov, 2019

Agenda Item:   9.5.3

Source:        Samsung

Title:          TP for TS38.174: IAB system parameters

Document for:  Approval

# Introduction

According to agreement in [1] and further analysis shown in [2] for IAB system parameters, this contribution provide the TP for TS38.174 on IAB system parameters.

# Reference

[1] R4-1912890, WF on IAB system parameter

[2] R4-191xxxx, Further discussion on IAB system parameter

# TP for IAB TS 38.174

<Start of the TP>

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios "

[5] 	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”

<Unchanged clause skipped>

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and IAB channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and IAB channel bandwidths may be considered in future releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which IAB can operate according to the present version of the specification are identified as described in table 5.1-1 in [5].

## 5.2	Operating bands

NR IAB is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2.

Table 5.2-1 NR IAB operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex Mode |
| --- | --- | --- | --- |
| n41 | 2469 MHz – 2690 MHz | 2469 MHz – 2690 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

Table 5.2-2: NR IAB operating bands in FR2

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex Mode |
| --- | --- | --- |
| n257 | 26500 MHz – 29500 MHz | TDD |
| n258 | 24250 MHz – 27500 MHz | TDD |
| n260 | 37000 MHz – 40000 MHz | TDD |
| n261 | 27500 MHz – 28350 MHz | TDD |

## 5.3	Channel bandwidth

### 5.3.1	General

The requirement specified in 5.3.1 of TS38.104 [5] applies for IAB-DU. And the requirement specified in 5.3.1 of TS38.101-1[2] applies for IAB-MT.

### 5.3.2	Transmission bandwidth configuration

The requirement specified in 5.3.2 of TS38.104 [5] applies for IAB.

### 5.3.3	Minimum guardband and transmission bandwidth configuration

The requirement specified in 5.3.3 of TS38.104 [5] applies for IAB.

### 5.3.4	RB alignment

The requirement specified in 5.3.4 of TS38.104 [5] applies for IAB-DU. And the requirement specified in 5.3.4 of TS38.101-1[2] applies for IAB-MT for NR FR1. The requirement specified in 5.3.4 of TS38.101-2[3] applies for IAB-MT for NR FR2.

### 5.3.5	IAB channel bandwidth per operating band

The requirements in this specification apply to the combination of IAB channel bandwidths, SCS and operating bands shown in table 5.3.5-1 for FR1 and in table 5.3.5-2 for FR2. For IAB-DU, the transmission bandwidth configuration in 5.3.2 shall be supported for each of the IAB channel bandwidths within the IAB-DU capability. For IAB-MT, the transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths. The IAB channel bandwidths are specified for both the Tx and Rx path.

Table 5.3.5-1: IAB channel bandwidths and SCS per operating band in FR1

| NR band / SCS / IAB channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 302 MHz | 40 MHz | 50 MHz | 60 MHz | 702 MHz | 80 MHz | 901 MHz | 100 MHz |
| n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |
|  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |
| NOTE 1: This IAB channel bandwidth is optional for IAB-MT in this release of the specification. NOTE 2: This IAB channel bandwidth applies for IAB-DU only. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3.5-2: IAB channel bandwidths and SCS per operating band in FR2

| NR band / SCS / IAB channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 50 MHz | 100 MHz | 200 MHz | 4001 MHz |
| n257 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n258 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n260 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n261 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| NOTE 1: This IAB channel bandwidth is optional for IAB-MT in this release of the specification. |  |  |  |  |  |

## 5.4	Channel arrangement

The requirements specified in 5.4 of TS38.104 [5] apply for IAB.

<End of the TP>