3GPP TSG-RAN WG4 Meeting #93            	R4-1913371

Reno, USA, 18th –22th Nov, 2019

Agenda item:	9.18.1.3

Source:	Samsung

Title:	Simulation results of UE demodulation requirements for LTE NR co-existence scenario in TDD

Document for:	Discussion

# 1	Introduction

In RAN4#92b meeting, the way forward was approved regarding to the simulation assumption for LTE-NR coexistence test [1].

- Number of tests

  - Test 1: Mapping Type A, Start symbol 3, Duration 9

  - Test 2: Mapping Type A, Start symbol 3, Duration 11

- Test applicability rule:

  - The test with duration 11 is applied only when UE supports additionalDMRS-DL-Alt

  - If UE supports alternative DMRS configuration then it can pass requirements only for scenario with PDSCH duration 11 and skip testing for scenario with PDSCH duration 9

- SCS for NR carrier: 15 kHz

- TDD UL/DL configuration

  - LTE: LTE UL-DL configuration 2, special subframe configuration 7

  - NR: DDDSU with S=10:2:2

- PDSCH scheduling

  - Skipped slots with NR SSB, LTE PBCH/PSS/SSS and special slots

  - PDSCH is scheduled in NR slots {1, 5, 6, 10, 11, 15, 16} every 20ms

In this paper, we provide our initial simulation results for alignment purpose.

# 2	Simulation results

Based the simulation assumption, we can conclude the slot structures for both normal downlink and special shown as Figure 2-1 and Figure 2-2.

Figure 2-1. Structure for special slot		 Figure 2-2. Structure for normal downlink slot

The initial simulation results was provided as the following figures

Figure 2-3 simulation results for Test1

Figure 2-4 simulation results for Test2

The related requirement for targeting SNR with 70% of TP are provided for two test cases, respectively

Table 2-1 simulation results of Test1 for LTE-NR co-existence for TDD

| Test1 | SNR@70% max throughput |
| --- | --- |
| 4Tx, 2Rx ULA Low | -3.41 |
| 4Tx, 4Rx ULA Low | -5.87 |

Table 2-2 simulation results of Test1 for LTE-NR co-existence for TDD

| Test2 | SNR@70% max throughput |
| --- | --- |
| 4Tx, 2Rx ULA Low | -3.56 |
| 4Tx, 4Rx ULA Low | -5.25 |

# 3	Conclusion

In this contrition, based on the agreement of WF on LTE NR co-existence scenario in TDD, the initial simulation results are provided.

# References

R4-1912711 “Way forward on LTE-NR coexistence for TDD” 3GPP RAN4 92bis

# Appendix

Simulation assumption summary

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| CHB/SCS |  |  | CBW:10MHz; SCS: 15kHz |
| Channel Model |  |  | TDLA30-10 |
| TDD configuration |  |  | - LTE: LTE UL-DL configuration 2, Special subframe configuration 7 - NR with 15kHz SCS: DDDSU with S=10:2:2 |
| SSB configuration |  |  | Periodicity 20 ms, allocated in first slot within 20 ms |
| TRS configuration for NR TDD configuration Option 1 |  |  | 2 slots, offset 10 ms, perioicity 20 ms |
| MCS |  |  | MCS 4 (QPSK, CR 0.30) |
| Antenna configuration and Rank |  |  | 4x2 and 4x4 ULA Low Rank 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | Starting symbol (S) |  | 3 |
|  | Length (L) |  | 9 and 11 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling size |  | 2 |
|  | PDSCH scheduling |  | Skipped slots with NR SSB, LTE PBCH/PSS/SSS and special slots: NR slots {1, 5, 6, 10, 11, 15, 16} every 20ms |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Position of the first DM-RS for downlink |  | 3 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CRS for rate matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| Number of HARQ Processes |  |  | 8 |
| Note 1: No MBSFN is configured on LTE carrier |  |  |  |

FRC

| Parameter | Unit | Values |  |
| --- | --- | --- | --- |
| Reference channel |  | Test 1 | Test 2 |
| Channel bandwidth | MHz | 10 | 10 |
| Subcarrier spacing | kHz | 15 | 15 |
| Number of allocated resource blocks | PRBs | 52 | 52 |
| Number of consecutive PDSCH symbols |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1} for i from {0,…,19} |  | 9 | 11 |
| Allocated slots per 2 frames | Slots | 7 | 7 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 4 | 4 |
| Modulation |  | QPSK | QPSK |
| Target Coding Rate |  | 0.30 | 0.30 |
| Number of MIMO layers |  | 1 | 1 |
| Number of DMRS REs |  | 12 | 12 |
| Overhead for TBS determination |  | 18 | 18 |
| Information Bit Payload per Slot |  |  |  |
| For Slots i = 0 | Bits | N/A | N/A |
| For Slots i, if mod(i, 5) = {2,3,4} for i from {1,…,19} | Bits | N/A | N/A |
| For Slots i, if mod(i, 5) = {0,1} for i from {1,…,19} | Bits | 2472 | 3240 |
| Transport block CRC per Slot |  |  |  |
| For Slots i = 0 | Bits | N/A | N/A |
| For Slots i, if mod(i, 5) = {2,3,4} for i from {1,…,19} | Bits | N/A | N/A |
| For Slots i, if mod(i, 5) = {0,1} for i from {0,…,19} | Bits | 16 | 16 |
| Number of Code Blocks per Slot |  |  |  |
| For Slots i=0 | CBs | N/A | N/A |
| For Slots i, if mod(i, 5) = {2,3,4} for i from {1,…,19} | CBs | N/A | N/A |
| For Slots i, if mod(i, 5) = {0,1} for i from {1,…,19} | CBs | 1 | 1 |
| Binary Channel Bits Per Slot |  |  |  |
| For Slots i=0 | Bits | N/A | N/A |
| For Slots i, if mod(i, 5) = {2,3,4} for i from {1,…,19} | Bits | N/A | N/A |
| For Slots i = 10, 11 | Bits | 7760 | 10256 |
| For Slots i, if mod(i, 5) = {0,1} for i from {1,…,9,12,…,19} | Bits | 8384 | 10880 |
| Max. Throughput averaged over 2 frames | Mbps | 0.865 | 1.134 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: No user data is scheduled on slots with LTE PBCH/PSS/SSS |  |  |  |