3GPP TSG-RAN4 Meeting #93	R4-1913384

# Reno, US, 18th - 22th Nov, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR |  | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on NR SRS carrier switching interruption in TS 36.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Enh_Core |  |  |  |  |  | Date: |  |  | 2019-10-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is agreed inR4-1912838 at RAN4#92bis - Specification impact - NR SRS carrier switching impacting LTE CC (in 36.133) - EN-DC: LTE PCC and LTE SCCs - NE-DC: LTE PSCC and LTE SCCs |  |  |  |  |  |  |  |  |
| Summary of change: |  | Interruptions at NR SRS carrier switching in EN-DC and NE-DC are specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.32, 7.36 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

7.32	Interruptions with EN-DC

7.32.1	Introduction

This section contains the requirements related to the interruptions on PCell, and MCG SCell when

NR PSCell is added or released, or

transitions between active and non-active during NR PSCell DRX, or

transitions from NR PSCell non-DRX to DRX, or

SCell in either E-UTRA MCG or NR SCG is added or released, or

SCell in either E-UTRA MCG or NR SCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either E-UTRA MCG or NR SCG, or

a downlink bandwidth part (BWP) and/or an uplink BWP is switched in NR PSCell or in any NR SCell, or

NR SRS carrier based switching.

The requirements shall apply for EN-DC.

This section contains interruption requirements when the victim cell is PCell or SCell belonging to MCG. Requirements for interruptions where victim cell is the NR PSCell or an NR SCell belonging to SCG are specified in [50].

For a UE which does not support per-FR measurement gaps, interruptions to the PCell or active MCG SCells may be caused by NR PSCell or NR SCells on any frequency range. For UE which support per-FR gaps, interruptions to the PCell or active MCG SCells may be caused by NR PSCell or NR SCells on FR1 only.

<End of Change 1>

<Start of Change 2>

7.32.2.8	Interruptions at NR SRS carrier based switching

A PUSCH-less SCC is a TDD SCC without PUCCH/PUSCH configured. When a UE needs to transmit periodic or aperiodic SRS  and/or non-contention based PRACH on a NR PUSCH-less SCC, the UE can perform carrier based switching to one or more NR PUSCH-less SCCs from a NR CC with PUSCH or from another NR PUSCH-less SCC prior to transmitting SRS and/or PRACH [51].

The interruption on PCC, PSCC and each of the activated SCCs during the NR SRS carrier switching to the NR PUSCH-less SCC shall not exceed 2 subframes including the first subframe where SRS transmission is configured on the NR PUSCH-less SCC.

The interruption on PCC, PSCC and each of the activated SCCs during the NR SRS carrier switching from the NR PUSCH-less SCC shall not exceed 2 subframes including the last subframe where SRS transmission is configured on the NR PUSCH-less SCC.

<End of Change 2>

<Start of Change 3>

7.36	Interruptions with NE-DC

7.32.1	Introduction

This clause contains the requirements related to the interruptions on PSCell and SCG SCells when

transitions between active and non-active during NR PCell DRX, or

transitions from NR PCell non-DRX to DRX, or

SCell in either NR MCG or E-UTRA SCG is added or released, or

SCell in either NR MCG or E-UTRA SCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either NR MCG or E-UTRA SCG, or

a downlink bandwidth part (BWP) and/or an uplink BWP is switched in NR PCell or in any NR SCell,

NR SRS carrier based switching.

The requirements shall apply for NE-DC.

This clause contains interruption requirements when the victim cell is PSCell or SCell belonging to SCG. Requirements for interruptions where victim cell is the NR PCell or an NR SCell belonging to MCG are specified in TS 38.133 [50].

For a UE which does not support per-FR measurement gaps, interruptions to the PSCell or active SCG SCells may be caused by NR PCell or NR SCells on any frequency range. For UE which support per-FR gaps, interruptions to the PSCell or active SCG SCells may be caused by NR PCell or NR SCells on FR1 only.

<End of Change 3>

<Start of Change 4>

7.36.2.7	Interruptions at SRS carrier based switching

A PUSCH-less SCC is a TDD SCC without PUCCH/PUSCH configured. When a UE needs to transmit periodic or aperiodic SRS  and/or non-contention based PRACH on a NR PUSCH-less SCC, the UE can perform carrier based switching to one or more NR PUSCH-less SCCs from a NR CC with PUSCH or from another NR PUSCH-less SCC prior to transmitting SRS and/or PRACH [51].

The interruption on PCC, PSCC and each of the activated SCCs during the NR SRS carrier switching to the NR PUSCH-less SCC shall not exceed 2 subframes including the first subframe where SRS transmission is configured on the NR PUSCH-less SCC.

The interruption on PCC, PSCC and each of the activated SCCs during the NR SRS carrier switching from the NR PUSCH-less SCC shall not exceed 2 subframes including the last subframe where SRS transmission is configured on the NR PUSCH-less SCC.

<End of Change 4>