3GPP TSG-RAN WG4 Meeting #93	R4-1913470

# Reno, NV USA, Nov. 18th – 22th, 2019

Agenda item:	7.1.1

Source: 	Intel Corporation

Title: 	Clarification on EN-DC dynamic power sharing

Document for:	Discussion

# Introduction

In past a few meetings, there were some analysis [1-4] concerning there would be always some cases in dynamic power sharing to allow SCG to drop whenever there is an MCG transmission. In this contribution, we clarify that ‘a’ and ‘b’ conditions are only used to derive PCMAX_ EN-DC _L. There is no implication to allow SCG to drop if sum of power is less than PCMAX_ EN-DC _L subject to PSD check.

# Discussion

RAN4 had been being extensively discussed EN-DC power sharing for about one year until RAN#82 meeting in Dec. 2018 when the first version of complete requirements was done.

Since first version of requirements was published, there were some contributions [1-4] to show current EN-DC dynamic power sharing rules may allow SCG to drop or scale unnecessarily.

The following arguments were excerpted from one of contributions [4].

The scaling and dropping conditions in TS 38.101-3 depend on conditions ‘a’ and ‘b’ which are defined as the following

a= 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) ] > PEN-DC,tot_L

b= 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) /X_scale] > PEN-DC,tot_L .

where the UE is allowed to drop the NR carrier if ‘b’ is true and the UE is allowed to scale pCMAX,f,c,NR by X_scale if ‘a’ is true and ‘b’ is FALSE.  As noted in [3-5] current requirements in TS 38.101-3 have several negative consequences, including the following:

- For multiple realistic deployment scenarios, the condition ‘b’ is always true and the dynamic power sharing UE is allowed to drop the SCG carrier whenever there is an MCG transmission.

- For deployment scenarios for which condition ‘a’ is TRUE and condition ‘b’ is FALSE, the UE is allowed to scale the SCG even when no scaling is needed to meet either emissions or total power constraints.

- There is no requirement that that power not used by the MCG be made available to the SCG. Thus, the specification allows dynamic power sharing, but does not require it.

We present our view on the statement highlighted above.

The whole discussion point is whether the scaling or dropping conditions in TS38.101-3 depend on condition ‘a’ and ‘b’?

a= 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) ] > PEN-DC,tot_L

b= 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) /X_scale] > PEN-DC,tot_L

If a= FALSE and the configured transmission power spectral density between the MCG and SCG differs by less than [6] dB

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,,NR c(q)], PEMAX, EN-DC ,PPowerClass, EN-DC - ΔPPowerClass,EN-DC }

ELSE If (a=TRUE) AND (b=FALSE) and the configured transmission power spectral density between the MCG and SCG differs by less than [6] dB

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,,NR c(q) /X_scale ], PEMAX, EN-DC ,PPowerClass, EN-DC - ΔPPowerClass,EN-DC }

ELSE If b= TRUE or the configured transmission power spectral density between the MCG and SCG differs by more than [6] dB

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) ], PEMAX, EN-DC ,PPowerClass, EN-DC- ΔPPowerClass,EN-DC }

In our understanding, for intra-band contiguous/non-contiguous EN-DC with dynamic power sharing specified in 38.101-3, condition ‘a’ and ‘b’ are ONLY for the purpose of deriving PCMAX_ EN-DC _L, the lower bound of PCMAX_ EN-DC.

There is no implication allowing SCG to drop or scale from condition ‘a’ and ‘b’.

Observation 1: In configured output power for intra-band contiguous/non-contiguous EN-DC with dynamic power sharing specified in 38.101-3, condition ‘a’ and ‘b’ are ONLY for the purpose of deriving PCMAX_ EN-DC _L, the lower bound of PCMAX_ EN-DC. There is no implication allowing SCG to drop or scale from condition ‘a’ and ‘b’.

The condition allowing SCG to drop or scale is whether the sum of independently configured power of MCG and SCG is greater than the lower bound PCMAX_ EN-DC _L (assume PCMAX_ EN-DC _L = PCMAX_ EN-DC ) . “independently configured power” means power control is performed separately between MCG and SCG since power control is per CC based.

- When sum of independently configured power of MCG and SCG is less than or equal to PCMAX_ EN-DC _L, there is no dropping or scaling allowed for SCG subject to PSD check.

- When sum of independently configured power of MCG and SCG is greater than PCMAX_ EN-DC _L, SCG is allowed to scale or to drop since MCG has power priority. If after X_scale dB down scaling of SCG, the sum power still greater than PCMAX_ EN-DC _L, then SCG is allowed to drop. Otherwise, SCG is allowed to scale only, subject to PSD check.

Observation 2:

- When sum of independently configured power of MCG and SCG is less than or equal to PCMAX_ EN-DC _L, there is no dropping or scaling allowed for SCG subject to PSD check.

- When sum of independently configured power of MCG and SCG is greater than PCMAX_ EN-DC _L, SCG is allowed to scale or to drop since MCG has power priority. If after X_scale dB down scaling of SCG, the sum power still greater than PCMAX_ EN-DC _L, then SCG is allowed to drop. Otherwise, SCG is allowed to scale only, subject to PSD check.

In the EN-DC dynamic power sharing, SCG behavior of dropping or scaling is implied by sum power against PCMAX_ EN-DC _L.

Observation 3: In the EN-DC dynamic power sharing, SCG behavior of dropping or scaling is implied by sum power against PCMAX_ EN-DC _L.

# Conclusion

In this contribution, we show our understanding on condition ‘a’ and ‘b’ in EN-DC dynamic power sharing. It is our understanding that condition ‘a’ and ‘b’ are not the conditions to determine SCG’s dropping or scaling. They are used to determine PCMAX_ EN-DC _L only. We have following observations.

Observation 1: In configured output power for intra-band contiguous/non-contiguous EN-DC with dynamic power sharing specified in 38.101-3, condition ‘a’ and ‘b’ are ONLY for the purpose of deriving PCMAX_ EN-DC _L, the lower bound of PCMAX_ EN-DC. There is no implication allowing SCG to drop or scale from condition ‘a’ and ‘b’.

Observation 2:

- When sum of independently configured power of MCG and SCG is less than or equal to PCMAX_ EN-DC _L, there is no dropping or scaling allowed for SCG subject to PSD check.

- When sum of independently configured power of MCG and SCG is greater than PCMAX_ EN-DC _L, SCG is allowed to scale or to drop since MCG has power priority. If after X_scale dB down scaling of SCG, the sum power still greater than PCMAX_ EN-DC _L, then SCG is allowed to drop. Otherwise, SCG is allowed to scale only, subject to PSD check.

Observation 3: In the EN-DC dynamic power sharing, SCG behavior of dropping or scaling is implied by sum power against PCMAX_ EN-DC _L.

# References

- R4-1904676, On the SCG MPR/A-MPR and Pcmax for EN-DC with Dynamic Power Sharing, Motorola Mobility, RAN4#90, April, 2019

- R4-1906957, Further Discussion of SCG MPR/A-MPR and PCMAX for EN-DC with Dynamic Power Sharing. Motorola Mobility, RAN4#91, May 2019

- R4-1909975, On the Lack of a Dynamic Power Sharing Requirement in TS 38.101-3, Motorola Mobility, RAN4#92, August 2019

- R4-1912915, Additional Requirements Needed for Dynamic Power Sharing, Motorola Mobility, RAN4#92bis, Oct. 2019