3GPP RAN WG4 Meeting #93		R4-1913506

Reno, USA, 18 – 22th November 2019

Agenda item:	9.13.1

Source:	Apple Inc.

Title:	On RRM Impact of Tx Switching

WI:	NR_RF_FR1

Release:	Rel-16

Document for:	Discussion

# 1	Introduction

As part of FR1 enhancement WI, a new objective was included

Specify UE requirements to allow switching between case 1 and case 2 as below for two uplink carriers case inter-band EN-DC without SUL, inter-band UL CA and standalone SUL for UE supporting maximum two concurrent transmission

| Case 1 | 1 Tx on carrier 1 and 1 Tx on carrier 2 |
| --- | --- |
| Case 2 | 0 Tx on carrier 1 and 2 Tx on carrier 2 |

  - UE RF requirements, e.g., time mask RF requirements and other necessary RF requirements if any

    - The options agreed at RAN4 #92 in R4-1910531 can be considered as starting point

  - Study if there are any impact to interruption and delay requirements, and specify the RRM requirements if needed

  - RAN1 will further study by Dec 2019 if there are any RAN1 potential impacts based on RAN4 LS if any

    - No new TDM pattern will be defined, i.e. scheduling-based switching is assumed.

    - Finalization of RAN4 requirements and approval of RAN4 CRs shall be based on RAN1 LS

    - Strive to minimize RAN1 impact.

    - Strive to achieve no impact to RAN1 E-UTRAN spec

    - Strive to avoid defining location of switching period impacting RAN1 spec

  - Define per band per band combination or per band combination UE capability signaling if needed

Note 1: Only addressing the case of co-located and synchronized network deployment for the two UL carriers

Note 2:  Only addressing the case of single TAG for the two UL carriers for SUL and for UL CA

Note 3:  The above objectives will not relax the existing requirements specified in Rel-15 38.101-3 for band combinations allowing single uplink transmission

Note 4: The UE is configured with two different uplink carrier frequencies.

In RAN4#92bis, the following agreements have been made

- Location of the switching period for EN-DC

  - in NR carrier

- Location of the switching period for UL CA and SUL

  - semi-statically configured by RRC on one specific carrier of the two uplink carriers

- Applicable carrier(s) of UL outage due to switching

  - Option 1: both carrier 1 and carrier 2

- Additional time for PUSCH preparation procedure

  - Inform RAN1 the potential issue identified

    - Information to RAN1: A potential issue was raised in RAN4 that UL switching period may impact PUSCH preparation procedure time.

  - RAN4 can continue discussing on whether the PUSCH preparation time can happen in parallel with the switching time, based on the UE implementation.

The remaining RRM related issues include

- Location of the switching period for EN-DC and SA CA

- Length of the switching period

- DL interruption and related requirements

# 2	Location of the switching period for EN-DC and SA CA

It has been agreed in RAN4#92bis that

- Location of the switching period for EN-DC

  - in NR carrier

- Location of the switching period for UL CA and SUL

  - semi-statically configured by RRC on one specific carrier of the two uplink carriers

In TS36.213, it is specified that for a serving cell that is not a LAA SCell, and for FDD and normal HARQ operation, the UE shall, upon detection on a given serving cell of PDCCH in subframe n, perform a corresponding PUSCH transmission in subframe n+4 if UE is not configured with higher layer parameter shortProcessingTime. The related 4ms PUSCH preparation time include DCI parsing and preparation time for PUSCH transmission. In the worst scenario, UE won’t be ready for PUSCH transmission until the end of 4ms window. As illustrated in Figure 1, if there is gap between adjacent LTE and NR UL transmission and the gap length is more than 4ms, UE will not be aware of coming UL scheduling is on NR or LTE.

In this case, there are two options

Option 1: Location of switching period can be on either LTE or NR in EN-DC

By the end of a slot used for NR UL transmission, if UE does not receive any PDCCH intended for the UE for UL transmission in coming slot for NR or subframe for LTE and the next UL transmission is scheduled on LTE, the location of switching period is located on LTE carrier.

Otherwise, the location of switching period is located on NR carrier.

Option 2: Location of switching period can only be on NR in EN-DC.

By the end of a slot used for NR UL transmission, if UE does not receive any PDCCH intended for the UE for PUSCH transmission in coming slot for NR or subframe for LTE, switching period is always introduced just before the end of the current NR slot regardless the coming UL transmission is on LTE or NR.

Figure 1: EN-DC: Tx switching with large gap between adjacent LTE and NR UL

Considering it is feasible for both LTE and NR to flexibly schedule PUSCH with a LTE UL subframe and NR UL slot, there is not significant drawback to have the switching period on LTE carrier. To avoid the unnecessary Tx switching, Option 1 is preferred.

Proposal 1: For EN-DC, if UE does not receive any PDCCH intended for the UE for UL transmission in coming slots for NR or subframes for LTE by the end of a slot used for NR UL transmission, and if the next available UL transmission is scheduled on LTE, the location of switching period is located on LTE carrier.

Otherwise, the location of switching period is located on NR carrier.

Similar situation can happen to SA CA too, where DCI for next UL transmission coming after the current UL slot. In this case, it is proposed to define a unified UE behaviour to cover all different scenarios.

Proposal 2: For SA CA, if a switching period is introduced between two UL slots, the location of it is always on the later slot.

# 3	Length of Switching period

In RAN4#92bis, RAN4 recommendation on the length of UL switching period for defining UE RF requirements and capability reporting include [0]us, 35us, 140 us, [250]us. RF session should continue discussing whether it is necessary to keep 0us and 250us. Theoretically, the switching period should be an integer of a symbol duration. To avoid ambiguity, it is proposed to specify the length of switching period in terms of number of symbols for different SCS.

Proposal 3: Length of switching period should be defined as in a unit of symbol duration

| Capability of switching period | SCS=15kHz | SCS=30kHz | SCS=60kHz |
| --- | --- | --- | --- |
| [0]us | [0] symbol | [0] symbol | [0] symbol |
| 35us | 1 symbol | 1 symbol | 2 symbols |
| 140us | 2 symbols | 4 symbols | 8 symbols |
| [250]us | [4] symbols | [7] symbols | [14] symbols |

# 4       DL interruption and related requirements

Depending on the implementation, Tx switching may result in RF chain re-configuration and PLL retuning/switching, which typically impact on DL reception due to shared power supply and limited isolations. Based on proposal 3, when switching period is more than 35us, a significant portion of slot will be used for Tx switching. Since NW may not have procise timing information on the occasion of Tx switching, the whole slot may have to be dropped.

Interruption due to RF transient effect has been well discussed in RAN4. More specifically, the interruption requirements at UL carrier RRC reconfiguration can be used as the reference for Tx switching related discussion.

Proposal 4: DL interruption requirements at UL carrier RRC reconfiguration can be used as the reference for Tx switching related interruption requirements.

# 5 Conclusion

In this contribution, the RRM impacts of Tx switching in super UL are analysed. It is proposed that

Proposal 1: For EN-DC, if UE does not receive any PDCCH intended for the UE for UL transmission in coming slots for NR or subframes for LTE by the end of a slot used for NR UL transmission, and if the next available UL transmission is scheduled on LTE, the location of switching period is located on LTE carrier.

Otherwise, the location of switching period is located on NR carrier.

Proposal 2: For SA CA, if a switching period is introduced between two UL slots, the location of the switching period is always on the later slot.

Proposal 3: Length of switching period should be defined as in a unit of symbol duration

| Capability of switching period | SCS=15kHz | SCS=30kHz | SCS=60kHz |
| --- | --- | --- | --- |
| [0]us | [0] symbol | [0] symbol | [0] symbol |
| 35us | 1 symbol | 1 symbol | 2 symbols |
| 140us | 2 symbols | 4 symbols | 8 symbols |
| [250]us | [4] symbols | [7] symbols | [14] symbols |

Proposal 4: DL interruption requirements at UL carrier RRC reconfiguration can be used as the reference for Tx switching related interruption requirements.

# References