3GPP TSG RAN WG4 Meeting #93	R4-1913526

Reno, NV, US, 18 - 22 November 2019

Agenda item:	13.1

Source:	SoftBank Corp.

Title:	On addition of "brand new" CBWs

Document for:	discussion

1.  Introduction

Addition of “brand new” CBWs, i.e. CBWs not supported in REL-15, was discussed in RAN4#92B and WF was approved [1]. This paper considers some UE aspects, relation to remedies mentioned in the previous meeting in addition.

In the sections below, adding 13MHz CBW is used as an example.

2.  Prerequisites for a brand new CBW

A motivation to define a brand new CBW is clearly for maximum spectrum utilization in a given bandwidth but a CBW is not just linked to a maximum RB configuration:

1) An index to comply with regulatory requirements (if any),

2) For co-existence with technologies in its vicinity (e.g. SEM),

3) A prerequisite for some RF requirements.

Generally speaking, there would be no spectrum available without any external (regulatory or co-ex) requirements. From this angle, we need to identify what is required for a specific proposal. For example, concerning Rx in section 3.1 (broader CBW reused) case, when a technology next to a 13MHz CBW is deployed very close to a border, spanning 15MHz CBW for 13MHz might have a risk on ACS and then it may not be acceptable.

If a proponent wants/needs to guarantee a certain level of requirements in 38.101-1, it would be necessary to define a CBW as it has been done.

[Observation-1] If a brand new CBW needs a certain level of RAN4 requirements, we have to define the CBW as it has been done.

However, the problem is that, to satisfy some RF requirements, UE chipset needs base-band modification to be fit to the CBW and it will take time. It is going to be cumbersome if we should address CA/MR-DC including the brand new CBW. When we consider remedies as above to mitigate the pain, one of the important criteria is in which item(s) a proponent can compromise.

[Observation-2] To consider remedies, an essential point is in which requirement(s) a proponent can compromise.

However, it should be noted that possibility or degree of compromise would largely depend on environment surrounding the brand new CBW, i.e. regulatory status and users/technologies in its vicinity.

[Observation-3] The degree of possible compromise would largely depend on regulatory and co-ex environment.

In this sense, sometimes the choice would be beyond operator’s reach.

3.  Comments on remedies for UE

3.1  One step broader CBW

This was described in [2] to use one step broader CBWs in REL-15 CBW set with confined RBs, most likely using a BWP. The proposal assumed that the center frequency should be aligned to the center of brand new CBW. In case of 13MHz CBW operation, 15MHz CBW (green) is configured with some lower/higher RBs left unused. With this scheme, the whole spectrum (13MHz: red) can be used but with relaxed (Green line) SEM. Our understanding is depicted in the figure below.

Figure 1. 13MHz in 15MHz UE CBW

Firstly, from co-ex standpoint, it would be same as occupying 15MHz CBW as there is no 13MHz SEM defined and tested. In addition, as mentioned in the previous section, Rx requirements would be relaxed somewhat such as ACS.

Another issue to clarify is, if frequency B is on the edge of a band, spanning 15MHz CBW from frequency B-1 could be regarded as "invalid" configuration from a UE/BS. In case of A-MPR/NS, since some NSs are defined with Fc, invalid Fc/CBW combination would cause a problem. We also need to clarify whether we should define a dedicated NS for 13MHz in 15MHz CBW or would reuse 15MHz NS assuming the current 13/15MHz alignment.

Then, all in all,

[Observation-4] If the use of one step broader CBW is further pursued, it is necessary to clarify which requirements are attained and which are not.

If an operation is permitted from regulatory/co-ex standpoint and issues mentioned above are positively solved, use of one-step broader BW could be attractive since a proponent can enjoy full RB allocation for a UE while there is almost no concern on UE availability for the brand new CBW.

3.2  One step narrower CBW

This was mentioned during the meeting. Based on the example above, UE CBW is limited to 10MHz while BS CBW could be 13MHz. It is a part of conventional operations in NR: just to set UE CBW to 10MHz and a BWP less than or equal to that. Considering a legacy UE which does not support the brand new CBW, we need to support this type of operation anyway. An apparent drawback is that full BW could not be used from UE perspective.

[Observation-5] For the use of one step narrower CBW, there is no technical concern but full BW is not available from UE perspective.

4   Conclusion

This paper is to discuss prerequisites for a brand new CBW and its remedy solutions. Whether or not the new CBW needs to be defined or not (= taking a remedy) largely depends on external conditions. Further discussion is expected to find a rational way out.

Reference

R4-1913079		Way forward on the handling new CBW to existing NR bands, RAN4 vice chairman

R4-1911519	   Views on adding new Channel Bandwidths for NR bands, Apple