3GPP TSG-RAN WG4 Meeting #93	R4-1913568

Reno, USA, 18th – 22nd Nov, 2019

Agenda item:	9.11.2.1

Source: 		NTT DOCOMO, INC.

Title: 	RRM core requirements on L1-SINR

Document for:	Discussion

1. Introduction

In this contribution, we discuss our views on RRM design for L1-SINR measurement for eMIMO based on the agreed WF in RAN4 #92bis [1]. It is noted that detail of RAN1 discussion is summarized in [2].

2. Discussion

## Scenario

Followings are agreements reached in the WF [1].

- Scenarios of L1-SINR measurement for RRM requirement:

  - In RAN4#93, RAN4 study and conclude which scenario(s) shall be included for L1-SINR measurement requirement and evaluation (i.e., measurement period and measurement accuracy).

  - Down-selecting scenarios to define L1-SINR measurement requirement from following possible scenarios:

    - Scenario 1: L1-SINR measured on CMR only (when dedicated IMR is not configured)

      - Scenario 1A: CSI-RS based CMR (only with density 3 REs/RB)

    - Scenario 2: L1-SINR measured on CMR+IMR (when dedicated IMR is configured)

      - Scenario 2A: SSB based CMR + ZP-IMR

      - Scenario 2B: CSI-RS based CMR + ZP-IMR

      - Scenario 2C: SSB based CMR + NZP-IMR (only with density 3 REs/RB)

      - Scenario 2D: CSI-RS based CMR + NZP-IMR (only with density 3 REs/RB)

      - Other options are not excluded, depending on RAN1 agreement.

  - At least the following aspects are identified to be needed for consideration:

    - Question-1: Whether to define separate requirements for CMR only and CMR+IMR?

    - Question-2: Whether to define separate requirements for SSB-based CMR and CSI-RS based CMR?

    - Question-3: Whether to define separate requirements for different IMR configuration?

Table I shows seven possible combinations of CMR and IMR for L1-SINR measurement that is supported by RAN1. It is noted that RAN1 specification doesn’t support L1-SINR measurement with (CMR, IMR)=(SSB, none). In addition, Case 3 and 7 are agreed as a working assumption and still under discussion in RAN1. Among these cases, ZP-IMR-only-based interference measurement is simple and useful that can measure various interference sources. In this sense, we prefer to prioritize Cases 1 and 5 (scenario2A and 2B), if down-selection is needed.

Table I: L1-SINR measurement with different configurations

|  |  |  | IMR |  |  | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | None | ZP-IMR | NZP-IMR |  |
| Case 1 | CMR | SSB |  |  |  | Scenario 2A |
| Case 2 |  |  |  |  |  | Scenario 2C |
| Case 3 |  |  |  |  |  | Working assumption |
| Case 4 |  | CSI-RS |  |  |  | Scenario 1A |
| Case 5 |  |  |  |  |  | Scenario 2B |
| Case 6 |  |  |  |  |  | Scenario 2D |
| Case 7 |  |  |  |  |  | Working assumption |

Proposal 1: Prioritize scenarios 2A and 2B, if down-selection is needed.

## Measurement period

Followings are agreements on measurement period reached in the WF [1].

- For L1-SINR measurements based on CMR only, define L1-SINR measurement period as a number of CMR periodicities and reuse the requirements on L1-RSRP measurement period for both FR1 and FR2. FFS for L1-SINR measurements based on CMR and IMR.

First of all, if L1-SINR is configured with aperiodic CSI-RS and/or measurement restriction, UE needs to perform measurement without averaging. For other cases, we can consider to apply multiple measurement samples in order to achieve sufficient measurement accuracy. For the L1-SINR measurement with IMR(s), S and I+N is measured using CMR and IMR, respectively. In order to perform accurate L1-SINR measurement, each of them should fulfil proper measurement accuracy. Suppose that S and I+N can be accurately measured using X CMR and Y IMR samples, respectively, measurement period should be designed with longer duration with X CMR samples and Y IMR samples.

Observation: In order to perform accurate L1-SINR measurement, each of S measurement and I+N measurement should fulfil proper measurement accuracy.

Proposal 1:

- If L1-SINR is configured with aperiodic CSI-RS and/or measurement restriction, measurement period should be specified without averaging.

- Otherwise, suppose that S and I+N can be accurately measured using X CMR and Y IMR samples, respectively, measurement period should be designed with the longer duration among X CMR samples and Y IMR samples.

## Measurement restriction

Followings are agreements on measurement period reached in the WF [1].

- Define L1-SINR measurement restriction requirement by applying the same principles as for L1-RSRP measurement.

For CMR-based L1-RSRP measurement, basic rules on measurement restrictions for L1-RSRP can be reused. In addition, we need to extend measurement restriction mechanism for IMR. At least for NZP-IMR, it seems possible to reuse the principles for CMR, since UE reception behaviour is similar to that for CMR. For ZP-IMR, if UE can perform other Rx behaviour than CMR/ZP-IMR, we can consider relaxing measurement restriction.

Proposal 2: At least for CMR and NZP-IMR, define L1-SINR measurement restriction by applying the same principles as for L1-RSRP measurement.

## Scheduling restriction

Followings are agreements on measurement period reached in the WF [1].

- Define L1-SINR scheduling restriction requirement by applying the same principles as for L1-RSRP measurement.

For CMR-based L1-RSRP measurement, basic rules on scheduling restrictions for L1-RSRP can be reused. In addition, we need to extend scheduling restriction mechanism for IMR. At least for NZP-IMR, it seems possible to reuse the principles for CMR, since UE reception behaviour is similar to that for CMR. For ZP-IMR, if UE can perform other Rx behaviour than CMR/ZP-IMR, we can consider relaxing measurement restriction.

Proposal 3: At least for CMR and NZP-IMR, define L1-SINR scheduling restriction by applying the same principles as for L1-RSRP measurement.

3. Conclusion

In this contribution, we presented our views on RRM design for L1-SINR measurement for eMIMO based on the agreed WF in RAN4 #92bis [1]. Our observations and proposal are shown as follows.

Observation: In order to perform accurate L1-SINR measurement, each of S measurement and I+N measurement should fulfil proper measurement accuracy.

Proposal 1:

- If L1-SINR is configured with aperiodic CSI-RS and/or measurement restriction, measurement period should be specified without averaging.

- Otherwise, suppose that S and I+N can be accurately measured using X CMR and Y IMR samples, respectively, measurement period should be designed with the longer duration among X CMR samples and Y IMR samples.

Proposal 2: At least for CMR and NZP-IMR, define L1-SINR measurement restriction by applying the same principles as for L1-RSRP measurement.

Proposal 3: At least for CMR and NZP-IMR, define L1-SINR scheduling restriction by applying the same principles as for L1-RSRP measurement.

References

[1] 3GPP, R4-1912742, Samsung, Huawei, “WF on RRM Requirement Scope for Rel-16 NR eMIMO WI,” Oct. 2019.

[2] 3GPP, R1-1911730, “FL summary NR_eMIMO RAN1#98bis,” Oct. 2019.