3GPP TSG-RAN WG4 Meeting #93	R4-1913574

Reno, USA, 18 – 22 November 2019

Source:	Intel Corporation

Title:	TP to TR 37.716-21-21: Addition of DC configuration for DC_2A-7A_n38A-n78A

Agenda item:	10.8.2

Document for:	Approval

# Background

This contribution provides text proposal on the EN-DC configuration DC_2A-7A_n38A-n78A as defined in Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) [1].

# Text Proposal

##### ---Start of changes---

## 7.x	DC_2A-7A_n38A-n78A

### 7.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of two LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-7A_n38A-n78A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of two LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A-7A_n38A-n78A | DC_2A-n78A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.x.3	Co-existence studies

The co-existence studies for the lower order DC configuration (DC_2-7_n78) is already captured in TR 37.716-21-11. For additional receiver band (band n38), there is no MSD issue for this DC configuration.

### 7.x.4	∆TIB and ∆RIB values

For DC_2A-7A_n38A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-7A_n38A-n78A | 2 | 0.6 |
|  | n78 | 0.8 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2A-7A_n38A-n78A | 2 | 0.2 |
|  | 7 | 0 |
|  | n38 | 0 |
|  | n78 | 0.5 |

### 7.x.5	MSD

No MSD is required for this DC configuration.

##### ---End of changes---

# Reference

[1]	R4-1906055, "Revised WID on LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) EN DC in rel-16," LG Electronics France