3GPP TSG-RAN WG4 Meeting #93                 	 R4-1913588

Reno, USA, 18th – 22nd Nov, 2019

Agenda Item:	9.16.1.1

Source:	MediaTek Inc.

Title:	Simulation results for CSI-RS measurement BW

Document for:		Discussion

# Introduction

In last meeting, a WF [1] was agreed with some progress on the measurement bandwidth of CSI-RS.

- CSI-RS configurations - Requirements shall be defined for N sets of CSI-RS configurations (bandwidth of CSI-RS and density of CSI-RS resource) - Option 1: N =1 - Option 2: N = 2 - Further decide N based on simulation results for different CSI-RS configurations - Define requirements at least for 48 PRBs CSI-RS BW configuration. CSI-RS density is FFS. - Encourage companies to submit simulation results based on agreed simulation assumption [1] in RAN4#93 meeting - Do not define requirements for CSI-RS with 24 PRBs BW and D = 1

In this paper, we express our view on this issue.

# Discussion

Regarding the simulation assumption, we follow the agreed documents [2]. The results are provided in Table 1 based on AWGN at SINR = -6dB with 15KHz SCS. In the discussion of SSB based requirements, the accuracy contributed by baseband is roughly 2dB. Therefore, we also highlighted those configurations with baseband accuracy < 2dB in Table 1.

Table 1. RSRP accuracy for AWGN at SINR = -6dB with 15KHz SCS

| RB # | Density | Sample # | 5% | 50% | 95% | 90% abs |
| --- | --- | --- | --- | --- | --- | --- |
| 24 | 3 | 1 | -2.45 | 2.01 | 4.36 | 4.11 |
|  |  | 3 | -0.99 | 1.51 | 3.08 | 2.82 |
|  |  | 5 | -0.51 | 1.32 | 2.64 | 2.38 |
|  |  | 10 | -0.2 | 1.11 | 2.16 | 1.93 |
| 48 | 1 | 1 | -2.51 | 2.43 | 4.96 | 4.69 |
|  |  | 3 | -0.82 | 1.85 | 3.67 | 3.33 |
|  |  | 5 | -0.35 | 1.63 | 3.16 | 2.85 |
|  |  | 10 | 0.03 | 1.41 | 2.59 | 2.34 |
|  | 3 | 1 | -1.92 | 1.42 | 3.45 | 3.22 |
|  |  | 3 | -0.81 | 1 | 2.37 | 2.12 |
|  |  | 5 | -0.55 | 0.85 | 2 | 1.78 |
|  |  | 10 | -0.3 | 0.72 | 1.57 | 1.38 |
| 96 | 1 | 1 | -2.21 | 1.75 | 3.97 | 3.72 |
|  |  | 3 | -0.89 | 1.25 | 2.81 | 2.53 |
|  |  | 5 | -0.53 | 1.07 | 2.38 | 2.12 |
|  |  | 10 | -0.24 | 0.91 | 1.88 | 1.67 |
|  | 3 | 1 | -1.41 | 1.01 | 2.66 | 2.43 |
|  |  | 3 | -0.69 | 0.67 | 1.77 | 1.56 |
|  |  | 5 | -0.5 | 0.55 | 1.47 | 1.28 |
|  |  | 10 | -0.3 | 0.45 | 1.12 | 0.97 |
| 192 | 1 | 1 | -1.7 | 1.24 | 3.11 | 2.89 |
|  |  | 3 | -0.73 | 0.85 | 2.11 | 1.88 |
|  |  | 5 | -0.51 | 0.71 | 1.78 | 1.55 |
|  |  | 10 | -0.3 | 0.58 | 1.36 | 1.19 |
|  | 3 | 1 | -1.03 | 0.73 | 2.05 | 1.83 |
|  |  | 3 | -0.5 | 0.46 | 1.31 | 1.15 |
|  |  | 5 | -0.38 | 0.38 | 1.07 | 0.92 |
|  |  | 10 | -0.23 | 0.3 | 0.78 | 0.68 |

The decision on the measurement BW has the big impact to both the measurement accuracy, measurement delay and UE complexity. To simplify the spec, we suggest that requirements is defined based on only one single measurement bandwidth. Another reason is that BWP can be dynamically changed in NR. Therefore, UE’s active BWP BW could be changed from time to time. BW-specific requirement adds additional complexity to UE which needs to change its scheduling based on the BW of active BWP. This is some unnecessary complexity. This is different to what we have in LTE, because LTE channel BW never changes.

Proposal 1: Requirement is defined based on single measurement bandwidth (N=1).

Regarding the size of the measurement BW, a common concern from UE vendor is that the larger BW demands higher UE complexity as well as higher power consumption. Although from the simulation results we can see that larger measurement BW brings more accurate measurement results, but it also introduces larger RF path loss calibration error which cannot be seen from the simulation result.

Observation 1: Larger measurement BW does not always lead to better measurement accuracy requirement because the RF path loss calibration error also increases.

For RLM, BFD, CBD and L1-RSRP, the requirements were specified based on 48 PRBs with density 3. We think it could provide UE a chance to re-use the measurement engines designed for L1 measurement for CSI-RS based L3 measurement. This is good to this feature because UE is able to support it through only software upgrade without hardware impact. After checking Table 1, the accuracy of 48 PRBs with density 3 is also within 2dB. Therefore, we think the requirements of CSI-RS based L3 measurement can be specified based on 48 PRBs and density 3.

Proposal 2: For CSI-RS based L3 measurement, the requirements are specified based on 48 PRBs and density 3.

# Summary

In this paper, we provide the simulation results of RSRP accuracy for CSI-RS based L3 measurement. We have the following observations and proposals:

Observation 1: Larger measurement BW does not always lead to better measurement accuracy requirement because the RF path loss calibration error also increases.

Proposal 1: Requirement is defined based on single measurement bandwidth (N=1).

Proposal 2: For CSI-RS based L3 measurement, the requirements are specified based on 48 PRBs and density 3.

# Reference

[1] R4-1910956, “WF on CSI-RS based RRM measurements requirements”, CATT

[2] R4-1910949, “Link-level simulation assumptions for CSI-RS based measurements”, CATT