3GPP TSG RAN WG4 Meeting #93	R4-1913639

Reno, USA, 18-22 Nov. 2019

Agenda item:	8.4.2

Source:	SoftBank Corp.

Title:	TP for TR 36.716-04-01: LTE CA_1-8-11-42

Document for:	Approval

1	Introduction

4B LTE CA of CA_1-8-11-42 was approved in RAN#85[1]. This TP is to capture the basic aspects for LTE CA.

2.  Reference

[1]  RP-191730  Revised WI: Rel'16 LTE inter-band CA for x bands DL (x=4, 5) with 1 band UL

Annex.

[Unchanged Parts Skipped]

5.X	CA_1-8-11-42

5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-8A-11A-42A | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-8A-11A-42C | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

5.X.2	∆TIB and ∆RIB values

For CA_1-8-11-42, the TIB,c and RIB,c values are given in the tables below.

Table 5.X.2-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-8-11-42 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 11 | 0.4 |
|  | 42 | 0.8 |

Table5.X.-2: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_1-8-11-42 | 1 | 0 |
|  | 8 | 0.2 |
|  | 11 | 0 |
|  | 42 | 0.5 |

5.X.3	REFSENS requirements

REFSENS requirements for CA_1-8-11-42 (exceptions due to harmonic issues) are defined in table 5.X.3-1 for inclusion in TS 36.101 table 7.3.1A-0a, and in table 5.X.3-2 for inclusion in TS 36.101 table 7.3.1A-0b.

Table 5.X.3-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-8A-11A-42A12,13 CA_1A-8A-11A-42C12,13 | 4233 |  |  |  |  |  | -84.8 |  | -84.7 |  | -84.6 |  | -84.5 |  | TDD |  |
| NOTE 12: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 13: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 5.X.3-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-8A-11A-42A CA_1A-8A-11A-42C | 8 |  |  | 8 | 16 | 25 | 25 | FDD |

[Unchanged Parts Skipped]