3GPP TSG-RAN WG4 Meeting #93	R4-1913703

Reno, USA, 18th – 22th Nov. 2019

Title: 	TP for channel arrangement design for 5G V2X UE

Source: 	CATT

Agenda item:	9.4.3.2

Document for:	Approval

- Introduction

In other papers [1][2], the channel arrangement design for 5G V2X UE is analysed and some proposals are presented. Based on these proposals, this document presents a TP for TR 38.886 v0.4.0 [3] for the channel arrangement design for 5G V2X UE in Section 3 Annex.

- Reference

[1]	R4-1910973, Further discuss on the channel arrangement design for 5G V2X UE, CATT

[2]	R4-1913702, Discuss on the channel arrangement design for 5G V2X UE, CATT

[3]	3GPP TR38.886 v0.4.0, V2X Services based on NR: User Equipment (UE) radio transmission and reception

- Annex: TP for channel arrangement in TR38.886

============================ Start of TP ============================

7.3	Channel arrangement

7.3.1	Channel arrangement in FR1

7.3.1.1	Channel raster

7.3.1.1.1	NR-ARFCN and channel raster

The NR-ARFCN and channel raster defined in subclause 5.4.2.1 in TS38.101-1 are applied for NR V2X.

For NR V2X UE, the reference frequency can be shifted by configuration.

FREF, shift = FREF + Δshift

where Δshift = -5kHz, -2.5kHz, 0, 2.5kHz, 5kHz, or 7.5kHz, is signalled by the network in higher layer parameter or pre-configuration parameter.

7.3.1.1.2	Channel raster to resource element mapping

Channel raster to resource element mapping defined in subclause 5.4.2.2 in TS38.101-1 are applied for NR V2X.

7.3.1.1.3	Channel raster entries for each operating band

The channel raster entries for each operating band defined in subclaue 5.4.2.3 in TS38.101-1 are applied for NR V2X. The RF channel positions on the channel raster in each NR V2X operating band are given through the applicable NR-ARFCN in Table 7.3.1.1-1, using the channel raster to resource element mapping in subclause 7.3.1.1.2.

Table 7.3.1.1-1: Applicable NR-ARFCN for NR V2X operating band

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n47 | 15 | 790334 – <1> – 795000 | 790334 – <1> – 795000 |

7.3.1.2	Synchronization raster

There is no synchronization raster definition for NR V2X in the specification.

7.3.2	Channel arrangement in FR2

============================ End of TP ============================