3GPP TSG-RAN WG4 Meeting #93	R4-1913708

Reno, US, 18 – 22 November 2019

Title: 	Discussion on transient period in S-SSB design for NR V2X

Source: 	CATT

Agenda item:	9.4.3.2

Document for:	Discussion

- Introduction

An LS on NR V2X S-SSB design from RAN1 was sent to RAN4 in RAN1#98 meeting [1]. In RAN4#92bis meeting, a WF on the transient period for NR V2X S-SSB was approved, in which companies are encouraged to provide further evaluation results for the transient period [2].

In this contribution, whether a transient period between S-PSS and S-SSS or PSBCH is required and how to set the transient period are primarily discussed based on simulation results. In addition, one solution to address the transient period is analyzed and proposed from RAN4 perspective.

- Discussion

As per the previous RAN1 meetings, the concrete agreements concerning NR V2X synchronization mechanisms are shown as follows [1]:

Agreements: - In NR V2X, S-SSB bandwidth is 11RBs. - PSBCH spans 11RBs. - The S-SSB is designed following combination 1. - Length-127 M-sequences for S-PSS and length-127 Gold sequences for S-SSS - Two symbols are used for each of S-PSS and S-SSS, respectively. Agreements: - For the evaluation of PSBCH performance, the payload size of NR V2X PSBCH is 56 bits including 24 bits of CRC. Working assumption: - For the NR SLSS, - Same sequence is used for both symbols of S-PSS - Same sequence is used for both symbols of S-SSS Agreements: - NR S-SSB structure for NCP is as follows: - For the case of ECP, the structure is the same as the above except that the number of PSBCH symbols after S-SSS is only 6 Send LS to RAN4 with the above agreements, and add: - It is up to RAN4 to decide whether a transient period is necessary or not. If so, to address the transient period, one possibility is to shift the symbols starting the first S-SSS symbol by at least one symbol.

In the initial NR S-SSB structure specified in the LS from RAN1, S-PSS symbols and S-SSS symbols are adjacently located in one S-SSB transmission. Based on the simulation results in [3], the PAPR of S-SSS is higher than that of S-PSS by 5dB at 99.9% CDF point, while it is almost the same as that of PSBCH. The MPR impact aroused by different PAPRs of S-PSS and S-SSS or PSBCH should be taken into account.

Based on RAN1 agreements in RAN1#97 [3], two alternatives were derived as follows:

- Alt 1: S-PSS symbols and S-SSS symbols are adjacent. Same MPR is used for S-PSS and S-SSS symbols without puncturing in PSBCH.

- Alt 2: S-PSS symbols and S-SSS symbols are not adjacent. Different MPRs are used for S-PSS and S-SSS symbols with 10us puncturing in PSBCH.

Figure 1: Joint detection probability of S-PSS/S-SSS/PSBCH for 15kHz SCS 6kmph

Figure 2: Joint detection probability of S-PSS/S-SSS/PSBCH for 30kHz SCS 6kmph

To compare these two alternatives, the simulation results are provided by evaluating the joint detection probability of S-PSS/S-SSS/PSBCH. From Figure 1 and Figure 2, it is observed that Alt 2 has higher joint detection probability for both 15kHz SCS and 30kHz SCS compared to that of Alt 1. Specifically, the detection performance gain of Alt 2 for 30kHz SCS at -3dB SNR is approximately 0.8dB, while it is about 0.2dB for 15kHz SCS. To conclude, Alt 2 outperforms Alt 1 considering detection performance of S-PSS, S-SSS and PSBCH.

Observation 1: Alt 2 outperforms Alt 1 considering detection performance of S-PSS, S-SSS and PSBCH.

Based on the simulation results, different MPRs used for S-PSS and S-SSS are essential to guarantee the detection performance of S-PSS, S-SSS and PSBCH.

Proposal 1: Different MPRs used for S-PSS and S-SSS are essential to guarantee the detection performance of S-PSS, S-SSS and PSBCH.

A transient period is needed when different MPRs are used for S-PSS and S-SSS. The transient period gives rise to performance degradation of S-SSS detection when S-PSS and S-SSS are adjacently located [4]. Nevertheless, S-PSS symbols and S-SSS symbols can be non-adjacent along with one (or several) PSBCH symbol(s) distributed in between. In this case, the transmit power of S-PSS no longer needs to back off to a relatively lower level, which can significantly guarantee the S-PSS detection performance. Given that S-SSS and PSBCH have the same MPR, a transient period is not required between these two signals. Therefore, the impact on S-SSB detection performance can be alleviated with the insertion of PSBCH between S-PSS and S-SSS symbols.

The performance of PSBCH and S-SSS detection is associated with the number of PSBCH symbols distributed between S-PSS and S-SSS symbols. In the LTE V2X design, S-PSS symbols and S-SSS symbols are respectively distributed in the head part and tail part of synchronisation subframe along with several PSBCH symbols in the middle part. One drawback of this design is that the receiver needs to first buffer the PSBCH symbols until detecting the S-SSS symbols, thereby affecting the S-SSS detection performance. The more the PSBCH symbol distributed between the S-PSS and S-SSS symbols, the larger the impact on the S-SSS detection performance will be.

Besides, fewer PSBCH symbols distributed between S-PSS and S-SSS lays a smaller burden upon the UE buffering capability. Also, the S-PSS symbol in close proximity to S-SSS is capable of enhancing the S-SSS detection performance with the assistance of S-PSS. Considering the impact on the S-SSS detection performance and the limited UE buffering capability, the number of PSBCH symbols distributed between S-PSS and S-SSS symbols shall not exceed a certain level. From RAN4 perspective, it is proposed to insert one PSBCH symbol between S-PSS and S-SSS symbols for NR V2X.

Proposal 2: To reduce the performance impact on S-SSS detection, it is proposed to insert one PSBCH symbol between S-PSS and S-SSS.

One S-SSB is composed of S-PSS, S-SSS and PSBCH, among which the S-PSS probably has the highest priority regarding detection performance. It is thus essential to first guarantee the S-PSS detection performance, even slightly scarifying the detection performance of PSBCH. When it comes to the transition between S-PSS and PSBCH, the transient period should be implemented in the PSBCH symbol to reduce the impact on S-PSS detection performance to a large extent.

Proposal 3: The transient period should be implemented in the PSBCH symbol to reduce the impact on S-PSS detection performance.

The increasing progress of communication RF device technology enables RF requirements to be more relaxed. In terms of the transient period between S-PSS and PSBCH, its length can be set to be much shorter compared to that of LTE V2X. In accordance with the transmit ON/OFF mask requirement for NR Uu, the transient period between S-PSS and PSBCH for NR V2X in FR1 should be 10us. To sum up, the proposed S-SSB structure with the transient period for NR V2X is shown in Figure 3.

Figure 3: The proposed S-SSB structure with the transient period for NR V2X

Proposal 4: The transient period for NR V2X in FR1 should be 10us following the transmit ON/OFF time mask requirement for NR Uu.

- Conclusion

This contribution primarily discusses the transient period between S-PSS and S-SSS or PSBCH. To deal with the transient period, one proposed S-SSB structure with the transient period for NR V2X is analyzed from RAN4 perspective. In conclusion, the following proposals concerning the transient period and S-SSB structure are provided for NR V2X.

Observation 1: Alt 2 outperforms Alt 1 considering detection performance of S-PSS, S-SSS and PSBCH.

Proposal 1: Different MPRs used for S-PSS and S-SSS are essential to guarantee the detection performance of S-PSS, S-SSS and PSBCH.

Proposal 2: To reduce the performance impact on S-SSS detection, it is proposed to insert one PSBCH symbol between S-PSS and S-SSS.

Proposal 3: The transient period should be implemented in the PSBCH symbol to reduce the impact on S-PSS detection performance.

Proposal 4: The transient period for NR V2X in FR1 should be 10us following the transmit ON/OFF time mask requirement for NR Uu.

- Reference

[1] 	R1-1909874, LS to RAN4 on NR V2X S-SSB design

[2]	R4-1913064, WF on transient period for S-SSB for V2X, CATT

[3]     	Chairman notes, RAN1#96bis, 8 -12 April 2019.

[4] 	R1-1906317, Sidelink synchronization mechanism in NR V2X, CATT