3GPP TSG-RAN WG4 Meeting #93	R4-1913844

Reno, US 18th – 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Revised the bandwidth combination set defined for n48B CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Verizon, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CA_R16_intra-Core |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN4#92bis R4-1912608 was endorsed. Tdoc contained following proposal Proposal 2: Update the band n48 CA configuration jointly with the bandwidth Class B to support both uplink and downlink CA configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR implements the endorsed proposal 2 into 38.101-1 by revising the bandwidth combination set defined for n48B CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA configuration would not support n48 uplink and downlink CA |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ Start of changes *****************************************

### 5.5A.1	Configurations for intra-band contiguous CA

Table 5.5A.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA for fallback group 1

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |  |
| CA_n41C | - | 40 | 80, 100 |  |  |  | 180 | 0 |  |
|  |  | 50 | 60, 80, 100 |  |  |  |  |  |  |
|  |  | 60 | 50, 60, 80, 100 |  |  |  |  |  |  |
|  |  | 80 | 40, 50, 60, 80, 100 |  |  |  |  |  |  |
|  |  | 100 | 40, 50, 60, 80 |  |  |  |  |  |  |
| CA_n48B | CA_n48B | 52, 10 | 10, 15, 20 |  |  |  | 40 | 0 |  |
|  |  | 15, 20 | 5, 10, 15, 20 |  |  |  |  |  |  |
|  | - | 10 | 50, 60, 80, 90 |  |  |  | 100 | 1 |  |
|  |  | 15, 20 | 40, 50, 60, 80 |  |  |  |  |  |  |
|  |  | 40 | 40, 50, 60 |  |  |  |  |  |  |
|  |  | 50 | 50 |  |  |  |  |  |  |
| CA_n48C | - | 10, 15 | 90, 100 |  |  |  | 150 | 0 |  |
|  |  | 20 | 80, 90, 100 |  |  |  |  |  |  |
|  |  | 40 | 60, 80, 90, 100 |  |  |  |  |  |  |
|  |  | 50 | 50, 60, 80, 90, 100 |  |  |  |  |  |  |
|  |  | 60 | 40, 50, 60, 80, 90 |  |  |  |  |  |  |
|  |  | 80 | 20, 40, 50, 60 |  |  |  |  |  |  |
|  |  | 90 | 10, 15, 20, 40, 50, 60 |  |  |  |  |  |  |
|  |  | 100 | 10, 15, 20, 40, 50 |  |  |  |  |  |  |
| CA_n66B | - | 5 2 | 20, 40 |  |  |  | 50 | 0 |  |
|  |  | 10 | 15, 20, 40 |  |  |  |  |  |  |
|  |  | 15 | 10, 15, 20 |  |  |  |  |  |  |
|  |  | 20 | 5 2, 10, 15 |  |  |  |  |  |  |
|  |  | 40 | 5 2, 10 |  |  |  |  |  |  |
| CA_n71B |  | 5 | 20 |  |  |  | 25 | 0 |  |
|  |  | 10 | 15 |  |  |  |  |  |  |
|  |  | 15 | 10 |  |  |  |  |  |  |
|  |  | 20 | 5 |  |  |  |  |  |  |
| CA_n77C CA_n78C CA_n79C | - | 50 | 60, 80, 100 |  |  |  | 200 | 0 |  |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |  |
| CA_n77D, CA_n78D, CA_n79D | - | 50 | 60, 80, 100 | 100 |  |  | 300 | 0 |  |
|  |  | 60 | 60, 80, 100 | 100 |  |  |  |  |  |
|  |  | 80 | 80, 90, 100 | 100 |  |  |  |  |  |
|  |  | 90 | 100 | 100 |  |  |  |  |  |
|  |  | 100 | 100 | 100 |  |  |  |  |  |
| CA_n77E, CA_n78E, CA_n79E | - | 50 | 60, 80, 100 | 100 | 100 |  | 400 | 0 |  |
|  |  | 60 | 60, 80 | 100 | 100 |  |  |  |  |
|  |  | 80 | 80, 90, 100 | 100 | 100 |  |  |  |  |
|  |  | 90 | 100 | 100 | 100 |  |  |  |  |
|  |  | 100 | 100 | 100 | 100 |  |  |  |  |
| NOTE 1: Unless otherwise stated, minimum requirements are applicable irrespective of the order of the component carriers. NOTE 2: 5 MHz is not applicable for 30/60 kHz SCS. |  |  |  |  |  |  |  |  |

************************************ End of changes *****************************************