3GPP TSG-RAN WG4 Meeting #93	                                                                         R4-1913934

# Reno, Nevada, 18th– 22nd Nov, 2019

Agenda Item:   9.5.5

Source:            AT&T

Title:                 IAB RRM requirements

Document for: Discussion/Approval

# Introduction

As part of the Work Item on Integrated Access and Backhaul for NR [1], 3GPP has agreed to support solutions for efficient operation of integrated access and wireless backhaul for NR. This document provides an overview of the key RRM requirements for IAB nodes.

# IAB RRM Requirements

During RAN4#92 the following WF on RRM requirements for IAB was agreed [2]:

| Requirements | Comments | Is requirement needed? |
| --- | --- | --- |
| Cell reselection | - Intra-frequency Inter-frequency | No |
| Handover |  | FFS |
| RRC Connection Mobility Control | - RRC re-establishment - Random access RRC release with redirection | Yes |
| SCell activation/deactivation delays |  | No |
| Interruptions related to SCells addition, release, activation, deactivation | Interruptions on serving cells due to any of the addition, release, activation, or deactivation procedures | No |
| PSCell addition/release delays |  | No |
| Active BWP switching | - Interruptions at active BWP switching Active BWP switching delay | No |
| TCI switching | TCI switching delay | FFS |
| RLM | Requirements for RLM, e.g., based on SSB, CSI-RS or both | FFS |
| Link recovery procedures | Requirements for SSB-based and CSI-RS based BFD and CBD | FFS |
| Measurements requirements (time duration, number of cells, etc.) |  | FFS |
| Measurement accuracy requirements |  | FFS |
| MT timing related requirements | - MT transmit timing (Te and Tq) Timing Advance | Yes |
|  | - MRTD MTTD | FFS |
| Switching time between MT and DU | --- | TBD in RAN4 RF session |
| DU timing related requirements | 3 us requirement | Yes |
|  | T_delta | FFS |

This contribution discusses the highlighted FFS RRM requirements for the backhaul link in order to efficiently support inter-IAB node discovery and measurements.

## Handover

Although Rel-16 IAB focuses on fixed deployments of IAB nodes, topology adaptation/migration is still a key feature because it can support optimization of the topology based on traffic load and to create redundant paths which are used in case of link failure events. As noted in the 38.300 running CR for IAB [3], these procedures are based on existing Uu handover and RRC connection reestablishment procedures:

#### 4.x.4.2	IAB-node migration

The IAB-node can migrate to a different parent node underneath the same or at a different IAB-donor CU. The IAB-node continues providing access and backhaul service when migrating to a different parent node underneath at least the same IAB-donor CU.

The IAB-node migration procedures are captured in TS 38.401[zz], clause 8.x.

Editor’s Note: For IAB-node migration, Uu handover and connection reestablishment procedures are baseline.

Thus, given that IAB node migration via handover is part of the Rel-16 scope it is critical that performance requirements are defined in RAN4 to ensure that the feature can be deployed and tested (including inter-operability). However, given that the SMTC periodicities for IAB nodes can be larger than for access UEs and DRX is not required for fixed nodes, the requirements can be based on the assumption that a SMTC configuration is provided to IAB nodes performing handover and the measurement period is defined assuming no DRX.

Proposal 1:  Performance requirements are defined in Rel-16 for intra- and inter-frequency handover (delay and interruption time, number of cells, cell detection time) of IAB-MTs which are provided with a SMTC configuration and a measurement period with DRX not in use.

In addition, since the decision to perform a handover of IAB nodes is decided by the network based on periodic, event-triggered periodic, and event-triggered measurement reports from the IAB-MTs, measurement accuracy requirements also need to be defined, to ensure that appropriate triggers can be defined and the performance of the handover procedures. This should apply to both SSB-based and CSI-RS-based measurements, since the use of CSI-RS may reduce the overhead of backhaul link quality measurements.

Proposal 2: IAB-MT measurement reporting requirements are defined in Rel-16 for periodic and event-triggered reporting, as well absolute and relative measurement accuracy requirements for SS-RSRP, SS-RSRQ, and CSI-RS RSRP.

## Backhaul Link Management

In addition to topology management through RRC CONNECTED mobility procedures, backhaul link quality needs to be monitored and in case of failures, recovery procedures initiated as noted in 38.300 [3]:

When RLF occurs at the IAB BH link, the same mechanisms and procedures are applied as for the access link. This includes BH RLF detection and RLF recovery using RRC reestablishment procedure.

In case the RRC reestablishment procedure to recover the BH link fails, the IAB-node transmits an RLF notification message to its child nodes. The child node considers the BH link on which it has received the RLF notification as failed (i.e. as if it has detected RLF on that BH link). The RLF notification message is transmitted on BAP layer.

Editor’s Note: The procedures for backhaul RLF recovery will be captured in TS 38.401[zz], clause 8.x.

As a result, performance requirements related to RLM and beam management/failure recovery as defined in section 8.1 of 38.133 need to also apply for IAB nodes. As with the handover-related requirements, the specific values of the SSB and CSI-RS configurations can be optimized for fixed backhaul links instead of access links.

Proposal 3: IAB-MT RLM/BFD/CBD/L1-RSRP measurement requirements are defined in Rel-16.

# Conclusion

This contribution analyzed the RRM requirements for IAB. The following proposals were made:

Proposal 1: Performance requirements are defined in Rel-16 for intra- and inter-frequency handover (delay and interruption time, number of cells, cell detection time) of IAB-MTs which are provided with a SMTC configuration and a measurement period with DRX not in use.

Proposal 2: IAB-MT measurement reporting requirements are defined in Rel-16 for periodic and event-triggered reporting, as well absolute and relative measurement accuracy requirements for SS-RSRP, SS-RSRQ, and CSI-RS RSRP.

Proposal 3: IAB-MT RLM/BFD/CBD/L1-RSRP measurement requirements are defined in Rel-16.

# Reference

[1] RP-182882, New WID: Integrated Access and Backhaul for NR, Qualcomm

[2] R4-1912829, Updated WF on IAB RRM Requirements, Qualcomm

[3] R2-1913998, IAB Running CR for 38.300, Qualcomm