3GPP TSG-RAN4 Meeting #93 	R4-1913939

# Reno, US, 18 – 22 November, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0153 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for cross link interference measurements (scetion 9) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2019-11-04 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add new section 9.7 for cross link interference measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce CLI section and define measurement requirements for CLI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CLI feature is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | section 9.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----- << Start of Change 1>> -----

9	Measurement Procedure

## 9.7	Cross Link Interference measurements

### 9.7.1	Introduction

The UE capable of performing CLI measurements shall be able to measure SRS-RSRP and CLI-RSSI which are defined in TS38.215 [4] within active DL BWP. The measurements shall be performed for TDD mode only.

When the UE measures SRS-RSRP and CLI-RSSI, the constant offset value to compensate reception timing error between downlink reference timing in the serving cell and SRS arrival timing from the neighbour cell shall be applied. The constant offset value is derived by UE implementation.

### 9.7.2	SRS-RSRP measurements

#### 9.7.2.1	Introduction

When configured by the network, the UE shall be able to perform SRS-RSRP measurements of configured srs-ResourceConfigCLI. The subcarrier spacing for SRS-RSRP measurement resource configuration should be the same as the subcarrier spacing of the active DL BWP of serving cell.

#### 9.7.2.2 	Requirements applicability

The requirements in clause 9.7.2 apply, provided:

- SRS resources configured for SRS-RSRP measurements are measurable.

An SRS resource configured for SRS-RSRP shall be considered measurable when for each relevant SRS the following conditions are met:

- SRS-RSRP related side conditions given in clauses 10.1.22.x for FR1 and FR2, respectively, for a corresponding band,

- SRS_RP and SRS Ês/Iot according to Annex B.2.7 for a corresponding band.

#### 9.7.2.3 	Measurement Reporting Requirements

The UE shall send SRS-RSRP reports only for report configurations according to reportType which is cliPeriodical or cliEventTriggered when SRS-RSRP report is configured.

The UE shall report the SRS-RSRP value as a 7-bit value in the range [-140, -44] dBm with 1dB step size according to clause 10.1.22.x for FR1 and FR2.

##### 9.7.2.3.1	Periodic Reporting

Reported SRS-RSRP measurements contained in periodically triggered measurement reports shall meet the requirements in clause 10.1.22.x.

##### 9.7.2.3.2	Event-triggered Periodic Reporting

Reported SRS-RSRP measurements contained in periodically triggered measurement reports shall meet the requirements in clauses 10.1.22.x.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.7.2.3.3.

##### 9.7.2.3.3	Event Triggered Reporting

Reported SRS-RSRP measurements contained in periodically triggered measurement reports shall meet the requirements in clause 10.1.22.x.

The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report on.

The event triggered measurement reporting delay, shall be less than TSRS_measurement_period defined in clause 9.7.2.5.

#### 9.7.2.4	Measurement capability

The UE shall be capable of performing SRS-RSRP measurements on the SRS resources configured for measurement, provided that the number of SRS to be monitored by UE should not exceed 8 within a slot, and the total number of SRSs to be monitored by the UE should not exceed 32.

#### 9.7.2.5	SRS-RSRP measurement requirements

The UE should be capable of performing SRS-RSRP measurement based on the configured SRS resource, and the UE physical layer shall be capable of reporting SRS-RSRP measured over measurement period of TSRS_RSRP_measurement_period for FR1 and FR2.

Table 9.7.2.5-1 Measurement period TSRS_RSRP_measurement_period

| Configuration | TSRS_measurement_period (ms) |
| --- | --- |
| No DRX | Max(60, [3] X TSRS) |
| DRX cycle ≤ 320ms | Max(60, [3] X max(TSRS, TDRX)) |
| DRX cycle > 320ms | [3] X TDRX |
| Note: TSRS is SRS measurement periodicity configured SRS-PeriodicityAndOffset, and TDRX is the DRX cycle length. |  |

The requirements apply when the SRS resources configured for measurement are not partially or fully overlapped with SMTC window for intra-frequency measurement or a measurement gap for inter-frequency measurement.

### 9.7.3	CLI-RSSI measurements

#### 9.7.3.1	Introduction

When configured by the network, the UE shall be able to perform CLI-RSSI measurement of configured rssi-ResourceConfigCLI. The subcarrier spacing for CLI-RSSI measurement resource configuration can be same or different from the subcarrier spacing of active BWP.

#### 9.7.3.2	Requirements applicability

The requirements in clause 9.7.3 apply, provided:

- The measurement resources configured for CLI-RSSI measurements are measurable.

A measurement resource configured for CLI-RSSI shall be considered measurable when for each relevant CLI-RSSI resource the following conditions are met:

- CLI-RSSI related side conditions given in clauses 10.1.22.x for FR1 and FR2 for a corresponding band.

#### 9.7.3.3	Measurement Reporting Requirements

The UE shall send CLI-RSSI reports only for report configurations according to reportType which is cliPeriodical or cliEventTriggered when CLI-RSSI report is configured.

The UE shall report the CLI-RSSI value as a 7-bit value in the range [-100, -25] dBm with 1dB step size according to clause 10.1.22.x for FR1 and FR2.

##### 9.7.3.3.1	Periodic Reporting

Reported CLI-RSSI measurements contained in periodically triggered measurement reports shall meet the requirements in clause 10.1.22.x.

##### 9.7.3.3.2	Event-triggered Periodic Reporting

Reported CLI-RSSI measurements contained in periodically triggered measurement reports shall meet the requirements in clauses 10.1.22.x.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.7.3.3.3.

##### 9.7.3.3.3	Event Triggered Reporting

Reported CLI-RSSI measurements contained in periodically triggered measurement reports shall meet the requirements in clause 10.1.22.x.

The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report on.

The event triggered measurement reporting delay, shall be less than TCLI_RSSI_measurement_period defined in clause 9.7.3.5.

#### 9.7.3.4	Measurement capability

The UE should be capable of performing CLI-RSSI measurement based on the configured resource, and one or multiple resources for CLI-RSSI measurement can be configured, provided that, the maximum number of CLI-RSSI measurement resources for the UE should not exceed 64.

#### 9.7.3.5	CLI-RSSI measurement requirements

The UE should be capable of performing CLI-RSSI measurement based on the configured measurement resource within TCLI_RSSI_measurement_period. The UE shall be provide to higher layers a single RSSI sample for each measurement resource configured for CLI-RSSI measurement occurring with a configured CLI-RSSI measurement periodicity. The CLI-RSSI measurement period TCLI_RSSI_measurement_period corresponds to RSSI-PeriodicityAndOffset which is configured for the CLI-RSSI measurement by higher layers.

The requirements apply when the CLI-RSSI measurement resources configured for measurement are not partially or fully overlapped with SMTC window for intra-frequency measurement or a measurement gap for inter-frequency measurement.

### 9.7.4	Scheduling availability of UE during CLI measurements

Scheduling availability restrictions when the UE is performing CLI measurements which are SRS-RSRP and CLI-RSSI are described in the following clause.

#### 9.7.4.1	Scheduling availability of UE performing measurement on FR1

The following scheduling restriction applies due to CLI measurements.

- UE is not expected to transmit PUCCH/PUSCH/SRS on CLI measured OFDM symbols, and on [1] data symbol before CLI measured symbol for 15 kHz and 30 kHz subcarrier spacing.

- Depending on UE capability, the UE is not expected to receive PDCCH/PDSCH on CLI measured OFDM symbols, and on [1] data symbol before CLI measured symbol for 15 kHz and 30 kHz subcarrier spacing.

- UE is not expected to transmit PUCCH/PUSCH/SRS on CLI measured OFDM symbols, and on [2] data symbol before CLI measured symbol for 60 kHz subcarrier spacing.

- Depending on UE capability, the UE is not expected to receive PDCCH/PDSCH on CLI measured OFDM symbols, and on [2] data symbols before CLI measured symbol for 60 kHz subcarrier spacing.

When TDD intra-band carrier aggregation is configured, the scheduling restrictions on serving cell where CLI measurements are performed apply on all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

#### 9.7.4.2	Scheduling availability of UE performing measurement on FR2

The following scheduling restriction applies due to CLI measurements.

- UE is not expected to transmit PUCCH/PUSCH/SRS on CLI measured OFDM symbols, and on [1] data symbol before CLI measured symbol for 60 kHz subcarrier spacing.

- Depending on UE capability, the UE is not expected to receive PDCCH/PDSCH on CLI measured OFDM symbols, and on [1] data symbol before CLI measured symbol for 60 kHz subcarrier spacing.

- UE is not expected to transmit PUCCH/PUSCH/SRS on CLI measured OFDM symbols, and on [2] data symbol before CLI measured symbol for 120 kHz subcarrier spacing.

- Depending on UE capability, the UE is not expected to receive PDCCH/PDSCH on CLI measured OFDM symbols, and on [2] data symbols before CLI measured symbol for 120 kHz subcarrier spacing.

When TDD intra-band carrier aggregation is configured, the scheduling restrictions on serving cell where CLI measurements are performed apply on all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

Editor’s note: for UE capability, RAN4 will capture detail description after RAN1 decision.

----- << End of Change 1>> -----