3GPP TSG RAN WG4 #93 meeting                   	R4-1913949

Reno, Nevada, U.S., 18th – 22nd Nov., 2019

Agenda Item:	9.4.4

Source:	LG Electronics

Title:	TP on 5G V2X UE RF requirements for supporting 30MHz CBW and 256QAM at n47

Document for:	Approval

- Introduction

In the last RAN4 #92bis meetings, RAN4 agreed the general RF core requirements for NR V2X service at n47.

Based on RAN1 additional agreements to support 256QAM at last RAN1 meeting and considering the benefits of 30MHz CBW at n47, we provide additional RF core requirements for single carrier operation in ITS spectrum.

****************** Start of the TP in clause 7.2 & 8 of TR38.886 ************************

7.2	Channel bandwidth

7.2.1	Channel bandwidth in FR1

The NR V2X Communication channel bandwidths and operating bands are shown in Table 7.2.1-1. The same (symmetrical) channel bandwidth is specified for both the TX and RX path.

Table 7.2.1-1 NR V2X Communication channel bandwidth

|  |  | NR V2X band / V2X channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X Operating Band | SCS kHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n47 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 | Yes | Yes | Yes | Yes |  |  |  |  |  |

<Unchanged sections are omitted>

8	Transmitter characteristics

8.1	NR V2X UE Tx requirements in FR1

8.1.1	Maximum output power for NR V2X UE

The following V2X UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 8.1-1: V2X UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |  |  |
| n47 |  |  |  |  | 23 | ±2 |  |  |
| NOTE 1: NR Band n47 is used for NR V2X Service. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance |  |  |  |  |  |  |  |  |

8.1.2	UE maximum output power reduction

[Editor Note]: Maximum output power reduction requirements for NR V2X service will be updated after RAN1 decision for the detail physical channel design of each sidelink channel.

The following assumption can serve as a starting point for MPR simulation assumptions.

Table 8.1.2-1: MPR simulation assumption for NR V2X

| parameter | Assumption |
| --- | --- |
| center frequency | 5.9GHz |
| Bandwidth | 10/20/30/40MHz |
| Maximum output power | 23dBm |
| numerology | 15 kHz/30kHz/[60kHz] |
| Modulation | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | [45dBc] |

8.1.3	UE maximum output power with additional requirements

[Editor Note]: Additional transmitted output power requirements for NR V2X service will be added after RAN1 decision for the detail physical channel design of each sidelink channel.

8.1.4	Configured transmitted power for NR V2X UE

[Editor Note]: Additional configured transmitted power requirements for NR V2X service will be added after RAN1 decision for the detail power control method of each sidelink channel.

8.1.5	Minimum output power for NR V2X UE

The minimum controlled output power of the NR V2X UE is defined as the broadband transmit power of the UE, i.e. the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value.

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands, it is proposed that the existing requirements as specified for legacy NR UE shall apply. The minimum output power is defined as the mean power in at least one sub-frame 1 ms.

Table 8.1.5-1: Minimum output power

| Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 10 | [-30] | 9.375 |
| 20 | [-30] | 19.095 |
| 30 | [-38.2] | 28.815 |
| 40 | [-27] | 38.895 |

8.1.6	Transmit OFF power NR V2X UE

Transmit OFF power is defined as the mean power when the transmitter is OFF. The transmitter is considered to be OFF when the UE is not allowed to transmit or during periods when the UE is not transmitting a sub-frame. During DTX and measurements gaps, the UE is not considered to be OFF.

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands, it is proposed that the existing requirements as specified for legacy NR UE shall apply. Transmit off power is defined as the mean power in at least one sub-frame 1 ms.

Table 8.1.6-1: Transmit OFF power

| Channel bandwidth (MHz) | Transmit OFF power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 10 | -50 | 9.375 |
| 20 | -50 | 19.095 |
| 30 | -50 | 28.815 |
| 40 | -50 | 38.895 |

8.1.7	ON/OFF time mask for NR V2X UE

For NR V2X service, additional requirements on ON/OFF time masks for V2X physical channels and signals are specified in this clause.

The existing general time mask requirements of NR UE in sub-clause 6.3.3.2 in TS38.101-1 apply for NR V2X sidelink UE.

8.1.8	Power control for NR V2X UE

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands, the requirements in sub-clause 8.1.8.1 apply for NR V2X transmission.

- 8.1.8.1	Absolute power tolerance

The existing absolute power tolerance requirements of legacy NR UE shall apply for V2X transmission.

Table 8.1.8.1-1: Absolute power tolerance

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 9.0 dB |

8.1.9	Transmit signal quality for NR V2X UE

- 8.1.9.1	Frequency error

The UE modulated carrier frequency for NR V2X sidelink transmissions shall be accurate to within ±0.1 PPM observed over a period of 1 ms compared to the absolute frequency in case of using GNSS synchronization source. The same requirements applied over a period of 1 ms compared to the relative frequency in case of using the gNB or V2X UE sidelink synchronization signals.

- 8.1.9.2	Transmit modulation quality

- 8.1.9.2.1	 Error Vector Magnitude

For V2X sidelink physical channels PSCCH and PSSCH, the Error Vector Magnitude requirements shall be as specified for PUSCH in Table 8.1.9.2.1-1 for the corresponding modulation and transmission bandwidth. When sidelink transmissions are shortened due to transmission gap of 1 symbol at the end of the slot, the EVM measurement interval is reduced by one symbol, accordingly.

Table 8.1.9.2.1-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level |
| --- | --- | --- | --- |
| QPSK or BPSK | % | 17.5 | 17.5 |
| 16QAM | % | 12.5 | 12.5 |
| 64QAM | % | 8 | 8 |
| 256QAM | % | 3.5 | 3.5 |

Table 8.1.9.2.1-2: Parameters for Error Vector Magnitude

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | Table 8.1.5-1 |
| UE Output Power for 256QAM | dBm | Table 8.1.5-1 + 10dB |
| Operating conditions |  | Normal conditions |

- 8.1.9.2.2	 Carrier leakage

It is proposed that no changes to the existing NR requirements for carrier leakage are required for NR V2X UE.

- 8.1.9.2.3 In-band emissions

For V2X sidelink physical channels PSCCH and PSSCH, In-band emissions requirements of legacy NR UE in sub-clause 6.4.2.3 shall apply according to the corresponding modulation and transmission bandwidth. When V2X transmissions are shortened due to transmission gap of 1 symbol at the end of the slot, the In-band emissions measurement interval is reduced by one symbol, accordingly.

- 8.1.9.2.4 EVM equalizer spectrum flatness

The existing requirements for EVM equalizer spectrum flatness for NR UE in sub-clause 6.4.2.4 in TS38.101-1 shall apply for NR V2X transmissions.

8.1.10	spectrum emission mask for V2X UE

For V2X, the existing NR general spectrum emission mask can be reused for all 10MHz, 20MHz, 30MHz and 40MHz channel bandwidth. The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies greater than (ΔfOOB), the power of any UE emission shall not exceed the levels specified in Table 6.5.2.2-1in TS 38.101-1 for the specified channel bandwidth.

- 8.1.10.1	Additional spectrum emission mask for V2X UE

In LTE V2X WI, RAN4 already studied the additional spectrum emission mask in ITS band. So RAN4 reuse the additional spectrum emission mask in session 6.6.2.2.4 and 6.6.2.2.7 in TS36.101 for LTE V2X UE.

When "NS_33" or “NS_34” is indicated in the cell, the power of any V2X UE emission shall not exceed the levels specified in Table 8.1.10.1-1.

Table 8.1.10.1-1: Additional spectrum mask requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

8.1.11	ACLR requirements for V2X UE

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

Based on co-existence simulation results, a 30dB ACLR can guarantee that V2X co-exists well with DSRC, i.e. the PRR degradation of V2X to DSRC is comparable with that of DSRC to DSRC. Hence the existing ACLR requirement for licensed bands can be reused for NR V2X.

Table 8.1.11-1: ACLR for NR V2X

|  |  |  | Channel bandwidth /ACLR1 / Measurement bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 40 MHz |
| ACLR1 |  |  |  |  | 30 dB |  | 30 dB | 30 dB | 30 dB |
| NR V2X channel Measurement bandwidth |  |  |  |  | 9.375 MHz |  | 19.095 MHz | 28.815 MHz | 38.895 MHz |
| Adjacent channel centre frequency offset [MHz] |  |  |  |  | +10 / -10 |  | +20 / -20 | +30 / -30 | +40 / -40 |

8.1.12	Spurious emission for V2X UE

General spurious emission requirement for V2X operation should be extended to 26GHz same with that in LAA BS. In addition, UE should also meet regional requirement which defined in ETSI EN 302 571 as additional spurious emission requirement.

Table 8.1.12-1: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |  |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz - 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for NR Band n22, Band n42 and Band n43. NOTE 2: Applies for NR Band n47. |  |  |  |

8.1.13	Spurious emission band UE co-existence for V2X UE

This clause specifies the spurious emission requirements for the specified NR V2X band, for coexistence with protected bands

Table 8.1.13-1: Requirements

| NR Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| … |  |  |  |  |  |  |  |
| n47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 1,2,3 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 1,3 |
| NOTE 1: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 2: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 3: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. |  |  |  |  |  |  |  |

In Note 2, the frequency range with -30dBm/MHz will be further discussed

8.1.14	Transmit intermodulation for NR V2X UE

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands, the requirements in sub-clause 6.5.4 in TS38.101-1 apply for NR V2X sidelink transmission.

8.2	NR V2X UE Tx requirements in FR2

9	Receiver characteristics

9.1	NR V2X UE Rx requirements in FR1

9.1.1	Reference sensitivity power level

The reference sensitivity power level REFSENS is the minimum mean power applied to the UE antenna connector at which the throughput shall meet or exceed 95% of the maximum throughput of the reference measurement channels.

The V2X UE REFSENS is defined by the following equation:

REFSENSV2X=kTB + SNRV2X +10log10(LCRB/NRB) +( NFV2X+ IM)

Where

-	kTB: Thermal noise level in units of dBm. -104 dBm, -101 dBm. -99.2dBm and -98dBm can be used for 10MHz, 20MHz, 30MHz and 40MHz V2X requirements, separately.

-	NF: Noise figure. 13 dB is used for LAA and can be reused for NR V2X requirements.

-	IM: 2.5 dB is assumed.

-	Other parameters are FFS

9.1.2	Maximum input level

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

For V2X, the requirement are defined for 10MHz, 20MHz, 30MHz and 40MHz. The parameters of reference measurement channel shall be changed according to the physical layer design specific to NR V2X.

Table 9.1.2-1: Maximum input level for V2X

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 10 MHz | 20 MHz | 30 MHz | 40 MHz |
| Power in Transmission Bandwidth Configuration | dBm | [-22~ -25] |  |  |  |  |  |  |
| NOTE 1: Reference measurement channel is FFS |  |  |  |  |  |  |  |  |

[9.1.3	Adjacent Channel Selectivity (ACS)]

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel.

ACS reflects the digital domain filter attenuation capability, which is model together with ACLR in ACIR. Based on PRR metric. In co-existence study, the existing NR ACS requirement  reuse for both NR UE to V2X UE and DSRC UE to V2X UE urban interference scenarios, PRR loss is less than 5%. Therefore, it is proposed that the same ACS values shall be kept unchanged for V2X.

As 10MHz is typical channel bandwidth used in the V2X band, the interferer bandwidth shall be used with 10MHz.

Table 9.1.3-1: Adjacent channel selectivity for V2X

|  |  | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 40 MHz |
| ACS | dB |  |  |  | 33.0 |  | 27.0 | 25.5 | 24.0 |

Table 9.1.3-2: Test parameters for Adjacent channel selectivity for V2X, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 40 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + 14 dB |  |  |  |  |  |  |  |
| PInterferer | dBm |  |  |  | REFSENS +45.5dB |  | REFSENS +39.5dB | REFSENS +38.0 dB | REFSENS +36.5dB |
| BWInterferer | MHz |  |  |  | 10 |  | 10 | 10 | 10 |
| FInterferer (offset) | MHz |  |  |  | 10 / -10 |  | 15 / -15 | 20 / -20 | 25 / -25 |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. |  |  |  |  |  |  |  |  |  |

Table 9.1.3-3: Test parameters for Adjacent channel selectivity for V2X, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 40 MHz |
| Power in Transmission Bandwidth Configuration | dBm |  |  |  | -53.5 |  | -47.5 | -46 | -44 |
| PInterferer | dBm | [-22 ~ -25] |  |  |  |  |  |  |  |
| BWInterferer | MHz |  |  |  | 10 |  | 10 | 10 | 10 |
| FInterferer (offset) | MHz |  |  |  | 10 / -10 |  | 15 / -15 | 20 / -20 | 25 / -25 |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. |  |  |  |  |  |  |  |  |  |

[9.1.4	Blocking characteristics]

- [9.1.4.1	In-band blocking]

In EN 302 571, the blocking level shall not be less than -30 dBm. And the blocking is tested at ± 50 MHz, ± 100 MHz, and ± 200 MHz frequency offset. Blocking testing shall be performed at least at 6 different frequency offset positions. The blocking level is larger than exiting case 2 blocking interferer in the specification, which is in the second alternate channel offset from the wanted signal. Considering the total 70MHz band width of the V2X band, ± 50 MHz is already outside the operating band or close at the band edge depends on the placement of the carrier.

RAN4 reuse the In-band blocking requirements in LTE V2X UE as shown in Table 9.1.4.1-1 and Table 9.1.4.1-2 NR V2X.

Table 9.1.4.1-1: In band blocking parameters for V2X

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 40MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 6 |  |  |  | 9 |  | 11 | 12 |
| BWInterferer | MHz |  |  |  | 10 |  | 10 | 10 | 10 |
| FIoffset, case 1 | MHz |  |  |  | 15 |  | 15 | 15 | 15 |
| FIoffset, case 2 | MHz |  |  |  | 25 |  | 25 | 25 | 25 |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. |  |  |  |  |  |  |  |  |  |

Table 9.1.4.1-2: In-band blocking for V2X

| NR band | Parameter | Unit |  | Case 1 | Case 2 |
| --- | --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 |  | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 |  | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| n47 | FInterferer | MHz |  | (NOTE 2) | FDL_low – 30 to FDL_high + 30 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band. NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies NOTE 4: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. |  |  |  |  |  |

- [9.1.4.2	Out-of-band blocking]

Out-of-band band blocking in existing specification for licensed bands is defined for an unwanted CW interfering signal falling more than 15 MHz below or above the UE receive band. For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity shall be applied.

RAN4 reuse the Out-of- band blocking of LTE V2X UE as shown in Table 9.1.4.2-1 and Table 9.1.4.2-2

Table 9.1.4.2-1: Out-of-band blocking parameters

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 40 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |  |  |  |  |
|  |  |  |  |  | 6 |  | 9 | 11 | 12 |
| NOTE 1: Reference measurement channel is FFS |  |  |  |  |  |  |  |  |  |

Table 9.1.4.2-2: Out of band blocking

| NR band | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| n47 | FInterferer (CW) | MHz | FDL_low -30 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +30 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| NOTE: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 4400 MHz. |  |  |  |  |  |

[9.1.5	Spurious response]

Spurious response is a measure of the receiver's ability to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained.

For V2X, 10MHz, 20MHz, 30MHz and 40MHz channel bandwidths are considered. RAN4 reuse the spurious response of LTE V2X UE as shown in Table 9.1.5-1 and Table 9.1.5-2 for V2X.

Table 9.1.5-1: Spurious response parameters for V2X

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz | 40 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 6 |  | 9 | 11 | 12 |
| NOTE 1: Reference measurement channel is FFS |  |  |  |  |  |  |  |  |  |  |

Table 9.1.5-2: Spurious response for V2X

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

[9.1.6	Intermodulation characteristics]

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

The wide band intermodulation requirement is defined using modulated NR carrier and CW signal as interferer. 10MHz channel bandwidth will be chosen as the modulated interfering signal for both 10MHz, 20MHz, 30MHz and 40MHz channel bandwidth.

RAN4 reuse the intermodulation characteristics of LTE V2X UE as shown in Table 9.1.6-1 for V2X.

Table 9.1.6-1: Wide band intermodulation

| NR band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 1.4 MHz |  | 3 MHz |  | 5 MHz | 10 MHz |  | 15 MHz |  | 20 MHz |  | 30 MHz | 40 MHz |
| n47 |  |  |  | Power in Transmission Bandwidth Configuration |  |  |  | dBm |  |  |  | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6 |  |  |  | 9 |  | 11 | 12 |
|  |  |  | PInterferer 1 (CW) |  |  |  | dBm |  |  |  | -46 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | PInterferer 2 (Modulated) |  |  |  | dBm |  |  |  | -46 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | BWInterferer 2 |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 |  |  |  | 10 | 10 | 10 |
|  |  |  |  |  | FInterferer 1 (Offset) |  |  |  | MHz |  |  |  |  |  |  |  |  |  | -BW/2 – 15 / +BW/2 + 15 |  |  |  | -BW/2 – 15 / +BW/2 + 15 | -BW/2 – 15 / +BW/2 + 15 | -BW/2 – 15 / +BW/2 + 15 |
|  | FInterferer 2 (Offset) |  |  |  |  | MHz |  |  |  | 2*FInterferer 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Reference measurement channel is FFS NOTE 2: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. |  |  |  |  |  |  |  |  |  |  |  |  |

9.2	NR V2X UE Rx requirements in FR2

<End of Changes>