3GPP TSG RAN WG4 #93 meeting                   	R4-1913953

Reno, Nevada, U.S., 18th – 22nd Nov., 2019

Agenda Item:	9.4.4

Source:	LG Electronics

Title:	TP on 5G V2X UE RF requirements for con-current operations

Document for:	Approval

- Introduction

In the last RAN4 #92bis meetings, RAN4 agreed the general RF core requirements for NR V2X service at n47.

Also the operating scenarios are approved as following

- 1st priority of NR V2X operation:

- Specify RF core requirements in the ITS spectrum,

- Specify RF core requirements for NR SL (at n47) and LTE SL (at B47) as TDM for Tx transmission and simultaneous receptions,

- Specify RF core requirements for licensed bands in which the entire band is allocated for SL operation in a region.

  - Sidelink operation in partial bandwidth in licensed band is not precluded in WI

- 2nd priority in con-current operation:

- Case 3A: NR V2X PC5 (at n47) + NR/LTE Uu (at licensed bands) on inter-band con-current operation

  - NR V2X PC5 (at n47) + NR Uu(at licensed bands)

  - NR V2X PC5 (at n47) + LTE Uu(at licensed bands)

- Case 3C: NR V2X SL (at licensed band Y) + LTE/NR Uu (at licensed band X) on inter-band con-current operation

  - NR V2X PC5 (at licensed band Y) + NR Uu(at licensed band X)

  - NR V2X PC5 (at licensed band Y) + LTE Uu(at licensed band X)

- 3rd Priority for con-current operation (without ITS spectrum):

- Case 3B: LTE Band 47 PC5 + NR Uu (at licensed bands) : need to study on RAN4 impact when NR Uu control the LTE sidelink operation since the agreed WID did not reflect RAN4 impact

- Case 3E: NR V2X PC5 (at licensed band) + LTE/NR Uu (at licensed band) on intra-band combination

  - NR V2X PC5 (at licensed band X) + LTE Uu (at licensed band X)

  - NR V2X PC5 (at licensed band X) + NR Uu (at licensed band X)

In this paper, we provide the NR V2X RF core requirements for con-current operations to address 2nd priority V2X operation.

****************** Start of the TP in clause 7 & 8 of TR38.886 ************************

7	Operating bands and channel arrangement

7.1	Operating bands

7.1.1	Operating bands in FR1

NR V2X communication is designed to operate in the operating bands in FR1 defined in Table 7.1.1-1.

Table 7.1.1-1 NR V2X operating bands in FR1

| NR V2X Operating Band | Sidelink (SL) Transmission operating band |  |  | Sidelink (SL) Reception operating band |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| n47 | 5855 MHz | - | 5925 MHz | 5855 MHz | - | 5925 MHz | TDD | PC5 |

7.1.2	Operating bands in FR2

7.1.3	Operating bands for inter-band con-current operation in FR1

NR V2X operation is designed to operate concurrent with NR uplink/downlink on the operating bands combinations listed in Table 7.1.3-1.

Table 7.1.3-1: NR V2X inter-band con-current operation

| V2X con-current operating Band | V2X Band | Interface |
| --- | --- | --- |
| NR V2X_nX-n47 | nX | Uu |
|  | n47 | PC5 |
| NOTE 1: NR licensed Band is operated for legacy NR operation and/or V2X service |  |  |

7.2	Channel bandwidth

<Unchanged sections are omitted>

7.2.3	Channel bandwidth for inter-band con-current operation in FR1

For NR V2X inter-band con-current operation in FR1, the NR V2X channel bandwidths for each operating band is specified in Table 7.2.3-1.

Table 7.2.3-1: NR V2X inter-band con-current configurations

| V2X con-current operating band Configuration | NR Bands | SCS kHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | Maximum bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X_nXA-n47A | nX | 15 | Yes | Yes |  |  |  | 60 | 0 |
|  |  | 30 | Yes | Yes |  |  |  |  |  |
|  |  | 60 | Yes | Yes |  |  |  |  |  |
|  | n47 | 15 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 30 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 60 | Yes | Yes | Yes | Yes |  |  |  |

<Unchanged sections are omitted>

10	Tx/Rx characteristics of concurrent operation

10.1	NR V2X SL operation in 5.9GHz or other potential ITS spectrum and LTE/NR Uu operation in licensed band for FR1

10.1.1	Tx requirements for inter-band con-current NR V2X operation

Maximum output power

For the inter-band con-current NR V2X operation, the following NR V2X UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 10.1.1.1-1: Con-current NR V2X UE Power Class for uplink inter-band combination (two bands)

| NR V2X con-current operating band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X_nXA-n47A |  |  |  |  | 23 | +2/-34 |  |  |
| NOTE 1: The con-current band combinations is used for NR V2X Service. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band con-current aggregation the maximum power requirement apply to the total transmitted power over all component carriers (per UE). NOTE 4: 4 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |

UE maximum output power reduction

For the inter-band con-current NR V2X operation, the allowed maximum power reduction (MPR) for the maximum output power shall be applied per each component carrier. The legacy MPR requirements for NR Uu operation shall be applied on the CC of licensed band, the MPR requirements for NR sidelink operation shall be applied on CC of NR Band n47.

UE maximum output power with additional requirements

For the inter-band con-current NR V2X operation, the allowed additional maximum power reduction (A-MPR) for the maximum output power shall be applied per each component carrier. The legacy A-MPR requirements for NR Uu operation shall be applied on CC of licensed band, the A-MPR requirements for NR sidelink operation shall be applied on CC of NR Band n47.

Configured transmitted power

For the inter-band con-current NR V2X operation, the configured transmitted power requirements for NR V2X UE shall be applied per each component carrier. The legacy NR configured transmitted power for inter-band NR CA shall be decided by MPRc and A-MPRc of each CC.

If the total transmitted power is over the power class of the NR V2X UE or given PEMAX,c for NR V2X service, the UE shall compare the priority between the legacy NR service on licensed band and NR V2X service at n47. Based on the priority, the configured transmitted power will be decided which transmission will be applied on power scale down or dropping between the legacy NR Uu transmission and NR V2X SL transmission.

UE Minimum output power

Expected no need to change the requirements for inter-band con-current NR V2X UE. The legacy minimum output power requirements will be applied on CC of NR licensed band and the NR V2X minimum output power requirements will be applied on CC of NR Band n47.

Transmit OFF power

Expected no need to change the requirements for inter-band con-current NR V2X UE. The legacy transmit OFF power requirements will be applied on CC of NR licensed band and the NR V2X transmit off power requirements will be applied on CC of NR Band n47.

ON/OFF time mask

When UE support inter-band con-current NR V2X operation in Table 10.1-1, the existing general time mask for NR UE will be applied on CC of NR licensed band. Also the existing general time mask for NR UE and additional requirements on ON/OFF time mask will be applied on CC of NR Band n47.

Power control

Expected no need to change the requirements for inter-band con-current NR V2X UE. The legacy power control requirements will be applied on CC of NR licensed band and the NR V2X power control requirements will be applied on CC of NR Band n47.

Transmit signal quality

Frequency error

Expected no need to change the requirements for inter-band con-current NR V2X UE. The legacy frequency error requirements will be applied on CC of NR licensed band and NR V2X frequency error requirements will be applied on CC of NR Band n47.

Transmit modulation quality

Error Vector Magnitude

Expected no need to change the requirements for inter-band con-current NR V2X UE. The legacy EVM requirements will be applied on CC of NR licensed band and the NR V2X EVM requirements will be applied on CC of NR Band n47.

Carrier leakage

Expected no need to change the requirements for inter-band con-current NR V2X UE. The legacy carrier leakage requirements will be applied on CC of NR licensed band and the NR V2X carrier leakage requirements will be applied on CC of NR Band n47.

In-band emissions

Expected no need to change the requirements for inter-band con-current NR V2X UE. The legacy in-band emission requirements will be applied on CC of NR licensed band and the NR V2X in-band emission requirements will be applied on CC of NR Band n47.

EVM equalizer spectrum flatness

Expected no need to change the requirements for inter-band con-current NR V2X UE. The legacy spectrum flatness requirements will be applied per carrier for inter-band con-current V2X operation.

Spectrum emission mask

Expected no need to change the requirements for inter-band con-current NR V2X UE.  The legacy general/additional SEM requirements will be applied on CC of NR licensed band and NR V2X general/additional SEM requirements will be applied on CC of NR Band n47.

ACLR requirements

Expected no need to change the requirements for inter-band con-current NR V2X UE.  The legacy ACLR requirements will be applied on CC of NR licensed band and NR V2X ACLR requirements will be applied on CC of NR Band n47.

Spurious emission requirements

Expected no need to change the requirements for inter-band con-current NR V2X UE.  The legacy general spurious emission requirements will be applied on CC of NR licensed band and NR V2X general spurious emission requirements will be applied on CC of NR Band n47.

Spurious emission band UE co-existence

This clause specifies the spurious emission requirements of the inter-band con-current V2X operation, for UE-to-UE coexistence to protect legacy protected bands

Table 10.1.1.13-1: Requirements

| NR V2X con-current operating band cofiguration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| NR V2X_nXA-n47A | E-UTRA Band 1, 5, 7, 8, 26, 28, 34, 39, 40, 44, 45, 65, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 22, 41, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| NOTE 1: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 2: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 3: Applicable when NS_XX is configured by the pre-configured radio parameters for power class 3 V2X UE. NOTE 4: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. |  |  |  |  |  |  |  |

Transmit intermodulation

Expected no need to change the requirements for inter-band con-current NR V2X UE.  The legacy transmit intermodulation requirements will be applied on CC of NR licensed band and NR V2X transmit intermodulation requirements will be applied on CC of NR Band n47.

    - Rx requirements for inter-band con-current NR V2X operation

REFSENS

For the NR V2X UE RF receiver requirements, RAN4 can refer the 2DL inter-band CA to define general UE RF Rx requirements for inter-band con-current NR V2X UE.

The legacy REFSENS requirement will be applied on CC of NR licensed band and NR V2X REFSENS requirements will be applied on CC of NR Band n47 if there was no self-interference problems in own receiver frequency band by own uplink and sidelink transmission.

Table 10.1.2.1-1 and Table 10.1.2.1-2 propose the uplink test configurations for inter-band con-current NR V2X REFSENS requirements. For the uplink configuration, RAN4 consider 10MHz Channel bandwidth.

Table 10.1.2.1-1: Uplink configuration for reference sensitivity of NR V2X bands (PC5)

| Inter-band NR V2X con-current band configuration |  | NR UL band / Channel BW / NRB / Duplex mode |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NR V2X band (PC5) | NR band (Uu) | NR UL band | Channel Bandwidth (MHz) | NRB | Duplex Mode |
| n47 | nX | nX | 10 | 50 | TDD or FDD |

Table 10.1.2.1-2: SL Tx configuration for reference sensitivity of NR V2X bands (Uu)

| Inter-band NR V2X con-current band configuration |  | NR UL band / Channel BW / NRB / Duplex mode |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NR V2X band (PC5) | NR band (Uu) | NR V2X band (PC5) | Channel Bandwidth (MHz) | NRB | Duplex Mode |
| n47 | nX | nX | 10 | 50 | HD |

Table 10.1.2.1-3 is proposed UE REFSENS requirements with inter-band con-current NR V2X UE reception.

Table 10.1.2.1-3: Reference sensitivity for NR V2X QPSK PREFSENS

| Inter-band V2X reception |  | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | NR band | NR Band | SCS (kHz) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| n47 | NR Band nX | nX | 15 | TBD | TBD | TBD | TBD |  |  | TDD or FDD |
|  |  |  | 30 | TBD | TBD | TBD | TBD |  |  |  |
|  |  |  | 60 | TBD | TBD | TBD | TBD |  |  |  |
|  |  | n47 | 15 | TBD |  | TBD |  | TBD | TBD | HD |
|  |  |  | 30 | TBD |  | TBD |  | TBD | TBD |  |
|  |  |  | 60 | TBD |  | TBD |  | TBD | TBD |  |

Maximum input level

No need to define additional requirements for con-current inter-band V2X operation since NR UE shall meet the requirements per each carrier while all downlink carriers are active.

The other Rx requirements

The other Rx requirements for inter-band con-current NR V2X UE can be reused. The legacy other RX requirement will be applied on CC of NR licensed band and legacy NR V2X other RX requirements will be applied on CC of NR Band n47 since there was no difference on UE receiver aspect.

- ACS: Keep the same requirements per CC

- In band Blocking: Keep the same requirements per CC. For the inter-band con-current NR V2X UE, PInterferer power is increased by ΔRIB,c in the requirement.

- Out-of-Band Blocking: Keep the same requirements per CC. For the inter-band con-current NR V2X UE, PInterferer power is increased by ΔRIB,c in the requirement.

- Narrow Band Blocking: NR NBB requirements only applied on CC of NR licensed band and no requirements for NR Band n47.

- Spurious response: Keep the same requirements per CC. For the inter-band con-current NR V2X UE, PInterferer power is increased by ΔRIB,c in the requirement.

- Wideband inter-modulation: Keep the same requirements per CC. For the inter-band con-current NR V2X UE, PInterferer power is increased by ΔRIB,c in the requirement.

10.2	NR V2X SL operation and LTE/NR Uu operation both in licensed band for FR1

10.3	NR V2X SL operation in 5.9GHz or other potential ITS spectrum and NR Uu operation in licensed bands for FR2

10.4	NR V2X SL operation and NR Uu operation both in licensed bands for FR2

<End of Changes>