3GPP TSG-RAN WG4 Meeting #93	R4-1913984

Reno, US 18th – 22nd Nov 2019

Source: 	Nokia, Nokia Shanghai Bell

Title: 					Discussion on reply LS on BCS reporting for EN-DC BC

Agenda Item: 			7.5.2

Document for:	Approval

# 1	Introduction

RAN4 received LS from RAN2 on BCS reporting for EN-DC BC. In this LS RAN2 asked some guidance from RAN4.

RAN2 observe that the existing BCS signalling cannot support the above EN-DC configuration and so RAN2 is working on extending the UE capability signalling. To support it, RAN2 understands that the following types of BCS need to be signalled, at most.

1)	BCS for LTE CA component

2)	BCS for NR CA component

3)	BCS for intra-band EN-DC component

Nevertheless, RAN2 is not sure the generic principle for what condition these three fields have to be included for an EN-DC band combination. For instance, for BCS reporting for EN-DC BC, RAN2 would like to ask RAN4 to clarify how to report BCS for EN-DC BC including:

- Intra-band EN-DC without LTE CA or NR CA, e.g. DC_(n)41AA, DC_3A_n3A

- Intra-band EN-DC with LTE CA and/or NR CA inside, e.g. DC_(n)41CA, DC_41C_n41A

- Intra-band EN-DC with additional inter-band LTE CA and/or NR CA, e.g. DC_2A-(n)71AA, DC_(n)41AA_n77C

# 2	Discussion

Next we discuss the questions one by one.

- Intra-band EN-DC without LTE CA or NR CA, e.g. DC_(n)41AA, DC_3A_n3A

Answer: RAN4 has defined BCS for intraband EN-DC configurations in sub-clause 5.3B.1 of 38-101-3, this BCS needs to be signalled

- Intra-band EN-DC with LTE CA and/or NR CA inside, e.g. DC_(n)41CA, DC_41C_n41A

Answer: Same as above

- Intra-band EN-DC with additional inter-band LTE CA and/or NR CA, e.g. DC_2A-(n)71AA, DC_(n)41AA_n77C

Answer: Intraband EN-DC as above, bands without CA no need for BCS and bands with CA same CA BCS as in SA mode

# 3	Conclusion

We propose to send a reply LS to RAN2 on BCS reporting for EN-DC BC. Content of the LS is summarized below.

- Intra-band EN-DC without LTE CA or NR CA, e.g. DC_(n)41AA, DC_3A_n3A

Answer: RAN4 has defined BCS for intraband EN-DC configurations in sub-clause 5.3B.1 of 38-101-3, this BCS needs to be signalled

- Intra-band EN-DC with LTE CA and/or NR CA inside, e.g. DC_(n)41CA, DC_41C_n41A

Answer: Same as above

- Intra-band EN-DC with additional inter-band LTE CA and/or NR CA, e.g. DC_2A-(n)71AA, DC_(n)41AA_n77C

Answer: Intraband EN-DC as above, bands without CA no need for BCS and bands with CA same CA BCS as in SA mode

# 4	References

[1] R2-1914226, LS on BCS reporting for EN-DC BC, RAN2