3GPP TSG-RAN WG4 Meeting #93	R4-1914101

Reno, US, 18th– 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | 0093 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.104: Introduction of NR PRACH performance requirements for restricted set type A and B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST-Perf |  |  |  |  |  | Date: |  |  | 2019-11-8 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is to introduce PRACH requirements for restricted set type A and B under conditions assuming a UE velocity of up to 350km/h. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new test cases of PRACH for restricted set type A and B assuming a UE velocity up to 350km/h |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PRACH performance for under high speed conditions assuming a UE velocity of up to 350km/h is not ensured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.141-1, TS 38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of text proposal-------------

8.4.2	PRACH detection requirements

8.4.2.1	General

The probability of detection is the conditional probability of correct detection of the preamble when the signal is present. There are several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. For AWGN and TDLC300-100, a timing estimation error occurs if the estimation error of the timing of the strongest path is larger than the time error tolerance given in Table 8.4.2.1-1.

The requirements for high speed mode restricted set type A (table 8.4.2.2-4) and high speed mode restricted set type B (table 8.4.2.2-5) are only valid for the base stations supporting high speed mode restricted set A and restricted set type B respectively.

Table 8.4.2.1-1: Time error tolerance for AWGN and TDLC300-100

| PRACH preamble | PRACH SCS (kHz) | Time error tolerance |  |
| --- | --- | --- | --- |
|  |  | AWGN | TDLC300-100 |
| 0 | 1.25 | 1.04 us | 2.55 us |
| A1, A2, A3, B4, C0, C2 | 15 | 0.52 us | 2.03 us |
|  | 30 | 0.26 us | 1.77 us |

The test preambles for normal mode are listed in table A.6-1 and the test parameter msg1-FrequencyStart is set to 0.

8.4.2.2	Minimum requirements

The probability of detection shall be equal to or exceed 99% for the SNR levels listed in Tables 8.4.2.2-1 to 8.4.2.2-3.

Table 8.4.2.2-1: PRACH missed detection requirements for Normal Mode, 1.25 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | AWGN | 0 | [-14.5] |
|  |  | TDLC300-100 Low | 400 Hz | [-6.6] |
|  | 4 | AWGN | 0 | [-16.7] |
|  |  | TDLC300-100 Low | 400 Hz | [-11.9] |
|  | 8 | AWGN | 0 | [-18.9] |
|  |  | TDLC300-100 Low | 400 Hz | [-15.8] |

Table 8.4.2.2-2: PRACH missed detection requirements for Normal Mode, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format A1 | Burst format A2 | Burst format A3 | Burst format B4 | Burst format C0 | Burst format C2 |
| 1 | 2 | AWGN | 0 | [-9.3] | [-12.6] | [-14.2] | [-16.8] | [-6.3] | [-12.5] |
|  |  | TDLC300-100 Low | 400 Hz | [-2.1] | [-4.8] | [-6.6] | [-8.8] | [0.8] | [-4.9] |
|  | 4 | AWGN | 0 | [-11.6] | [-14.3] | [-16.0] | [-19.0] | [-8.7] | [-14.1] |
|  |  | TDLC300-100 Low | 400 Hz | [-7.3] | [-10.3] | [-11.7] | [-13.8] | [-4.3] | [-10.2] |
|  | 8 | AWGN | 0 | [-13.8] | [-16.7] | [-18.2] | [-21.2] | [-11.1] | [-16.6] |
|  |  | TDLC300-100 Low | 400 Hz | [-11.0] | [-13.9] | [-15.2] | [-17.3] | [-8.1] | [-13.9] |

Table 8.4.2.2-4: PRACH missed detection requirements for High speed Mode restricted set type A, 1.25 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | AWGN | [0] Hz | TBD |
|  |  | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | [625] Hz | TBD |
|  |  | AWGN | 1340 Hz | TBD |
|  | 4 | AWGN | [0] Hz | TBD |
|  |  | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | [625] Hz | TBD |
|  |  | AWGN | 1340 Hz | TBD |
|  | 8 | AWGN | [0] Hz | TBD |
|  |  | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | [625] Hz | TBD |
|  |  | AWGN | 1340 Hz | TBD |

Table 8.4.2.2-5: PRACH missed detection requirements for High speed Mode restricted set type B, 1.25 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | AWGN | [0] Hz | TBD |
|  |  | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | [625] Hz | TBD |
|  |  | AWGN | [2334 Hz] | TBD |
|  | 4 | AWGN | [0] Hz | TBD |
|  |  | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | [625] Hz | TBD |
|  |  | AWGN | [2334 Hz] | TBD |
|  | 8 | AWGN | [0] Hz | TBD |
|  |  | TDLC300-100 Low | 400 Hz | TBD |
|  |  | AWGN | [625] Hz | TBD |
|  |  | AWGN | [2334 Hz] | TBD |

--------------End of text proposal-------------