3GPP TSG-RAN WG4 Meeting #93Bis	R4-1914119

Reno, USA, 18th – 22th Nov., 2019

Agenda item:		9.6.3.3

Source:	MediaTek Inc.

Title:	Discussion on NR fast Scell activation

Document for:		Discussion

# 1	Introduction

In RAN1 #98Bis meeting [1], companies agreed to introduce two BWPs for a SCell and utilize an explicit information field in DCI to indicate the transitions between ‘dormancy’ and ‘non-dormancy’ behavior:

Agreements: - For the L1 based Scell dormancy indication sent on primary cell within active time - UE is configured with at least two BWPs for an Scell - The explicit information field in DCI indicates switching to/from dormant BWP configured for the Scell - FFS definition of dormant BWP - FFS whether or not to the same BWP switching delay to the non-dormant to dormant transition delay - Note: Rel15 behavior for case when 1BWP is configured for the Scell (i.e., no dormancy indication for that Scell)

Besides, in last meeting RAN4 sent an LS [2] to inform RAN1 that RAN4 are evaluating if the existing latencies defined for DCI-based BWP switching are adequate for transition between ‘dormancy’ and ‘non-dormancy’ behavior:

RAN4 only discussed change in PDCCH monitoring and CSI-RS reporting configuration when changing from dormancy behaviour to non-dormancy behaviour. - RAN4 is still discussing if the existing latencies defined for DCI-based BWP switching are adequate for transition between ‘dormancy’ and ‘non-dormancy’ behaviour. - The interruption time has not been discussed.

In this paper, we provide our evaluation results and discuss the UE capability of BWP framework related features.

# 2	Discussion

In our previous paper [3], we already provided the evaluation results that type 1 BWP switching delay is achievable when only MIMO layer is adapted and there is no RF re-tuning involved. The same conclusion can also be applied in transition from “dormancy” and “non-dormancy” behavior, it is possible for UE to conduct the fast switching and reduce the corresponding schedule latency.

Observation 1: For UE staying at light sleep mode and transiting from “dormancy” to “non-dormancy” behavior, current BWP switch delay for Type 1 UE in 38.133 is achievable: For DCI-triggered switch, when UE receives the request at slot n, UE shall be ready to receive PDSCH with new settings at slot n+ TDormancy

|  | NR Slot length (ms) | MIMO switch delay TDormancy (slots) |
| --- | --- | --- |
|  |  | Type 1Note 1 |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 6 |
| Note 1: Depends on UE capability. Note 2: Rel-15 per-CC maximum number of MIMO layers is not changed by the dynamic adaptation |  |  |

Observation 2: For UE staying at light sleep mode and transiting from “dormancy” to “non-dormancy” behavior, 1 slot interruption time is sufficient for all SCSs

|  | NR Slot length (ms) | Interruption length X (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 1 |
| 3 | 0.125 | 1 |

However, as shown in Figure 1, we found that it is also possible that UE is configured with very long CSI-RS periodicity such that UE can enter the so-called “Deep sleep mode” to save more power. We see the need to discuss if RAN4 should specify different requirements for different scenarios.

SMTC periodicity may also be a determining factor. However, considering that UE is not required to conduct measurement in each SMTC window duration on the SCell, so we firstly focus on the impact of CSI-RS periodicity.

Observation 3: Whether UE should go to “light sleep mode” or “deep sleep mode” at dormancy behavior are actually depending on the configured CSI-RS periodicity.

Proposal 1: RAN4 to discuss how to handle the delay and interruption requirements for transitions between ‘dormancy’ and ‘non-dormancy’ behaviors in different scenarios

Figure 1: Options for power saving mode.

Currently, there are 3 features agreed in Rel-15/16 applying the BWP framework to conduct the state transition, i.e., BWP switching, maximal MIMO layer adaptation and dormancy behavior. A UE who can achieve Type 1 requirement for maximal MIMO layer adaptation may also achieve Type 1 requirement for the transition from “dormancy” to “non-dormancy” behavior in light sleep mode. Furthermore, even if all above parameters are changed at the same time, UE may still be able to conduct BWP switching with Type 1 requirements. In our understanding, all above 3 features are not only share the same BWP framework but also somehow related to UE power saving. Therefore, we see the benefit to also discuss them as a package. So we suggest

Proposal 2: The delay and interruption requirements as well as the corresponding UE capabilities of BWP framework related features, i.e., BWP switching, MIMO layer adaptation, and dormancy behavior, should be discussed as a package.

# 3	Summary

In this contribution, we have the following observations:

Observation 1: For UE staying at light sleep mode and transiting from “dormancy” to “non-dormancy” behavior, current BWP switch delay for Type 1 UE in 38.133 is achievable: For DCI-triggered switch, when UE receives the request at slot n, UE shall be ready to receive PDSCH with new settings at slot n+ TDormancy

|  | NR Slot length (ms) | MIMO switch delay TDormancy (slots) |
| --- | --- | --- |
|  |  | Type 1Note 1 |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 6 |
| Note 1: Depends on UE capability. Note 2: Rel-15 per-CC maximum number of MIMO layers is not changed by the dynamic adaptation |  |  |

Observation 2: For UE staying at light sleep mode and transiting from “dormancy” to “non-dormancy” behavior, 1 slot interruption time is sufficient for all SCSs

|  | NR Slot length (ms) | Interruption length X (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 1 |
| 3 | 0.125 | 1 |

Observation 3: Whether UE should go to “light sleep mode” or “deep sleep mode” at dormancy behavior are actually depending on the configured CSI-RS periodicity.

And we propose

Proposal 1: RAN4 to discuss how to handle the delay and interruption requirements for transitions between ‘dormancy’ and ‘non-dormancy’ behaviors in different scenarios

Proposal 2: The delay and interruption requirements as well as the corresponding UE capabilities of BWP framework related features, i.e., BWP switching, MIMO layer adaptation, and dormancy behavior, should be discussed as a package.

# Reference

- RAN1 #98Bis Minutes report, Oct., 2019.

- R4-1912830, Reply LS on NR fast SCell activation, Oct., 2019.

- R4-1910915, Discussion on UE dynamic adaptation to the maximum number of MIMO layers, Oct., 2019.