3GPP TSG-RAN WG4> Meeting # 93	R4-1914183

# Reno, Nevada, USA  18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0174 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.133: capturing agreements for NR-U inter-frequency measurements (9.3A) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 08-11-2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The CR updates the NR-U inter-frequency measurements section based on agreements made at previous meetings. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduces a new clause 9.3A, including the agreements for inter-frequency measurements in carrier frequencies with CCA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 9.3.8.3	SFTD Measurement reporting delay

The SFTD measurement reporting delay is defined as the time between a command that will trigger an SFTD measurement report and the point when the UE starts to transmit the measurement report over the air interface, excluding the RRC procedure delay defined in TS 38.331 [2]. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty of 2 x TTIDCCH resulting when inserting the measurement report to the TTI of the uplink DCCH. This measurement reporting delay excludes any delay caused by lack of UL resources for UE to send the measurement report.

The SFTD measurement reporting delay shall be less than Tmeasure_SFTD1 defined in clause 9.3.8.2.

## 9.3A	NR inter-frequency measurements in carrier frequencies with CCA

### 9.3A.1	Introduction

A measurement is defined as an SSB based CCA inter-frequency measurement provided it is not defined as an intra-frequency measurement according to clause 9.2 and CCA is used in the carrier frequency being measured.

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP, SS-RSRQ, and SS-SINR measurements of identified inter-frequency cells if carrier frequency information is provided by PCell or PSCell, even if no explicit neighbour list with physical layer cell identities is provided.

SSB based measurements are configured along with a measurement timing configuration (SMTC) per carrier, which provides periodicity, duration and offset information on a window of up to 5ms where the measurements on the configured inter-frequency carrier are to be performed. For inter-frequency connected mode measurements, one measurement window periodicity may be configured per inter-frequency measurement object.

When measurement gaps are needed, the UE is not expected to detect SSB on an inter-frequency measurement object which start earlier than the gap starting time + switching time, nor detect SSB which end later than the gap end – switching time.

### 9.3A.2	Requirements applicability

The requirements in clause 9.3A apply, provided:

-	The cell being identified or measured is detectable.

An inter-frequency CCA cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in clauses TBD for FR1,

-	SS-RSRQ related side conditions given in clauses TBD for FR1,

-	SS-SINR related side conditions given in clauses TBD for FR1,

-	SSB_RP and SSB Ês/Iot according to TBD.

### 9.3A.3	Number of cells and number of SSB

#### 9.3A.3.1	Requirements for FR1

For each inter-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	4 identified cells, and

-	7 SSBs with different SSB index and/or PCI on the inter-frequency layer.

### 9.3A.4	Inter frequency cell identification

When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_cca_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise UE shall be able to identify a new detectable inter frequency cell, in carrier frequencies with CCA, within Tidentify_inter_cca_with_index. The UE shall be able to identify a new detectable inter frequency SS block, in carrier frequencies with CCA, of an already detected cell within Tidentify_inter_cca_without_index.

Tidentify_inter_cca_without_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca) ms

Tidentify_inter_cca_with_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca + TSSB_time_index_inter_cca) ms

Where:

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

TSSB_time_index_inter_cca: it is the time period used to acquire the index of the SSB being measured given in table 9.3A.4-3.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

CSSFinter : it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps.

Table 9.3A.4-1: Time period for PSS/SSS detection, (Frequency range FR1)

| Condition NOTE1,2,3,4 | TPSS/SSS_sync_inter_cca |
| --- | --- |
| No DRX | max(600ms, (8+LPSS/SSS,gaps) x max(MGRP, SMTC period)) x CSSFinter |
| DRX cycle ≤ 320ms | max(600ms, ceil((8+LPSS/SSS,gaps)x1.5) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle > 320ms | (8+LPSS/SSS,gaps) x DRX cycle x CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: LPSS/SSS,gaps is the number of SMTC periods not available at the UE during TPSS/SSS_sync_inter_cca, where LPSS/SSS,gaps ≤ LPSS/SSS,gaps,max. NOTE 4: LPSS/SSS,gaps,max =TBD |  |

UE behaviour upon exceeding LPSS/SSS,gaps,max is FFS.

Table 9.3A.4-3: Time period for time index detection (Frequency range FR1)

| Condition NOTE1,2,3,4 | TSSB_time_index_inter_cca |
| --- | --- |
| No DRX | max(120ms, (3+ Lind,gaps) x max(MGRP, SMTC period)) x CSSFinter |
| DRX cycle ≤ 320ms | max(120ms, ceil((3+ Lind,gaps) x 1.5) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle > 320ms | (3 + Lind,gaps) x DRX cycle x CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: Lind,gaps is the number of SMTC periods not available at the UE during TSSB_time_index_inter_cca, where Lind,gaps ≤ Lind,gaps,max NOTE 4: Lind,gaps,max =TBD |  |

The UE shall restart the time index detection upon exceeding Lind,gaps,max.

#### 9.3A.4.1	Void

#### 9.3A.4.2	Void

### 9.3A.5	Inter frequency measurements

When measurement gaps are provided for inter frequency measurements in carrier frequencies with CCA, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in sub-clauses TBD, respectively, as shown in table 9.3A.5-1:

Table 9.3A.5-1: Measurement period for inter-frequency measurements with gaps (Frequency FR1)

| Condition NOTE1,2,3,4 | T SSB_measurement_period_inter_cca |
| --- | --- |
| No DRX | max(200ms, (8+ Lmeas) x max(MGRP, SMTC period)) x CSSFinter |
| DRX cycle ≤ 320ms | max(200ms, ceil((8+ Lmeas) x 1.5) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle > 320ms | (8+ Lmeas) x DRX cycle x CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: Lmeas is the number of SMTC periods not available at the UE during T SSB_measurement_period_NR_cca, where Lmeas ≤ Lmeas,max NOTE 4: Lmeas,max =TBD |  |

The UE shall restart the measurement upon exceeding Lmeas,max.

#### 9.3A.5.1	Void

#### 9.3A.5.2	Void

#### 9.3A.5.3	Void

### 9.3A.6	NR Inter frequency measurements reporting requirements

#### 9.3A.6.1	Periodic Reporting

Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in periodically triggered measurement reports shall meet the requirements in clauses TBD.

#### 9.3A.6.2	Event-triggered Periodic Reporting

Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in event triggered periodic measurement reports shall meet the requirements in clauses TBD.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause TBD.

#### 9.3A.6.3	Event-triggered Reporting

#### 9.3.6.3	Event-triggered Reporting

Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses TBD.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be within Tidentify_inter_cca_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index. Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_cca _with_index. Both Tidentify_inter_cca _without_index and Tidentify_inter_cca _with_index are defined in clause 9.3A.4. When L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period Tidentify_inter_cca _without_index or Tidentify_inter_cca _with_index defined in clause 9.3A.4. If a cell which has been detectable at least for the time period Tidentify_inter_cca _without_index or Tidentify_inter_cca _with_index defined in clause 9.3A.4 becomes undetectable for a period and then the cell becomes detectable again with the same spatial reception parameter and then triggers the measurement report as per TS 38.331 [2], the event triggered measurement reporting delay shall be less than TSSB_measurement_period_inter_cca defined in clause 9.3A.5 provided the timing to that cell has not changed more than  3200 Tc while measurement gap has not been available and the L3 filtering has not been used. When L3 filtering is used an additional delay can be expected.

### 9.3A.7	Void

<End of Change 1>