3GPP TSG-RAN WG4 Meeting #93				  												R4-1914334

Reno, USA, 18th Nov. 2019 – 22nd Oct. 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	NR-U – Scheduling of Type-1 intra-carrier guardbands

Agenda item:	9.1.1.2 Wideband operations (UE and BS) [NR_unlic-Core]

Document for:	Approval

# Introduction

Guardbands for NR-U and their application for wideband operation have been discussed since the start of the work item [1] in multiple contributions [2-12]. Two WF have been compiled [7] and [12] and some of the agreed main points herein are captured below.

- Define two different categories of guardbands for NR-U:

  - Carrier guardbands (inter-carrier guardbands)

  - In-carrier guardbands (intra-carrier guards)

  - Type-1 Intra-carrier guardband.

  - Type-2 Intra-carrier guardband.

- Intra-carrier guards for a carrier configured by a serving cell are defined as an integer number of full PRBs.

  - These are defined on a common RB grid which reference point is configured by the gNB as specified in section 5.3.4 of 38.101-1.

  - No intra-carrier guards are defined for 20 MHz carriers.

- RAN4 is tasked to specify necessary sizes of the inter-carrier guardbands and number of PRBs needed for intra-carrier guards to meet spectral emission mask defined for NR-U.

- The intra-carrier guard band could be scheduled if it is located within the adjacent contiguous LBT successful sub-bands where all scheduled LBT sub-bands have passed. Introduction of flexible guard-bands at sub-block edge is left FFS. Meaning scheduling of either additional or a reduced number of sub-block edge guard PRBs.

  - From the coexistence perspective, there is no need for Type-1 intra-carrier GBs.

  - Scheduling in Type-2 intra-carrier GBs is not feasible.

In this contribution, we elaborate on how we see the possibility of scheduling Type-1 intra-carrier guardbands.

# Discussion

Scheduling of intra-carrier guard PRBs

Based on WF [12], the intra-band GBs is separated into two types:

- Type-1 intra-carrier GBs: an intra-carrier guard-bands between two transmitted sub-bands.

- Type-2 intra-carrier GBs: an intra-carrier guard-bands at sub-block edge

The agreement that Type-2 intra-carrier GBs cannot be scheduled has impact on the scheduling, because partial PRGs (precoding granularity) and RBGs (scheduling granularity of TYPE-0 frequency domain RA) start to occur in DL carrier due to failed sub-band in the middle and TYPE-2 intra-carrier GBs. Therefore, we think that RAN1 should be informed about the previous agreement.

Proposal 1: 	Inform RAN1 that scheduling of Type-2 intra-carrier guardbands is not feasible from a RAN4 point of view.

The agreement that from a coexistence perspective, there is no need for Type-1 intra-carrier GBs means that there is potential for scheduling these and thereby improve the spectral utilization of wideband carrier when applying wideband operation.

It is not possible to know the LBT outcome a priori of the COT why at least in the first TTI of the gNB acquired COT guard-bands must be assumed as illustrated in Figure 1. However, from the second TTI the UE could be scheduled, by the gNB, in the Type-1 intra-carrier GBs, at least for the UEs with capability.

Figure 1 Two slots of the COT where guard-bands are used for all LBT sub-bands in first slot but in the following removed for the contiguous set of LBT sub-bands [13].

Following presented example, it is therefore proposed that.

Proposal 2: 	For Rel-16 consider the possibility of scheduling Type-1 intra-carrier guardbands at least for DL for UEs with capability.

If proposal 2 are agreed, then this should be included in the information send to RAN1 for consideration in their further design.

Proposal 3: 	Send LS RAN1 about feasibility of scheduling of Type-1 intra-carrier guardbands.

# Conclusion

In this contribution, we discussed the possibility of scheduling Type-1 intra-carrier guardbands.

Proposal 1: 	Inform RAN1 that scheduling of Type-2 intra-carrier guardbands is not feasible from a RAN4 point of view.

Proposal 2: 		For Rel-16 consider the possibility of scheduling Type-1 intra-carrier guardbands at least for DL

Proposal 3: 	Send LS RAN1 about feasibility of scheduling of Type-1 intra-carrier guardbands.

# References

- RP-182878	New WID on NR-based Access to Unlicensed Spectrum, Qualcomm

- R4-1904618	Guard-bands for NR-U wideband operation, Qualcomm

- R4-1904633	Discussion on channel bandwidths for NR-U, Nokia

- R4-1904637	Discussion on wideband carrier operation for NR-U, Nokia, Nokia Shanghai Bell

- R4-1905847	Discussion on NR-U wide carrier operation, OPPO

- R4-1906420	Discussion on filter adaptation if PRBs are scheduled in guard-bands for NR-U LBT sub-bands, Nokia

- R4-1910386	WF on Guardbands for NR-U, Nokia

- R4-1910852 	Guard band on wideband operation, Huawei, HiSilicon

- R4-1911611	NR-U - Scheduling of guard PRBs	Nokia, Nokia Shanghai Bell

- R4-1912382	On channel raster, SSB raster and wide-band operation for NR-U, LG Electronics Finland

- R4-1911547	Channel raster design and intra-carrier GB for wideband (WB2) operation	, Ericsson

- R4-1913060 	WF on Guardbands for NR-U, Nokia

- R4-1900196	On RAN1 Reply LS on wideband carrier operation for NR-U, AT&T