3GPP TSG-RAN WG4 Meeting#93                	R4-1914462

Reno, US, November 18th – 22nd 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for CSI-RS based measurements

Agenda item:	9.16.1.6

Document for:	Discussion

Introduction

In this paper, we provide our simulation results for CSI-RS based measurements.

Simulation assumptions

In this contribution, a single link noise-only scenario is considered with no other interference sources. The general parameters of the scenario are listed in Table 1.

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency | 4 GHz |
| BS transmit antennas | 1 tx |
| UE receive antennas | 2 rx (uncorrelated with equal gain, no rx beamforming) |
| Measurement period (in number of samples) | 1, 3, 5, 10 |
| Data Modulation | QPSK |
| Slot length | 14 symbols |
| CP length | Normal |
| SNR | -6 dB |
| Propagation channel | AWGN |

Table 3: CSI-RS configuration parameters

| Parameter | 4 GHz |
| --- | --- |
| CSI-RS bandwidth | 24, 48, 96, 192 PRB |
| CSI-RS SCS | 15 kHz |
| Periodicity | 40 ms |
| CSI-RS configuration: <X,D,N,(Y,Z),CDM>, where: X=number of CSI-RS ports, D=density [RE/RB/port], N=number of OFDM symbols in the same slot, Y=adjacent in frequency REs in PRB, Z=adjacent in time REs in PRB | <1,1,1,N/A,noCDM> <1,3,1,N/A,noCDM> |

Simulation results

Simulation results are provided for CSI-RSRP. The 5th, 50th and 95th percentiles of the CDF curves of the delta measurement are presented below.

CSI-RSRP

AWGN - 24 PRBs, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -6 | -1,51 | 2,03 | 4,61 | 4,61 |
| 3 | -6 | -1,44 | 1,25 | 3,16 | 3,16 |
| 5 | -6 | -1,01 | 1,04 | 2,65 | 2,65 |
| 10 | -6 | -0,65 | 0,75 | 2,01 | 2,01 |

AWGN - 24 PRBs, D=3

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -6 | -1,6 | 1,11 | 3 | 3 |
| 3 | -6 | -0,91 | 0,62 | 1,88 | 1,88 |
| 5 | -6 | -0,79 | 0,47 | 1,53 | 1,53 |
| 10 | -6 | -0,59 | 0,3 | 1,11 | 1,11 |

AWGN - 48 PRBs, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -6 | -1,8 | 1,26 | 3,53 | 3,53 |
| 3 | -6 | -1,26 | 0,71 | 2,27 | 2,27 |
| 5 | -6 | -1,04 | 0,6 | 1,74 | 1,74 |
| 10 | -6 | -0,8 | 0,41 | 1,22 | 1,22 |

AWGN - 48 PRBs, D=3

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -6 | -1,25 | 0,79 | 2,28 | 2,28 |
| 3 | -6 | -0,69 | 0,45 | 1,43 | 1,43 |
| 5 | -6 | -0,52 | 0,34 | 1,15 | 1,15 |
| 10 | -6 | -0,39 | 0,23 | 0,85 | 0,85 |

AWGN - 96 PRBs, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -6 | -1,23 | 1,04 | 2,63 | 2,63 |
| 3 | -6 | -0,73 | 0,62 | 1,68 | 1,68 |
| 5 | -6 | -0,54 | 0,49 | 1,38 | 1,38 |
| 10 | -6 | -0,29 | 0,36 | 0,98 | 0,98 |

AWGN - 96 PRBs, D=3

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -6 | -0,75 | 0,57 | 1,65 | 1,65 |
| 3 | -6 | -0,45 | 0,32 | 1,02 | 1,02 |
| 5 | -6 | -0,38 | 0,23 | 0,83 | 0,83 |
| 10 | -6 | -0,23 | 0,15 | 0,59 | 0,59 |

AWGN - 192 PRBs, D=1

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -6 | -0,94 | 0,63 | 1,97 | 1,97 |
| 3 | -6 | -0,63 | 0,32 | 1,25 | 1,25 |
| 5 | -6 | -0,45 | 0,26 | 0,97 | 0,97 |
| 10 | -6 | -0,32 | 0,16 | 0,7 | 0,7 |

AWGN - 192 PRBs, D=3

| Number of samples | SNR (dB) | 5 % | 50 % | 95 % | Absolute accuracy |
| --- | --- | --- | --- | --- | --- |
| 1 | -6 | -0,61 | 0,38 | 1,25 | 1,25 |
| 3 | -6 | -0,32 | 0,24 | 0,77 | 0,77 |
| 5 | -6 | -0,25 | 0,19 | 0,6 | 0,6 |
| 10 | -6 | -0,19 | 0,13 | 0,41 | 0,41 |

Conclusion

In this contribution, CSI-RSRP simulation results are provided for scenarios in line with the simulation assumptions presented in Section 2.

- References