3GPP TSG-RAN4 Meeting #93	R4-1914527

# Reno, US, 18th - 22th November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | 0060 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.141-2: Additional information about alignment needed for TRP measurements in Annex I |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-11-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The reason for alignment in TRP assessment is not described, and the necessary alignment between test object and measurement antenna is different for different methods. When making TRP measurements appropriate alignment is crucial. Firstly, the measurement antenna needs to be aligned with the measurement surface, here a sphere, in order to correctly measure the entire TRP. Secondly, in methods where an over-estimate of the TRP is the end result, careful angular alignment is needed in order to measure peak values in the main beams and the side lobe regions. The remaing TRP methods are designed to be independent of rotations of the angular grid, and hence angular alignment is not needed. This is a CR based on endorsed draft CR (R4-1912954) from last meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Additional information is added in Annex I.1 - Correction of structure for Annex I.8 - Changed test object to EUT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the actual measurement error may become larger than the agreed MU from the measurement error evaluation for the TRP measurement method. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex I |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.145-2 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Annex I (normative):
TRP measurement procedures

# I.1	General

The annex describes various procedures for BS OTA TRP measurments. These procedures can provide either an accurate or an over-estimate of TRP values. The procedures for an accurate estimate can be applied to all TRP requirements. However, if a TRP requirement does not need accurate TRP estimate then the procedures for over-estimate of TRP may be used in order to have a reasonable OTA test time. Pre-scan does not provide an accurate TRP estimate or over-estimate of TRP. Pre-scan is a fast but coarse method that is used to identify the spurious emission frequencies with emission power as described in annex I.13. A sequential measurement is then made at the emission frequencies, to assess the TRP as described in annex I.2 to annex I.9.

When making TRP measurements the alignment between EUT and measurement antenna is important to achive expected measurement uncertainty;

- The measurement antenna needs to be aligned tangential to the measurement surface forming a sphere around the EUT, in order to correctly measure the TRP properly.

- Test methods described in subclauses I.5.1, I.5.2, I.10, I.11 and I.12 require angular alignment between the selected measurement grid and EUT radiation pattern in order to measure peak values in the main beams. Angular misalignment can lead to differences in the actual and measured angular positions of the intended maximum EIRP.

- Test methods described in subclause I.5.3, I.6 and I.9 are designed to be independent of rotations of the angular grid, and hence angular alignment between the measurement grid and EUT is not needed.

# I.2	Spherical equal angle grid

## I.2.1	General

TRPEstimate is defined as

TRPEstimate=π2NMn=0N-1m=0M-1EIRP(θn,ϕm)sinθn

when EIRP measurements is used or as

TRPEstimate=4πd2π2NMn=1N-1m=0M-1PD(θn,ϕm)sinθn

when power density measurements are used, and d is the test distance. N and M are the number of samples in the θ and ϕ angles. Each (θn,ϕm) is a sampling point. The sampling angular intervals for θ and ϕ angles are Δθ=πN and Δϕ=2πM. The sampling intervals Δθ and Δϕ are described in I.2.2.

## I.2.2	Reference angular step criteria

The reference angular steps Δϕref and Δθref in degrees are defined as:

Δϕref= min⁡(180∘πλDcyl,15∘)

Δθref=⁡min(180∘πλD,15∘)

The upper limit for these reference angular steps of 15∘ ensures a low Summation Error (SE) when λ is large compared to the DUT dimensions.

Dcyl and D are calculated as:

`	Dcyl=d2+w2

D=d2+w2+h2

The definition of d, w and h is shown in figure I2.2-1. The radiation source can be EUT antenna array or the whole of EUT.

Figure I2.2-1: Dimensions of a radiation source: depth (d), width (w) and height (h)

Optionally, in the case of Uniform Linear Array (ULA), when d is negligible (d ≈0) and the EUT is mounted along the yz plane as shown in figure I2.2-2, the reference angular steps, in degrees, can be determined by

∆θref=180°πarcsin⁡(λ/Dz)

Δϕref=180°πarcsin⁡(λDy)

Where Dy is the length of radiating parts of EUT along y-axis, Dz is the length of radiating parts of EUT along the z-axis and  is wavelength for the measured frequency.

Figure I2.2-2: Spherical coordinate for OTA conformance testing of EUT

Where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the SF (sparsity factor) of the grid is defined as

SF=maxΔθgridΔθref,ΔϕgridΔϕref

Where Δθgrid and Δϕgrid are the actual angular steps used in the measurement.

Alternatively, when EUT radiating dimensions are not known, for each frequency within the downlink operating band including ΔfOBUE, the reference angular steps can be specified in terms of the beamwidth of the wanted signal as

∆ϕref=λλo BeWϕ

Δθref=λλo BeWθ

where λo is the wavelength of the wanted signal, and BeWφ and BeWθ are the beamwidth of the wanted signal in the φ-axis and θ-axis, respectively.

BeWφ and BeWθ may be set to beamwidth declared for the OTA BS radiated transmit power requirement provided the same beam is applied to test in-band TRP requirements.

NOTE:	Beamwidth is approximately equal to half the first-null beam width.

# I.3	Spherical equal area grid

TRPEstimate is defined as

TRPEstimate=1Nn=1NEIRP(θn,ϕn)

N is the total number of samples and specified as

The sampling intervals Δθref and Δϕref are described in annex I.2.2. Each (θn,ϕn) is a sampling point.

# I.4	Spherical Fibonacci grid

TRPEstimate is defined as

TRPEstimate=1Nn=0N-1EIRP(θn,ϕn)

N is the total number of samples and specified as

The sampling intervals Δθref and Δϕref are described in annex I.2.2. Each (θn,ϕn) is a sampling point, where θn and ϕn, in degrees, are defined as:

θn=cos-11-2n+1N

ϕn=360°nΨ,  Ψ=1+52

# I.5	Orthogonal cut grid

## I.5.1	General

Here, at least two cuts (default) shall be used, an optional third cut can be used. The alignment of the cuts must be along the symmetry planes of the antenna array. No alignment is required for spurious emissions.

When alignment is required:

1)	The first mandatory cut is a horizontal cut passing through the peak direction of the main beam.

2)	The second mandatory is a vertical cut passing through the peak direction of the main beam. Using the data from these two mandatory cuts, a conditional pattern multiplication can be used.

3)	The third optional cut is a vertical cut orthogonal to the first and the second cut.

When alignment is not required, the cuts can be aligned arbitrarily.

Once the number and the orientation of the cuts are decided, the total EIRP is measured on the orthogonal cuts and the TRP is then calculated as follows: First the contributions from each cut is calculated as

EIRPav,cut-n=1Pj=1PEIRP(j)

where P is the number of sampling points in the cut. The final contribution for all cuts is calculated as

TRPEstimate=1Nn=1NEIRPav,cut-n

where N is the number of cuts. Note that when orthogonal cuts are measured, the intersection points are measured multiple times and the repeated values can be removed from the samples before averaging.

When two cuts measurements are used, a conditional pattern multiplication can be applied. The following are the conditions for applying pattern multiplication:

i.	The vertical cut (and the main beam) is in the xz -plane

ii.	The frequency of the emission is within the downlink operating band.

iii.	The bandwidth of the emission is the same as the bandwidth of the in-band modulated signal

iv.	The emission appears/disappears when the Tx power is turned on/off.

v.	The antenna arrays of the EUT

1)	Have rectangular grids of antenna element positions

2)	Have symmetry planes that are vertical and horizontal.

3)	Have parallel antenna planes

The antenna array is here assumed to be placed in the yz-plane. The pattern multiplication is performed in uv-coordinates and the data in the two cuts are denoted EIRPcut1ϕ at θ=θH and a vertical cut with data EIRPcut2θ at ϕ=0. The data is split in two parts corresponding to the forward and backward hemispheres. The uv-coordinates are the projections of the angular directions onto the antenna plane, here the yz-plane. Using the spherical coordinates as depicted in figure I.2.2-1 the u and v coordinates are defined as:

u=sinθsinϕv=cosθ

Note that only the data on the cuts are measured.

Calculate power density/EIRP values outside the two cardinal cuts as

EIRPu,v=EIRPcut1uEIRPcut2vEIRP0,vH

The pattern multiplication is applied separately for the forward (fwd) and backward (bwd) hemisphere. The TRP is then calculated as

TRP=14πfwdEIRPfwdu,vdudv1-u2-v2+bwdEIRPbwdu,vdudv1-u2-v2

NOTE:	The numerical singularity at u2+v2=1 must be treated with care, e.g. by change of variables.

## I.5.2	Operating band unwanted emissions

The procedure is as follows:

1)	Follow steps described in annex I.5.1 for the first two mandatory cuts and calculate the TRPEstimate.

2)	Compare the TRPEstimate to the limit.

3)	If the TRPEstimate is above the limit, perform the measurement on an additional third cut and repeat steps 1 to 2.

## I.5.3	Spurious unwanted emissions

The procedure is as follows:

1)	Follow steps described in annex I.5.1 for two cuts and calculate the preliminary TRPEstimate.

2)	Add the appropriate correction factor ΔTRP according to table I.5.3-1 to ensure overestimation with 95% confidence.

3)	Compare the corrected TRPEstimate (including ΔTRP) to the limit.

4)	If the corrected TRPEstimate is above the limit, perform the measurement on an additional third cut and repeat steps 1 to 3.

Table I.5.3-1: The correction factor for two or three cuts dense sampling

|  | Three cuts | Two cuts |
| --- | --- | --- |
| Correction factor ΔTRP (dB) | 2.0 | 2.5 |

# I.6	Wave vector space grid

If EUT is mounted along the yz plane as shown in figure I2.2-1, the reference step in wave vector space can be determined by

Δuref=λDy

Δvref=λDz

where Dy is the length of radiating parts of EUT along y-axis, Dz is the length of radiating parts of EUT along the z-axis.

According to the relationship between the normalized wave vector and spherical coordinate, the wave vector can be represented as following:

u=sinθsinϕ, v=cos⁡(θ)

The total radiated power (TRP) in the wave vector space is determined by:

TRPestimate=ΔurefΔvref4πu2+v2<1cosϕ>0EIRPθn,ϕm,nsinθncosϕm,n +u2+v2<1cosϕ<0EIRPθn,ϕm,nsinθncosϕm,n

For spurious Tx or Rx emissions and where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the SF (sparsity factor) of the grid is defined as

SF=max∆ugrid∆uref,∆vgrid∆vref

Where ∆ugridand ∆vgrid are the actual steps used in the wave vector space in the measurement and the upper bound of their value is 1/12, corresponding to 15 degree steps close to boresight.

# I.7	Void

# I.8	Void

# I.9	Full sphere with sparse sampling

The procedure is as follows:

1)	Set the angular grid:

a.	Non-harmonic frequencies: choose the angular steps Δϕ and Δθ smaller than or equal to 15 degrees. Calculate the sparsity factor (SF) as

SF=maxΔϕΔϕref,ΔθΔθref

and the correction factor as:

ΔTRP=SF-1SFmax-1⋅1.0 dB,

where SFmax corresponds to 15 degrees angular step. If the sparsity factor is smaller than 1, the correction factor ΔTRP is 0 dB.

Harmonic frequencies with fixed beam test signal: choose the angular steps smaller than or equal to the reference angular steps Δϕref and Δθref. Correction factor ΔTRP is 0 dB.

[Harmonic frequencies with beam sweeping test signal set the angular steps to [15] degrees. Correction factor is ΔTRP 0 dB].

2)	Apply a suitable numerical integration to calculate the preliminary TRPEstimate.

3)	Add the appropriate correction factor ΔTRP according to step 1 to ensure an overestimation with 95% confidence.

4)	Compare the corrected TRPEstimate (including ΔTRP) with the limit. If the corrected TRPEstimateis above the limit, choose a smaller angular step and repeat steps 2-4. If the sparsity factor is less than one, no significant improvement of accuracy is expected.

# I.10 Beam-based directions

Beam-based direction can be used in the base station operating band only if the directivity of the radiation pattern of the emssions being measured is known. TRPEstimate is defined as

TRPEstimate=EIRPpeakDEUT, where EIRPpeak is the maximum EIRP in the beam peak direction within a particular beam direction pair and DEUT is the directivity of the EUT.

# I.11	Peak method

The peak method can be used when frequencies with unwanted peak emissions are identified during pre-scan. The method does not provide an estimate of TRP.

For each peak emission frequency identified during pre-scan, measure peak EIRP or power density as follows:

1)	Move EUT and test antenna to the same position where the peak emission is recorded during the pre-scan.

2)	Move the EUT around the position and test antenna orientation to find the final peak EIRP or power density.

3)	The measured peak power density or EIRP shall be used to demonstrate conformance.

NOTE:	Peak EIRP is the linear sum of two orthogonal polarized components.

# I.12	Equal sector with peak average

Equal sector with peak average can be performed on frequencies with unwanted peak emission, which are considered by the peak method for further measurements.

The spherical angle ϕ is divided into K equal sectors. If the largest dimension of EUT is less than 60 cm, then each sector is a half quadrant of 45°.

For each peak emission frequency, measure peak EIRP of beams belonging to different sectors of the sphere as follows:

1)	Move EUT and test antenna to the same position where the emission peak is recorded during the pre-scan.

2)	Move EUT around the position and test antenna orientation to find the final peak EIRP.

3)	Repeat Steps 1 to 2 until all sectors are covered.

4)	Calculate TRPEstimate as

TRPEstimate=1Kk=1KEIRPk, where EIRPk is the peak EIRP in the kth sector.

NOTE:	Peak EIRP is the linear sum of two orthogonal polarized components.

# I.13 Pre-scan

Pre-scan is used to identify frequencies with unwanted emission power levels above a certain threshold. The pre-scan does not provide an estimate of TRP. An emission frequency identified by a pre-scan may be further investigated by any of the TRP measurement methods in this annex.

The procedure for pre-scan is as follows:

1)	Scan the entire surface around EUT.

2)	Rotate test antenna to cover all possible polarizations of emissions to detect maximum emissions.

3)	Record the list of frequencies and corresponding unwanted emission power levels, EUT spatial positions, and test antenna polarization for which the maximum emission levels occur.

4)	Emissions which 20 dB or more below the specified limit shall not require further measurements.