TSG-RAN Working Group 4 (Radio) meeting #93	R4-1914528

Reno, US, 18th – 22nd November 2019

Source:	Ericsson

Title:	TP to TR 38.820: Update of technical background for BS out-of-band blocking in subclause 7.4.2.5

Agenda item:	11.4.8.4

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#92bis in Chongqing) the technical background for out-of-band blocking was discussed extensively. The outcome from the discussion was captured in a text proposal to TR 38.820 [1]. The text proposal was technically endorsed but due to issue with change marks in MS word it was not implemented.

In this contribution a new version with correct change marks have been created. The text proposal is attached at the end of this contribution.

# Discussion

Summary of technically endorsed changes (including corrections without change marks in previous version);

- Addition of general sections in subclause 7.4.2.5.1.

- The structure is updated with correct numbering.

- Information about the wanted signal is added.

- The table with information about interferer signal characteristics is updated (Table 7.4.2.5.2-2)

- Figures in subclause7.4.2.5.2 is combined into one figure.

- In subclause 7.4.2.5.3 the figure is moved and updated.

- In subclause 7.4.2.5.3 a table with parameters to consider for 7 to 24 GHz is added.

# Conclusion

At the end of this contribution a text proposal is attached for approval. The contents were technically endorsed at the last meeting. In this version the change marks are following the last updated version of TR 38.820 [1].

# References

[1]	R4-1913025, “TP to TR 38.820: Update of technical background for BS out-of-band blocking in subclause 7.4.2.2”, Ericsson

[2]	R4-1913080, “Draft TR 38.820 V0.3.0”, Huawei

TEXT PROPOSAL:

#### 7.4.2.5	Out-of-band blocking requirements

##### 7.4.2.5.1	General

The out-of-band blocking requirement relies on the sensitivity as reference. The base station is illuminated with a wanted signal and an interferer signal at the same time maintaining specified link quality. Hence, the FRC used for conformance testing must support the carrier bandwidth to be supported for the frequency range 7 GHz to 24 GHz.

##### 7.4.2.5.2	Background

There are currently three different NR concepts for how the out-of-band blocking requirement is defined with respect to the wanted signal power used as reference for the requirement, see Table 7.4.2.5.2-1.

Table 7.4.2.5.2-1: Wanted signal characteristics

| BS type | Wanted signal conducted power (dBm) | Wanted signal EIS (dBm) |
| --- | --- | --- |
| 1-C, 1-H | PREFSENS + 6 dB | NA |
| 1-O | NA | EISminSENS + 6 dB |
| 2-O | NA | EISREFSENS_50M + 6 dB |

For BS type 1-C and BS type 1-H, reference sensitivity (PREFSENS) is used as reference for out-of-band receiver blocking. For BS type 1-O, the minimum sensitivity (EISminSENS) relating to the full array antenna sensitivity is used as reference while for BS type 2-O, the reference sensitivity (EISREFSENS) relating to the sub-element sensitivity is used as reference.

The conducted requirement defined for BS type 1-C and BS type 1-H is only defined for FR1. It is based on the analysis done for UTRA BS and the derivation of the -15 dBm conducted interferer level is based on interference analysis showing that worst case interference is from other BS at approximately the same frequency.

For simplicity, it is treated as the worst case and applied to the whole test frequency range up to 12.75 GHz which is shown below as figure 7.4.2.5.2-2.

The OTA requirement is defined for both FR1 and FR2 which are concluded below in table 7.4.2.5.2-2 and it can be found that a discontinuity point of the FR2 requirement occurs at 12.75 GHz.

Table 7.4.2.5.2-2: Interferer signal characteristics

| BS type | Frequency range of interferer signal (MHz) | Interferer level at TAB/antenna connector (dBm) | Interferer RMS field-strength at RIB (V/m) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1-C, 1-H | 30 to FUL,low – ΔfOOB | -15 | NA | CW |
| 1-C, 1-H | FUL,high + ΔfOOB to 12750 | -15 | NA | CW |
| 1-O | 30 to FUL,low – ΔfOOB | NA | 0.36 | CW |
| 1-O | FUL,high + ΔfOOB to 12750 | NA | 0.36 | CW |
| 2-O | 30 to 12750 | NA | 0.36 | CW |
| 2-O | 12750 to FUL,low – ΔfOOB | NA | 0.10 | CW |
| 2-O | FUL,high + ΔfOOB to flimit | NA | 0.10 | CW |
| Note: Where flimit is the 2nd harmonic of the upper frequency edge of the operating band. |  |  |  |  |

The interferer signal level 0.36 V/m from 30 MHz to 12750 MHz is based on the calculation of -15 dBm conducted interferer signal level at frequency 2000 MHz and 17 dBi antenna gain and a distance between aggressor and victim of 30 m.

The EIRP at the aggressor is then calculated as:

EIRP(30m) = Prx – Gant + FSPL(30m) = -15dBm – 17dBi + 68dB(frequency=2 GHz, FSPL 30m)  = 36 dBm

, where the aggressor EIRP level in linear scale is then converted to an RMS field strength level at the victim base station as:

E=30*1036-301030=0.36V/m

For FR2 requirements the blocking signal is calculated to 0.1 V/m and applied to the whole frequency range as shown below in figure 7.4.2.5.2-1. Note when FR2 requirements were derived no systems between 7 to 24 GHz were considered.

The aggressor out of band BS is considered to have a total power of 29 dBm, an antenna gain of 26 dBi hence an EIRP of 55 dBm. As the aggressor main beam is pointed at UE’s at the ground, an aggressor sidelobe suppression level of 13 dB to get the aggressor signal pointing towards the victim is considered.

With the interferer at a distance of 200 m this gives an interfering field strength of:

E=30*1055-13-3010200=0.1V/m

In figure 7.4.2.5.2-1, the interferer signal level is plotted for FR1 and FR2.

Figure 7.4.2.5.2-1: OOB blocking interferer level

##### 7.4.2.5.3	Technical considerations for 7 to 24 GHz

When developing out-of-band blocking requirements for base stations operating within the frequency range 7 to 24 GHz RAN4 needs to consider requirements defined for FR1 and FR2. In table 7.4.2.5.3-1, the wanted signal characteristics is captured.

Table 7.4.2.5.3-1: Wanted signal characteristics for base stations operating within 7 to 24 GHz

| BS type | Wanted signal power (dBm) | Wanted signal (dBm) |
| --- | --- | --- |
| -C, -H | PREFSENS + X dB | NA |
| -O | NA | EIS + Y dB |
| Note: Where X and Y will be defined in the WI phase. |  |  |

, where PREFSENS and EIS will be defined in the WI phase.

In figure 7.4.2.5.3-1, the interferer level for frequencies outside the range 7125 MHz to 24250 MHz is plotted. The interferer level between 7125 MHz to 24250 MHz will be defined in the WI phase.

Figure 7.4.2.5.3-1: Baseline for interferer signal level