3GPP TSG-RAN WG4 Meeting #93	R4-1914610

Reno, Nevada, USA, 18-22 November 2019

Agenda Item:	9.1.1.1

Source:	Ericsson

Title:	Number of bands and CCs to be considered in Rel-16 NR-U WI

Document for:	Approval

# 1	Introduction

A number of scenarios have been defined under Rel-16 NR-U. With the intention to defining band combinations for all scenarios within the timeframe of Rel-16 work item, we need to prioritize the band combinations as well.

In this contribution, we resubmit our original proposals in RAN4#92bis [2] to streamline the RAN4 work for Scenarios A, B, C in Rel-16 timeframe.

# 2	Discussion

Due to limited time, we propose to limit the scope of Scenario A and B to a maximum of two bands.

Proposal-1: Consider maximum two bands in NR-U combinations in Rel-16 for scenario A and scenario B.

However, for the purpose of completion of this WI on time, we can in general concentrate on 100MHz aggregated BW in NR-U. Then, RAN4 can focus on 1 CC (licensed) and 5 CC (unlicensed) combination for both scenario A and one PCell in LTE, one PSCell in NR-U and 4 Scells in scenario B. Similarly, RAN4 can focus on 1 Pcell and 4 Scells in scenario C. So, we propose the following:

Proposal-2: RAN4 to focus on the following in Rel-16 work:

| Scenario A | - PCell in NR + 5 DL SCell in NR-U - PCell in NR + 5 DL SCell in NR-U + 1 UL SCell in NR-U |
| --- | --- |
| Scenario B | PCell in LTE + PSCell in NR-U + 4 DL SCell in NR-U |
| Scenario C | PCell in NR-U + 4 DL Scell in NR-U |
| Note 1: PCell and PSCell have both DL and UL Note 2: All NR-U serving cells are contiguous |  |

Note that, any future band combination consisting of more than two bands will be release independent from Rel-16. Also, any band combination with more than 2 CCs will also be release independent from Rel-16

Proposal-3: Any future band combination consisting of more than two bands or more than 5 NR-U CCs can be introduced later but will be release independent from Rel-16.

# 3	Summary

In this contribution, we propose to prioritize some band combinations for Rel-16 NR-U work, so that the specification work can be completed considering the WI completion schedule. We propose that RAN4 agrees with the following:

Proposal-1: Consider maximum two bands in NR-U combinations in Rel-16 for scenario A and scenario B.

Proposal-2: RAN4 to focus on the following in Rel-16 work:

| Scenario A | - PCell in NR + 5 DL SCell in NR-U - PCell in NR + 5 DL SCell in NR-U + 1 UL SCell in NR-U |
| --- | --- |
| Scenario B | PCell in LTE + PSCell in NR-U + 4 DL SCell in NR-U |
| Scenario C | PCell in NR-U + 4 DL Scell in NR-U |
| Note 1: PCell and PSCell have both DL and UL Note 2: All NR-U serving cells are contiguous |  |

Proposal-3: Any future band combination consisting of more than two bands or more than 5 NR-U CCs can be introduced later but will be release independent from Rel-16.

# References

- RP-182878, NR-based Access to Unlicensed Spectrum, Qualcomm Incorporated.

- R4-1912176, Number of bands and CCs to be considered in Rel-16 NR-U WI, Ericsson