3GPP TSG-RAN WG4 #93	R4-1914638

Reno, USA, 14th – 18th November, 2019

Title: 					Requirements for DRX operation in NR high speed

Source: 				Ericsson

Agenda item:	9.17.1

Document for:	Discussion

# Introduction

In this contribution we develop an analysis framework based on free space path loss considerations, and the rate of change of RSRP at the cell edge for different sizes of cell (ISD). Based on this we examine legacy requirements for NR SpCell in non SRX. We examine the suitability of detection and measurement periods for:

- SpCell in RRC connected state at speeds up to 500km/h

# Discussion

## System level analysis framework

In this section we present a system model which we will use to evaluate the necessity of improvements to RRM requirements to support UE speeds up to 500 km/h. The approach is less thorough than system level simulation but more formal than the “moving time” analysis that has been previously presented in RAN4.

We start by defining UE speed vue  in metres per second, with the convention that positive velocity is velocity away from the basestation, and we define intersite distance as ISD. For an SFN deployment, ISD should be interpreted as the distance between the last radio node in one SFN area and the first radio node in the adjacent SFN area. In the analysis we assume constant speed UEs, so if a UE passes a basestation transmitter at time t=0, its subsequent distance from that basestation is simply

x(t)=vue ∙ t

For freespace propagation and penetration into a vehicle such as train, the pathloss may be modelled as

PLFS (d,f) = Lpenetration(f)+ 20 log10(d)+20*log10(f)+32.54 where d is in units of m, and f is frequency in GHz.

Lpenetration is the loss due to the UEs being inside the train carriage (frequency dependent).

The RSRP experienced by a UE at a distance d may thus be expressed as

RSRP(d,f)= PSSB – PLFS(d,f) = PSSB - Lpenetration(f)– 20 log10(d)-20*log10(f)-32.54

Where PSSB is the SSB transmission power in dBm/SSB SCS. Strictly for an SFN network, we should sum the RSRP contribution from all SFN nodes along with their respective distances d1…dN however for simplicity in the analysis we assume the RSRP to be dominated by a single SFN node at the edge of the SFN area, since all other SFN nodes are at a distance of at least 3xISD from the UE.

We can now substitute x(t) to derive RSRP as a function of time as the UE moves away from a cell at constant speed

RSRP(t,f) = PSSB - Lpenetration(f)– 20 log10(vue t)-20*log10(f)-32.54 																		(1)

Since the rate of change of RSRP is also of interest, we differentiate with respect to time, noting that most of the terms in (1) except the 20log10(…) term are constant w.r.t time

From this we derive RSRP’(t)=-20/ln(10) ∙(1/t)																							(2)

Specifically, it can be noted that the derivative (rate of change) of RSRP is independent of frequency, although naturally the pathloss is higher on higher frequency bands, and that it has a 1/t (or equivalently 1/d due to constant speed assumption) dependency on time (and distance) from the basestation.

Without hysteresis, L3 filtering or time to trigger configuration, the handover will ideally be triggered at time t= 0.5xISD / vue since the cell radius is ISDx0.5. Hence the ideal RSRP and rate of change of RSRP at the ideal handover point is

RSRP (at ideal handover) = PSSB- Lpenetration – 20 log10( 0.5xISD)  -20*log10(f)-32.54					(3)

RSRP’(at ideal handover)=-20/ln(10).( vue /0.5xISD)															(4)

## Analysis of necessary requirements

From equation (4), it can be seen that the rate of change of RSRP (eg in dB per second) depends on the UE velocity (eg in metres per second) relative to 0.5xISD. 0.5xISD is simply the radius of the source (and target) cells. On different frequency bands, the rate of change of RSRP is the same, based on the assumption of free space path loss.

The most important parameters for RRM requirements are cell detection time, Tidentify and measurement period Tmeasure. For successful cell identification to trigger the handover, side conditions are also relevant. If we assume a simple 2 cell model, and no other noise or interference, cell detection can be successfully performed when RSRPtarget>(RSRPsource-6) dB.  Around the handover area, the source cell will have RSRPsource reducing at rate -20/ln(10).( vue /0.5xISD) and the target cell has RSRPtarget increasing at rate 20/ln(10).( vue /0.5xISD). Allowing for a safety margin (due to the fact that RSRP is not reducing/increasing linearly, and there may be other noise and interference), it seems reasonable to assume that cell detection will be successful if the rate of change of RSRP (for one cell) is less than around 1.5dB during the Tidentify period which is 50% of the theoretical value. Hence, we assume for successful identification of target before handover should be possible if the following condition applies (looking from serving cell perspective, which is falling)

0≥Tidentfy∙RSRP’(at ideal handover) ≥ -1.5dB															(Cond 1)

For already identified cells, a similar approach can be taken. Since RSRP relative accuracy is ±3dB, we follow a similar approach and it seems reasonable to assume that measurement will give a reasonably accurate indication of cell usability and ranking if the rate of change of RSRP (for one cell) is less than around 0.75dB during the Tmeasure period.

0≥Tmeasur∙RSRP’(at ideal handover) ≥ -0.75dB															(Cond 2)

The next step is to substitute RSRP’(at ideal handover) from the previous section. This gives the following conditions

Identify∙-20/ln(10).( vue /0.5xISD)≤ 1.5dB																(Cond1a)

Tmeasure∙-20/ln(10).( vue /0.5xISD)≤ 0.75dB																(Cond2a)

Conditions 1a and 2a are readily evaluated to verify the suitability of requirements for different conditions (eg different UE speed, different ISD, different UE requirements due to DRX etc.)

## Requirements for DRX

We begin by looking at the legacy requirements for intrafrequency measurement. From TS38.133 section 9.2

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms

For simplicity we will assume that for intrafrequency HST, Tidentify_intra_without_index applies. One justification is that in an SFN network good synchronisation between the SFN nodes is necessary, and likely neighbour SFN areas may be synchronised as well.

TPSS/SSS_sync_intra = max( 600ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra (DRX≤320ms)

TPSS/SSS_sync_intra = ceil(5] x Kp) x DRX cycle x CSSFintra(DRX>320ms)

T SSB_measurement_period_intra= max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra

T SSB_measurement_period_intra= ceil(5] x Kp) x DRX cycle x CSSFintra(DRX>320ms)

For simple analysis we will also assume CSSFintra=Kp=1. This could represent mobility using PCell measurements with no gap pattern configured. For this configuration, the numerical Tidentify_without_index and Tmeasurement_period are as shown in table 1

| Tdrx | Tmeasure | Tidentify |
| --- | --- | --- |
| 40 | 320 | 920 |
| 80 | 640 | 1280 |
| 160 | 1280 | 2560 |
| 320 | 2560 | 5120 |
| 640 | 3200 | 6400 |
| 1280 | 6400 | 12800 |
| 2560 | 12800 | 25600 |

Table 1: UE minimum intrafrequency measurement requirements with no time index reading, CSSF=1 and Kp=1

Intuitively, it should be kept in mind that the UE velocity is around 138m/s, so it will move from the cell centre to the cell edge (500m) in 3.6s. Considering the values for Tmeasure and Tidentify in table 1, it looks challenging to operate with even 160ms DRX cycle (cell identification requirement corresponds to 16 DRX cycles).

In table 2 and 3, we evaluate the dB change in one cell during Tidentify and Tmeasure for different ISD and SMTC period. Entries highlighted in red are for cases where condition 1a/1b is not met.

|  | ISD | 500 | 1000 | 1500 | 2000 |
| --- | --- | --- | --- | --- | --- |
| Tdrx |  |  |  |  |  |
| 40 |  | 4.439455 | 2.219727 | 1.479818 | 1.109864 |
| 80 |  | 6.176633 | 3.088316 | 2.058878 | 1.544158 |
| 160 |  | 12.35327 | 6.176633 | 4.117755 | 3.088316 |
| 320 |  | 24.70653 | 12.35327 | 8.23551 | 6.176633 |
| 640 |  | 30.88316 | 15.44158 | 10.29439 | 7.720791 |
| 1280 |  | 61.76633 | 30.88316 | 20.58878 | 15.44158 |
| 2560 |  | 123.5327 | 61.76633 | 41.17755 | 30.88316 |

Table 2 : dB change in one cell during Tidentify at 500km/h

|  | ISD | 500 | 1000 | 1500 | 2000 |
| --- | --- | --- | --- | --- | --- |
| Tdrx |  |  |  |  |  |
| 40 |  | 1.544158 | 0.772079 | 0.514719 | 0.38604 |
| 80 |  | 3.088316 | 1.544158 | 1.029439 | 0.772079 |
| 160 |  | 6.176633 | 3.088316 | 2.058878 | 1.544158 |
| 320 |  | 12.35327 | 6.176633 | 4.117755 | 3.088316 |
| 640 |  | 15.44158 | 7.720791 | 5.147194 | 3.860395 |
| 1280 |  | 30.88316 | 15.44158 | 10.29439 | 7.720791 |
| 2560 |  | 61.76633 | 30.88316 | 20.58878 | 15.44158 |

Table 3 : dB change in one cell during Tmeasure at 500km/h

From the table 1 and table 2 we observe

Observation 1 : Legacy cell identification and measurement requirements with DRX are insufficient in most cases at 500km/h except for large ISD and relatively short DRX cycle

UE enhancements

Given observation 1, it seems that UE enhancements are clearly necessary for DRX 500 km/h operation of NR.

Proposal 1 : Intrafrequency DRX RRM requirements in RRC connected state are enhanced for high speed (500km/h) operation

Firstly, the 1.5x relaxation factor which was introduced for power saving for DRX≤320ms (due to possible misalignment between SMTC and DRX on duration) is counterproductive in this case, since it will lead to a scaling factor of 8 samples (ceil(1.5x5)) compared with 5 samples, or equivalently a 60% increase in measurement delays. To mitigate this and achieve 500km/h operation the gNB may configure a short enough DRX cycle (such as 40ms), however this will lead to higher UE power consumption than if the UE were operating with 80ms DRX and not having the 1.5x relaxation.

Proposal 2 : The 1.5x relaxation factor for DRX≤320ms is not applied for high speed enhanced RRM measurements at 500km/h

In addition, based on link level simulation results, it seems feasible to consider an enhanced measurement period for DRX of 3 samples.

Proposal 3 : The measurement period for high speed is enhanced to 3 samples (3*max(SMTC period, DRX cycle))

If proposal 2 is followed, the resulting requirements are shown in table 3 and 4

|  | ISD | 500 | 1000 | 1500 | 2000 |
| --- | --- | --- | --- | --- | --- |
| Tdrx |  |  |  |  |  |
| 40 |  | 1.544158 | 0.772079 | 0.514719386 | 0.386039539 |
| 80 |  | 3.088316 | 1.544158 | 1.029438772 | 0.772079079 |
| 160 |  | 6.176633 | 3.088316 | 2.058877544 | 1.544158158 |
| 320 |  | 12.35327 | 6.176633 | 4.117755088 | 3.088316316 |
| 640 |  | 24.70653 | 12.35327 | 8.235510175 | 6.176632632 |
| 1280 |  | 49.41306 | 24.70653 | 16.47102035 | 12.35326526 |
| 2560 |  | 98.82612 | 49.41306 | 32.9420407 | 24.70653053 |

Table 3 : dB change in one cell during Tidentify at 500km/h without 1.5x relaxation and enhanced measurement period

|  | ISD | 500 | 1000 | 1500 | 2000 |
| --- | --- | --- | --- | --- | --- |
| Tdrx |  |  |  |  |  |
| 40 |  | 0.579059 | 0.28953 | 0.19302 | 0.144765 |
| 80 |  | 1.158119 | 0.579059 | 0.38604 | 0.28953 |
| 160 |  | 2.316237 | 1.158119 | 0.772079 | 0.579059 |
| 320 |  | 4.632474 | 2.316237 | 1.544158 | 1.158119 |
| 640 |  | 9.264949 | 4.632474 | 3.088316 | 2.316237 |
| 1280 |  | 18.5299 | 9.264949 | 6.176633 | 4.632474 |
| 2560 |  | 37.0598 | 18.5299 | 12.35327 | 9.264949 |

Table 4 : dB change in one cell during Tmeasure at 500km/h without 1.5x relaxation and enhanced measurement period

This change makes 80ms DRX cycle operation feasible for 500km/h with 1km ISD. However, we note that <1km ISD has been discussed within the context of high speed NR demodulation. In addition,  RAN4 needs to discuss the ambition to use other DRX cycles; ultimately any requirement for any DRX cycle can be enhanced if the UE is assumed to wake up more frequently, but this partially takes away the possibility for power saving. Roughly speaking, to support 160ms DRX cycle, the legacy RRM requirements would need both removal of the 1.5x scaling factor, and an additional halving of the number of samples needed (based on 1 sample per DRX). Alternatively a more moderate enhancement could be considered, however it would be more useful with larger than 1000m ISD.

Proposal 4 : RAN4 discusses which RRC connected DRX cycles to further enhance beyond proposal 2 for high speed operation.

# Conclusions

In this contribution we developed an analysis framework based on free space path loss considerations, and the rate of change of RSRP at the cell edge for different sizes of cell (ISD).

Based on this we observe

Observation 1 : Legacy cell identification and measurement requirements with DRX are insufficient in most cases at 500km/h except for large ISD and relatively short DRX cycle

From this observation we propose

Proposal 1 : Intrafrequency DRX RRM requirements in RRC connected state are enhanced for high speed (500km/h) operation

Proposal 2 : The 1.5x relaxation factor for DRX≤320ms is not applied for high speed enhanced RRM measurements at 500km/h

Proposal 3 : The measurement period for high speed is enhanced to 3 samples (3*max(SMTC period, DRX cycle))

Proposal 4 : RAN4 discusses which RRC connected DRX cycles to further enhance beyond proposal 2 for high speed operation.

# References

- R4-1912480, “Initial system simulation results for Rel-16 NR high speed train scenarios”, Nokia, Nokia Shanghai Bell