3GPP TSG-RAN WG4 Meeting #93	R4-1914666

# Reno, US, November 18-22, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-1 | CR | 0205 | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR Modulation fallback for total power dynamic range in 37.145-1 clause 6.3.4.4.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Futurewei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The endorsed draft CR is R4-1912029. The total power dynamic range test uses a full bandwidth test model (TM3.1, TM3.1a) and a single RB test model (TM2, TM2a). There are several rules: If 256QAM is supported with no backoff: TM3.1a and TM2a are used If 256QAM is not supported: TM3.1 and TM2 are used If 256QAM is supported with backoff: TM3.1 is used. It is unclear which test model to use: TM2 or TM2a This will mirror the change captured in R4-1908470 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reorder the conditions for test model selection |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.4.4.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.3.4.4	Method of test

##### 6.3.4.4.1	Initial conditions

6.3.4.4.1.1	General test conditions

Test environment: normal; see annex B.

RF channels to be tested: M; see subclause 4.12.1.

6.3.4.4.1.2	UTRA FDD

Base Station RF Bandwidth positions to be tested for multi-carrier: BRFBW, MRFBW and TRFBW in single band operation; see subclause 4.12.1.

Set each TAB connector to output a signal in accordance to TM2, in TS 25.141 [18], subclause 6.1.1.2.

6.3.4.4.1.3	E-UTRA

Set the Channel set-up of the TAB connector transmitted signal according to:

-	E-TM3.1, or

-	sE-TM3.1-1 for subslot TTI, or

-	sE-TM3.1-2 for slot TTI.

6.3.4.4.1.4	NR

Set the Channel set-up of the TAB connector transmitted signal according to:

-	NR-FR1-TM3.1a if 256QAM is supported by BS without power back off, or

-	NR-FR1-TM3.1 if 256QAM is supported by BS with power back off, or

-	NR-FR1-TM3.1 if 256QAM is not supported by BS.

##### 6.3.4.4.2	Procedure

6.3.4.4.2.1	General procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect TAB connector to measurement equipment as shown in annex D.1.1. All TAB connectors not under test shall be terminated.

2)	Set the TAB connector to transmit at manufacturers declared rated carrier output power per TAB connector (PRated,c,TABC).