3GPP TSG RAN WG4 Meeting #93	R4-1914732

Reno, USA, November 18 - 22, 2019

Agenda Item:	9.4.5.4

Source: 	Ericsson

Title:	Discussions related to interruptions requirements for NR SL V2X

Document for:	Discussions

# Introduction

At previous RAN4 (#92bis) meeting, RAN4 discussed interruption due to simultaneous NR Uu BWP switching and NR V2X SL BWP configuration/reconfiguration and reach following agreement [1]:

Agreement Keep 2 options, one of options will be selected in next meeting - Opt1 : Not define the interruption requirement and NR Uu BWP switching delay is not impacted - Opt2 : NR Uu BWP switching delay could be extended

In this contribution, we discuss and provide our view on the options.

# Discussion

RAN4 agreed on following UE behaviour when both NR Uu BWP switching and NR SL V2X BWP configurations take place simultaneously:

  - Opt1 : Not define the interruption requirement and NR Uu BWP switching delay is not impacted

  - Opt2 : NR Uu BWP switching delay could be extended

We support option 1, i.e. no interruptions to the NR Uu operation is allowed when these operations take place at the same in the UE. This implies that UE may either delay the SL operation until the NR Uu operation is completed or it may simply discard the SL operation. But the exact UE behaviour needs to be discussed and specified otherwise, there is a risk that UE becomes unavailable for scheduling in the configured BW. We see an advantage is delaying the SL BWP operation instead of discarding it which may have degrade the SL performance, e.g. the unicast transmissions/receptions.

- Proposal: When NR Uu BWP switching and NR SL V2X BWP configurations take place simultaneously, option 1 (as agreed at last meeting) is supported, and the UE shall delay the SL BWP operation.

# Summary

We have in this contribution discussed the interruption requirements for both NR Uu and NR SL V2X, more specifically the options related to the UE behaviour when both NR Uu BWP switching and NR SL V2X BWP operations take place simultaneously.

- Proposal: When NR Uu BWP switching and NR SL V2X BWP configurations take place simultaneously, option 1 (as agreed at last meeting) is supported, and the UE shall delay the SL BWP operation.

# References

- R4-1912821, “Ad-hoc minutes of NR V2X RRM”, LG Electronics, October 2019