3GPP TSG-RAN WG4 Meeting #93	R4-1914826

# Reno, Nevada, USA, Nov 18th – 22nd, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0271 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on introducing DAPS handover requirements in TS38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | R16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For NR mobility enhancement WID, the DAPS handover has been introduced and the |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To define DAPS handover requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The report mapping for L1-SINR measurement will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

6.1.3	NR DAPS Handover

6.1.3.1	Introduction

The purpose of NR DAPS handover is to change the NR PCell to another NR cell.

6.1.3.2	NR FR1 - NR FR1 DAPS Handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers from NR FR1 cell to NR FR1 cell.

6.1.3.2.1	DAPS handover delay

Procedure delays for the procedure that can command a DAPS handover are specified in TS 38.331 [2].

When the UE receives a RRC message implying handover, the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover1 seconds from the end of the last TTI containing the RRC command when UE is configured with dual active protocol stack handover.

Dhandover1 = TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin ms

Where:

TRRC_procedure is the maximum RRC procedure delay to be defined in clause 12 in TS 38.331 [2].

Tsearch, TIU, Tprocessing, T∆ and Tmargin are defined in clause 6.1.1.2.2.

After successful RACH procedure of the target cell, when the UE receives a [TBD] message implying source cell release command, the UE shall accomplish the release actions specified in TS 38.331 [2] within Dhandover2.

Dhandover2 = TRRC_procedure + Tprocessing_delay ms

Where:

TRRC_procedure is the maximum RRC procedure delay to be defined in clause 12 in TS 38.331 [2].

Tprocessing_delay equals [TBD].

6.1.3.2.2	Interruption time

During Dhandover1, the UE is allowed an interruption on source cell.

For intra-frequency deployment, no interruption is allowed to source cell when the same bandwidths is used for target and source cell, otherwise the interruption time to source cell shall be less than Tinterrupt1 as specified in Table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Tinterrupt1 for intra-frequency DAPS HO

|  | NR Slot length (ms) | Interruption length X (slotsnote 1) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 Note 2 | 3 |
| Note 1: The same SCS of source cell and target cell is assumed. Note 2: Both source cell and target cell is on FR1. |  |  |

For inter-frequency intra-band deployment, the interruption time to source cell shall be less than Tinterrupt1 as specified in Table 6.1.3.2.2-2.

Table 6.1.3.2.2-2: Tinterrupt1 for inter-frequency intra-band sync DAPS HO

|  | NR Slot length (ms) | Interruption length (slot) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration |
| 1 | 0.5 | 2 + TSMTC_duration |
| 2 | 0.25 Note 1 | 4 + TSMTC_duration |
| Note 1: Both source cell and target cell are on FR1. Note 2: TSMTC_duration is the longest SMTC duration between source cell and target cell. |  |  |

For inter-frequency inter-band deployment, the interruption time to source cell shall be less than Tinterrupt1 as specified in Table 6.1.3.2.2-3.

Table 6.1.3.2.2-3: Tinterrupt1 for inter-frequency inter-band DAPS HO

|  | NR Slot length (ms) of source cell | Tinterrupt1 (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25Note 1 | 5 | 5 |
| Note 1: Both source cell or target cell is on FR1. |  |  |  |

During Dhandover2, the UE is allowed an interruption on target cell.

For intra-frequency deployment, no interruption is allowed to target cell when the same bandwidths is used for target and source cell, otherwise the interruption time to target cell shall be less than Tinterrupt2 as specified in Table 6.1.3.2.2-4.

Table 6.1.3.2.2-4: Tinterrupt2 for intra-frequency DAPS HO

|  | NR Slot length (ms) | Interruption length X (slotsnote 1) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 Note 2 | 3 |
| Note 1: The same SCS of source cell and target cell is assumed. Note 2: Both source cell and target cell is on FR1. |  |  |

For inter-frequency intra-band deployment, the interruption time to target cell shall be less than Tinterrupt2 as specified in Table 6.1.3.2.2-5.

Table 6.1.3.2.2-5: Tinterrupt2 for inter-frequency intra-band sync DAPS HO

|  | NR Slot length (ms) | Interruption length (slot) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration |
| 1 | 0.5 | 2 + TSMTC_duration |
| 2 | 0.25 Note 1 | 4 + TSMTC_duration |
| Note 1: Both source cell and target cell is on FR1. Note 2: TSMTC_duration is the longest SMTC duration between source cell and target cell. |  |  |

For inter-frequency inter-band deployment, the interruption time to target cell shall be less than Tinterrupt2 as specified in Table 6.1.3.2.2-6.

Table 6.1.3.2.2-6: Tinterrupt2 for inter-frequency inter-band DAPS HO

|  | NR Slot length (ms) of Target cell | Tinterrupt2 (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25Note 1 | 5 | 5 |
| Note 1: Either source cell or target cell is on FR1. |  |  |  |

6.1.3.3	NR FR2- NR FR1 DAPS Handover

The requirements in this clause are applicable to inter-frequency handovers from NR FR2 cell to NR FR1 cell.

6.1.3.3.1	DAPS handover delay

Procedure delays for the procedure that can command a DAPS handover are specified in TS 38.331 [2].

When the UE receives a RRC message implying handover, the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover1 seconds from the end of the last TTI containing the RRC command when UE is configured with dual active protocol stack handover.

Dhandover1 = TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin ms

Where:

TRRC_procedure is the maximum RRC procedure delay to be defined in clause12 in TS 38.331 [2].

Tsearch, TIU, Tprocessing, T∆ and Tmargin are defined in clause 6.1.1.3.2.

After successful RACH procedure of the target cell, when the UE receives a [TBD] message implying source cell release command, the UE shall accomplish the release actions specified in TS 38.331 [2] within Dhandover2.

Dhandover2 = TRRC_procedure + Tprocessing_delay ms

Where:

TRRC_procedure is the maximum RRC procedure delay to be defined in clause12 in TS 38.331 [2].

Tprocessing_delay equals [TBD].

6.1.3.3.2	Interruption time

During Dhandover1, the UE is allowed an interruption on source cell.

For inter-frequency inter-band deployment, the interruption time to source cell shall be less than Tinterrupt1 as specified in Table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Tinterrupt1 for inter-frequency inter-band DAPS HO

|  | NR Slot length (ms) of Source cell | Tinterrupt1 (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 2 | 0.25Note 1 | 5 | 5 |
| 3 | 0.125 | 9 | 9 |
| Note 1: Source cell is on FR2. |  |  |  |

During Dhandover2, the UE is allowed an interruption on target cell.

For inter-frequency inter-band deployment, the interruption time to target cell shall be less than Tinterrupt2 as specified in Table 6.1.3.3.2-2.

Table 6.1.3.3.2-2: Tinterrupt2 for inter-frequency inter-band DAPS HO

|  | NR Slot length (ms) of Target cell | Tinterrupt2 (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25Note 1 | 5 | 5 |
| Note 1: Target cell is on FR1. |  |  |  |

6.1.3.4	NR FR1- NR FR2 DAPS Handover

The requirements in this clause are applicable to inter-frequency handovers from NR FR1 cell to NR FR2 cell.

6.1.3.4.1	DAPS handover delay

Procedure delays for the procedure that can command a DAPS handover are specified in TS 38.331 [2].

When the UE receives a RRC message implying handover, the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover1 seconds from the end of the last TTI containing the RRC command when UE is configured with dual active protocol stack handover.

Dhandover1 = TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin ms

Where:

TRRC_procedure is the maximum RRC procedure delay to be defined in clause12 in TS 38.331 [2].

Tsearch, TIU, Tprocessing, T∆ and Tmargin are defined in clause 6.1.1.5.2.

After successful RACH procedure of the target cell, when the UE receives a [TBD] message implying source cell release command, the UE shall accomplish the release actions specified in TS 38.331 [2] within Dhandover2.

Dhandover2 = TRRC_procedure + Tprocessing_delay ms

Where:

TRRC_procedure is the maximum RRC procedure delay to be defined in clause12 in TS 38.331 [2].

Tprocessing_delay equals [TBD].

6.1.3.4.2	Interruption time

During Dhandover1, the UE is allowed an interruption on source cell.

For inter-frequency inter-band deployment, the interruption time to source cell shall be less than Tinterrupt1 as specified in Table 6.1.3.4.2-1.

Table 6.1.3.4.2-1: Tinterrupt1 for inter-frequency inter-band DAPS HO

|  | NR Slot length (ms) of source cell | Tinterrupt1 (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25Note 1 | 5 | 5 |
| Note 1: Source cell is on FR1. |  |  |  |

During Dhandover2, the UE is allowed an interruption on target cell.

For inter-frequency inter-band deployment, the interruption time to target cell shall be less than Tinterrupt2 as specified in Table 6.1.3.4.2-2.

Table 6.1.3.4.2-2: Tinterrupt2 for inter-frequency inter-band DAPS HO

|  | NR Slot length (ms) of Target cell | Tinterrupt2 (slots) |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 2 | 0.25Note 1 | 5 | 5 |
| 3 | 0.125 Note 2 | 9 | 9 |
| Note 1: Target cell is on FR2. |  |  |  |

<End of Change 1>