3GPP TSG-RAN WG4 Meeting #93	R4-1914830

Reno, Nevada, USA, Nov 18th – 22nd, 2019

Title: 	Simulation results of SyncRef UE identification for NR V2X

Source: 	Huawei, HiSilicon

Agenda item:	9.4.5.2

Document for:	Discussion

Introduction

In this paper, we present the simulation results of SyncRef UE identification performance.

Discussion

Simulation Assumption

Based on the simulation assumption for NR V2X S-PSS/S-SSS detection agreed in [1], we evaluate the  SyncRef UE identification performance for both synchronous and asynchronous cases as listed in Table 1.

Table 1: SLSS IDs of the SyncRef UEs

| Case | SyncRef UE 1 | SyncRef UE 2 | Synchronous/ Asynchronous |
| --- | --- | --- | --- |
| #1 | 336+112 | 112 | Synchronous |
| #2 | 336+224 | 112 | Synchronous |
| #3 | 336+112 | 112 | Asynchronous |
| #4 | 336+224 | 112 | Asynchronous |

Simulation Results

In this section, we provide the 90% percentile of NR V2X S-PSS/S-SSS detection time for SyncRef UE identification. The results in Table 4 show the number of required S-SSB occasions for S-PSS/S-SSS detection.

Table 2: 90% NR V2X S-PSS/S-SSS detection time for AWGN

| SL SCS | Case | AWGN |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | SINR= -2dB | SINR= -1dB | SINR= 0dB | SINR= 1dB | SINR= 1dB |
| 15KHz | #1 | 1 | 1 | 1 | 1 | 1 |
|  | #2 | 1 | 1 | 1 | 1 | 1 |
|  | #3 | 1 | 1 | 1 | 1 | 1 |
|  | #4 | 1 | 1 | 1 | 1 | 1 |
| 30KHz | #1 | 1 | 1 | 1 | 1 | 1 |
|  | #2 | 1 | 1 | 1 | 1 | 1 |
|  | #3 | 1 | 1 | 1 | 1 | 1 |
|  | #4 | 1 | 1 | 1 | 1 | 1 |
| 60KHz | #1 | 1 | 1 | 1 | 1 | 1 |
|  | #2 | 1 | 1 | 1 | 1 | 1 |
|  | #3 | 1 | 1 | 1 | 1 | 1 |
|  | #4 | 1 | 1 | 1 | 1 | 1 |

Table 3: 90% NR V2X S-PSS/S-SSS detection time for TDLC-100ns-150Hz

| SL SCS | Case | TDL-C with 100ns 150Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | SINR= -2dB | SINR= -1dB | SINR= 0dB | SINR= 1dB | SINR= 1dB |
| 15KHz | #1 | 2 | 2 | 1 | 1 | 1 |
|  | #2 | 2 | 1 | 1 | 1 | 1 |
|  | #3 | 1 | 1 | 1 | 1 | 1 |
|  | #4 | 1 | 1 | 1 | 1 | 1 |
| 30KHz | #1 | 2 | 1 | 1 | 1 | 1 |
|  | #2 | 1 | 1 | 1 | 1 | 1 |
|  | #3 | 1 | 1 | 1 | 1 | 1 |
|  | #4 | 1 | 1 | 1 | 1 | 1 |
| 60KHz | #1 | 1 | 1 | 1 | 1 | 1 |
|  | #2 | 1 | 1 | 1 | 1 | 1 |
|  | #3 | 1 | 1 | 1 | 1 | 1 |
|  | #4 | 1 | 1 | 1 | 1 | 1 |

Table 4: 90% NR V2X S-PSS/S-SSS detection time for TDLC-100ns-300Hz

| SL SCS | Case | TDL-C with 100ns 300Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | SINR= -2dB | SINR= -1dB | SINR= 0dB | SINR= 1dB | SINR= 1dB |
| 15KHz | #1 | 2 | 1 | 1 | 1 | 1 |
|  | #2 | 2 | 2 | 1 | 1 | 1 |
|  | #3 | 1 | 1 | 1 | 1 | 1 |
|  | #4 | 1 | 1 | 1 | 1 | 1 |
| 30KHz | #1 | 2 | 1 | 1 | 1 | 1 |
|  | #2 | 2 | 1 | 1 | 1 | 1 |
|  | #3 | 1 | 1 | 1 | 1 | 1 |
|  | #4 | 1 | 1 | 1 | 1 | 1 |
| 60KHz | #1 | 1 | 1 | 1 | 1 | 1 |
|  | #2 | 1 | 1 | 1 | 1 | 1 |
|  | #3 | 1 | 1 | 1 | 1 | 1 |
|  | #4 | 1 | 1 | 1 | 1 | 1 |

Table 5: 90% NR V2X S-PSS/S-SSS detection time for TDLC-10ns-1400Hz

| SL SCS | Case | TDL-C with 10ns 1400Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | SINR= -2dB | SINR= -1dB | SINR= 0dB | SINR= 1dB | SINR= 1dB |
| 15KHz | #1 | 2 | 1 | 1 | 1 | 1 |
|  | #2 | 2 | 2 | 1 | 1 | 1 |
|  | #3 | 1 | 1 | 1 | 1 | 1 |
|  | #4 | 1 | 1 | 1 | 1 | 1 |
| 30KHz | #1 | 2 | 2 | 1 | 1 | 1 |
|  | #2 | 2 | 2 | 1 | 1 | 1 |
|  | #3 | 1 | 1 | 1 | 1 | 1 |
|  | #4 | 1 | 1 | 1 | 1 | 1 |
| 60KHz | #1 | 2 | 2 | 1 | 1 | 1 |
|  | #2 | 2 | 2 | 1 | 1 | 1 |
|  | #3 | 1 | 1 | 1 | 1 | 1 |
|  | #4 | 1 | 1 | 1 | 1 | 1 |

Table 5: 90% NR V2X S-PSS/S-SSS detection time for TDLC-30ns-1400Hz

| SL SCS | Case | TDL-C with 30ns 1400Hz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | SINR= -2dB | SINR= -1dB | SINR= 0dB | SINR= 1dB | SINR= 1dB |
| 15KHz | #1 | 1 | 1 | 1 | 1 | 1 |
|  | #2 | 2 | 2 | 1 | 1 | 1 |
|  | #3 | 1 | 1 | 1 | 1 | 1 |
|  | #4 | 1 | 1 | 1 | 1 | 1 |
| 30KHz | #1 | 2 | 2 | 1 | 1 | 1 |
|  | #2 | 2 | 1 | 1 | 1 | 1 |
|  | #3 | 1 | 1 | 1 | 1 | 1 |
|  | #4 | 1 | 1 | 1 | 1 | 1 |
| 60KHz | #1 | 2 | 1 | 1 | 1 | 1 |
|  | #2 | 2 | 1 | 1 | 1 | 1 |
|  | #3 | 1 | 1 | 1 | 1 | 1 |
|  | #4 | 1 | 1 | 1 | 1 | 1 |

From the table, we can see the following observations.

Observation 1: For AWGN channel, the S-PSS/S-SSS detection time for SyncRef UE identification requires one S-SSB occasion with SINR ≥ -2dB.

Observation 2: For fading channel TDL-C, the S-PSS/S-SSS detection time for SyncRef UE identification requires one S-SSB occasions with SINR ≥ 0dB.

Conclusions

In this contribution, we provide the simulation results of S-PSS/S-SSS detection performance for NR V2X sidelink. The followings are provided:

Observation 1: For AWGN channel, the S-PSS/S-SSS detection time for SyncRef UE identification requires one S-SSB occasion with SINR ≥ -2dB.

Observation 2: For fading channel TDL-C, the S-PSS/S-SSS detection time for SyncRef UE identification requires one S-SSB occasions with SINR ≥ 0dB.

Reference

- R4-1912828, “Link-level simulation assumptions for NR V2X synchronization”, MediaTek Inc., LG Electronics