3GPP TSG-RAN WG4 Meeting #93	R4-1914835

# Reno, Nevada, USA, Nov 18th – 22nd, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0274 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on introducing L1-SINR mapping in TS38.133 R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | R16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN4#92bis meeting, the following has been agreed on SINR measurement reporting table for NR eMIMO: - Extend current L3 SS-SINR measurement reporting table for L1 SS-SINR and L1 CSI-SINR usage, with maintaining the range and quantization of SINR mapping. - Introduce differential SS-SINR and CSI-SINR measurement (for L1 reporting) report mapping table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To extend Table 10.1.16.1-1 for L1-SINR mapping. - To introduce Table 10.1.16.1-2 for differential L1-SINR mapping |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The report mapping for L1-SINR measurement will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 10.1.16	SINR report mapping

#### 10.1.16.1	SS-SINR and CSI-SINR measurement report mapping

The reporting range of SS-SINR and CSI-SINR is defined from -23 dB to 40 dB with 0.5 dB resolution. The mapping of measured quantity is defined in Table 10.1.16.1-1. The range in the signalling may be larger than the guaranteed accuracy range.

The reporting range of differential SS-SINR and CSI-SINR for L1 reporting is defined from 0 dBm to -15 dB with 1 dB resolution.

The mapping of measured quantity is defined in Table 10.1.16.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.16.1-1: SS-SINR and CSI-RSRP measurement report mapping

| Reported value | Measured quantity value (L3 SS-SINR) | Measured quantity value (L1 SS-SINR and CSI-SINR) |  | Unit |
| --- | --- | --- | --- | --- |
| SINR_0 | SS-SINR<-23 | SINR<-23 |  | dB |
| SINR_1 | -23≤ SS-SINR<-22.5 | -23≤SINR<-22.5 |  | dB |
| SINR_2 | -22.5≤ SS-SINR<-22 | -22.5≤SINR<-22 |  | dB |
| SINR_3 | -22≤ SS-SINR<-21.5 | -22≤L1-SINR<-21.5 |  | dB |
| SINR_4 | -21.5≤ SS-SINR<-21 | -21.5≤SINR<-21 |  | dB |
| .. | .. | .. |  | … |
| SINR_123 | 38≤ SS-SINR<38.5 | 38≤SINR<38.5 |  | dB |
| SINR_124 | 38.5≤ SS-SINR<39 | 38.5≤SINR<39 |  | dB |
| SINR_125 | 39≤ SS-SINR<39.5 | 39≤SINR<39.5 |  | dB |
| SINR_126 | 39.5≤ SS-SINR<40 | 39.5≤SINR<40 |  | dB |
| SINR_127 | 40≤ SS-SINR | 40≤SINR |  | dB |

Table 10.1.16.1-2: Differential SS-SINR and CSI-SINR measurement (for L1 reporting) report mapping

| Reported value | Measured quantity value (difference in measured SINR from strongest SINR) | Unit |
| --- | --- | --- |
| DIFFSINR_0 | 0≥ΔSINR>-1 | dB |
| DIFFSINR_1 | -1≥ΔSINR>-2 | dB |
| DIFFSINR_2 | -2≥ΔSINR>-3 | dB |
| DIFFSINR_3 | -3≥ΔSINR>-4 | dB |
| DIFFSINR_4 | -4≥ΔSINR>-5 | dB |
| DIFFSINR_5 | -5≥ΔSINR>-6 | dB |
| DIFFSINR_6 | -6≥ΔSINR>-7 | dB |
| DIFFSINR_7 | -7≥ΔSINR>-8 | dB |
| DIFFSINR_8 | -8≥ΔSINR>-9 | dB |
| DIFFSINR_9 | -9≥ΔSINR>-10 | dB |
| DIFFSINR_10 | -10≥ΔSINR>-11 | dB |
| DIFFSINR_11 | -11≥ΔSINR>-12 | dB |
| DIFFSINR_12 | -12≥ΔSINR>-13 | dB |
| DIFFSINR_13 | -13≥ΔSINR>-14 | dB |
| DIFFSINR_14 | -14≥ΔSINR>-15 | dB |
| DIFFSINR_15 | -15≥ΔSINR | dB |

<End of Change 1>