3GPP TSG-RAN4 Meeting #93	R4-1914970

# Reno, US, 18th – 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6762 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on accuracy requriements for quality report in eMTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2019-10-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Accuracy requirements for DL quality reporting should be defined for Rel-16 eMTC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define accuracy requirements for DL quality reporting for Rel-16 eMTC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DL quality reporting for Rel-16 eMTC is not supported from RRM perspective. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 9.1.21.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 9.1.21.22	Downlink Channel Quality Measurement Accuracy for UE Category M1

The requirements for accuracy of downlink channel quality reporting in this clause apply for UE Category M1.

The accuracy requirements in Table 9.1.21.22-1 and Table 9.1.21.22-2 are valid under the following conditions:

-	Cell specific reference signals are transmitted either from one or two ports.

-	Conditions defined in TS 36.101 [5] Clause 7.3 for reference sensitivity are fulfilled.

-	RSRP|dBm according to Annex B.3.1 for a corresponding Band.

Table 9.1.21.22-1: Downlink channel quality reporting accuracy for UE Category M1, repetition number in DL quality information larger than 1

| PDCCH Repetition | Pm-Dsg (%) | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ês/Iot | Io NOTE 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups NOTE 2 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -6 dB | FDD-M1_A, TDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_B | -120.5 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| R/4 NOTE 1 | >1 | -6 dB | Note 4 | Note 4 | Note 4 | Note 4 |
| R NOTE 1 | ≤1 | -15 ≤ Ês/Iot ≤ -6 dB | Note 4 | Note 4 | Note 4 | Note 4 |
| R/8 NOTE 1 | >1 | -15 ≤ Ês/Iot ≤ -6 dB | Note 4 | Note 4 | Note 4 | Note 4 |
| NOTE 1: AL is the reported MPDCCH aggregation level that UE has reported. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as R reported and Ês/Iot-6 dB. |  |  |  |  |  |  |

Table 9.1.21.22-2: Downlink channel quality reporting accuracy for UE Category M1, repetition number in DL quality information equal to 1

| PDCCH Aggregation Level | Pm-Dsg (%) | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ês/Iot | Io NOTE 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups NOTE 2 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| AL NOTE 1 | ≤1 | -6 dB | FDD-M1_A, TDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_B | -120.5 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| AL/4 NOTE 1 | >1 | -6 dB | Note 4 | Note 4 | Note 4 | Note 4 |
| NOTE 1: AL is the reported MPDCCH aggregation level that UE has reported. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for AL reported. |  |  |  |  |  |  |

<End of Change 1>