3GPP TSG-RAN4 Meeting #93	R4-1914971

# Reno, US, 18th – 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6763 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on report mapping for quality report in eMTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2019-10-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Report mapping for DL quality reporting should be defined for Rel-16 eMTC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define report mapping for DL quality reporting for Rel-16 eMTC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DL quality reporting for Rel-16 eMTC is not supported from RRM perspective. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 9.1.21.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 9.1.21.23	Report Mapping for Downlink Channel Quality Measurement for UE Category M1

The report mapping for DL channel quality measurement are specified in Table 9.1.21.23-1 for reporting with 8-bit, and in Table 9.1.21.23-2 for reporting with 2-bit.

Table 9.1.21.23-1: Downlink channel quality measurement report mapping of DL channel quality reporting with 8-bit

| Reported level | MPDCCH repetition level | MPDCCH aggregation level | CE mode |
| --- | --- | --- | --- |
| 0 | No measurement reporting | No measurement reporting | A, B |
| 1 | 1 | 1 | A |
| 2 | 1 | 2 | A |
| 3 | 1 | 4 | A |
| 4 | 1 | 8 | A |
| 5 | 1 | 12 | A |
| 6 | 1 | 16 | A |
| 7 | 1 | 24 | A |
| 8 | 2 | 24 | A, B |
| 9 | 4 | 24 | A, B |
| 10 | 8 | 24 | A, B |
| 11 | 16 | 24 | A, B |
| 12 | 32 | 24 | A, B |
| 13 | 64 | 24 | A, B |
| 14 | 128 | 24 | A, B |
| 15 | 256 | 24 | A, B |

Table 9.1.21.23-2: Downlink channel quality measurement report mapping of DL channel quality reporting with 2-bit

| Reported value | NPDCCH repetition level |
| --- | --- |
| candidateRep-1 | Rmax/8 (NOTE 1) |
| candidateRep-2 | Rmax/4 (NOTE 2) |
| candidateRep-3 | Rmax (NOTE 3) |
| candidateRep-4 | 4×Rmax (NOTE 4) |
| NOTE 1: When Rmax is less than 8, set candidateRep-1 to 1. NOTE 2: When Rmax is less than 4, set candidateRep-2 to 1. NOTE 3: When Rmax is 1, set candidateRep-3 to 2. NOTE 4: When Rmax is more than 64, set candidateRep-4 to 256. NOTE 5: MPDCCH Aggregation level is assumed as 24. |  |

<End of Change 1>