3GPP TSG-RAN WG4 RAN4#93 	R4-1914983

Reno, Nevada November 18th – 22nd, 2019

Agenda item:	9.5.5.2

Source: 	Qualcomm

Title: 	MT Timing Related Requirements

Document for:	Discussion

# Introduction

During the last meeting, RAN4 RRM session made the following agreement [1]

RAN4 agrees to define IAB RRM requirements based on the following table. Requirements; Comments; Is requirement needed?. Cell reselection; - Intra-frequency - Inter-frequency; No. Handover; FFS. RRC Connection Mobility Control; - RRC re-establishment - Random access - RRC release with redirection; Yes. SCell activation/deactivation delays; No. Interruptions related to SCells addition, release, activation, deactivation; Interruptions on serving cells due to any of the addition, release, activation, or deactivation procedures; No. PSCell addition/release delays; No. Active BWP switching; - Interruptions at active BWP switching - Active BWP switching delay; No. TCI switching; TCI switching delay; FFS. Requirements; Comments; Is requirement needed?. RLM; Requirements for RLM, e.g., based on SSB, CSI-RS or both; FFS. Link recovery procedures; Requirements for SSB-based and CSI-RS based BFD and CBD; FFS. Measurements requirements (time duration, number of cells, etc.); FFS. Measurement accuracy requirements; FFS. MT timing related requirements; - MT transmit timing (Te and Tq) - Timing Advance; Yes. - MRTD - MTTD; FFS. Switching time between MT and DU; ---; TBD in RAN4 RF session. DU timing related requirements; - 3 us requirement; Yes. FFS. - T_delta command.

This contribution focuses on the detailed requirements regarding RRC connection mobility control for IAB nodes.

# Discussion

## MT Transmit Timing

During the last meeting, RAN4 agreed to support Te and Tq transmit requirements.

Tq defines the maximum aggregate adjustment rate during gradual timing adjustment. The Tq requirement that have been currently defined for UEs can be reused for MTs. RAN4 also needs to define the minimum aggregate adjustment for IAB MTs and reuse the Tp requirement that has been currently defined for UEs.

Observation 1: RAN4 has agreed to define requirement for maximum aggregate adjustment rate, Tq, but has not defined requirement for minimum aggregate adjustment rate, Tp, yet.

Proposal 1: RAN4 supports defining requirements for minimum aggregate adjustment rate Tp.

- For an IAB node, the requirements for Tp and Tq are same as those for a Rel-15 UE.

Te defines the limit of initial transmission timing error and is applied when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS or it is the PRACH transmission. According to current version of 38.133, “The UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms.” [2]. If a DL SSB is available during the last 160 ms, the Te requirement for an MT can be same as that for a Rel-15 UE.

But, RAN1 allowed network to configure SMTC with periodicity higher than 160 ms in an IAB network. The corresponding RAN1 agreement is shown below:

Agreements: The configurable values of the parameters in the SSB reception configurations including SMTC for IAB node discovery and measurement are provided in the following - SMTC window periodicity: - 5ms, 10ms, 20ms, 40ms, 80ms, 160ms, 320ms, 640ms, 1280ms - SMTC window timing offset: - [0, ..., (number of subframes within SMTC window periodicity) – 1] - SMTC window duration: - [1, …, 5] (subframes) - FFS larger than 5 - List of physical cell IDs to be measured - [cell ID 0, …, cell ID M-1 ] - FFS value of M - SSB-ToMeasure - Same as Rel-15

If network has not configured any additional SSB to MT for RLM and BFD, then an SMTC periodicity of 1280 ms will not always guarantee the presence of DL SSB during the last 160 ms.

Observation 2: The initial UL TX timing error requirement is applicable if a DL SSB is available during the last 160 ms. In IAB, network can configure SMTC window periodicity to be up to 1280 ms. An SMTC periodicity of greater than 160 ms will not obviously guarantee the presence of DL SSB during the last 160 ms.

Recently, NR-U discussed a similar topic. The possible options were: 1) If no DL SSB is not available during the last 160 ms, MT does not transmit. 2) If no DL SSB is not available during the last 160 ms, MT can transmit but the requirement for Te is relaxed.

Proposal 2: If a DL SSB is available during the last 160 ms, the Te requirement for an MT is same as that for a Rel-15 UE. If no DL SSB is available during the last 160 ms, down-select from the following two options:

- MT does not transmit.

- MT transmits but Te requirement is relaxed.

  - Relaxed values of Te are upper bounded by Te,max. Value of Te,max is TBD.

## Timing Advance

RAN4 has already agreed to define timing advance requirements for IAB MTs. The timing advance adjustment delay and TA adjustment accuracy requirements of MT can be same as those of Rel-15 UE.

Proposal 3: The TA adjustment delay and accuracy requirements of an IAB MT are same as those of a Rel-15 UE.

## MRTD and MTTD Requirement

RAN4 RF session will discuss whether to support EN-DC and NR CA in IAB networks or not. If EN-DC and NR CA are supported, defining MRTD and MTTD are essential to gurantee performance in IAB networks.

Proposal 4: RAN4 defines MRTD and MTTD requirements for IAB MTs in EN-DC and NR CA. The MRTD and MTTD requirements for IAB MTs are same as those for Rel-15 UE.

# Conclusion

Observation 1: RAN4 has agreed to define requirement for maximum aggregate adjustment rate, Tq, but has not defined requirement for minimum aggregate adjustment rate, Tp, yet.

Observation 2: The initial UL TX timing error requirement is applicable if a DL SSB is available during the last 160 ms. In IAB, network can configure SMTC window periodicity to be up to 1280 ms. An SMTC periodicity of greater than 160 ms will not obviously guarantee the presence of DL SSB during the last 160 ms.

Proposal 1: RAN4 supports defining requirements for minimum aggregate adjustment rate Tp.

For an IAB node, the requirements for Tp and Tq are same as those for a Rel-15 UE.

Proposal 2: If a DL SSB is available during the last 160 ms, the Te requirement for an MT is same as that for a Rel-15 UE. If no DL SSB is available during the last 160 ms, down-select from the following two options:

- MT does not transmit.

- MT transmits but Te requirement is relaxed.

  - Relaxed values of Te are upper bounded by Te,max. Value of Te,max is TBD.

Proposal 3: The TA adjustment delay and accuracy requirements of an IAB MT are same as those of a Rel-15 UE.

Proposal 4: RAN4 defines MRTD and MTTD requirements for IAB MTs in EN-DC and NR CA. The MRTD and MTTD requirements for IAB MTs are same as those for Rel-15 UE.

# References

- Chairman’s notes, 3GPP TSG RAN WG4 Meeting #92bis

- Chairman’s notes, 3GPP TSG RAN WG1 Meeting #97.