3GPP TSG-RAN WG4 Meeting #93	R4-1914999

# Reno, US, 18th November – 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6767 | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 36.133 on interruption requirements for CGI reading |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Enh-Core |  |  |  |  |  | Date: |  |  | 2019-10-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In WID RP-191601, CGI reading requirements with autonomous gap for NR capable UE should be specified for various scenarios. - CGI reading for NR cell (NR capable UE in LTE SA, NR SA UE, EN-DC UE, NE-DC UE, and NR-DC) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Interruption requirements for CGI identification of NR cell are introduced for EN-DC, NR SA, NE-DC and NR-DC. - Interruption requirements for CGI identification of NR cell are introduced for EN-DC and NE-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No interruption requirments for CGI identification of an NR cell or of an LTE cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.32.2.8, 7.36.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change #1 >

7.32.2.8	Interruptions during measurements with autonomous gaps

When a UE is identifying CGI of an NR cell with autonomous gaps, the UE is allowed interruptions on PCell or any activated SCell:

-	with interrupted subframes up to ratio of interruption length X1 subframe specified in Table 7.32.2.8-1 over SMTC period of victim serving cell during MIB decoding time period TMIB_interRAT specified in clause 8.17.4.3 and clause 8.17.4.4.

-	the length of each interruption during MIB decoding time period shall not exceed the number of X1 subframes specified in Table  7.32.2.8-1.

-	with interrupted subframes up to ratio of interruption length Y1 subframe specified in Table 7.32.2.8-1 over 20 ms during SIB1 decoding time period TSIB1_interRAT specified in clause 8.17.4.3 and clause 8.17.4.4, for SSB and CORESET for RMSI scheduling multiplexing patterns 1.

-	the length of each interruption during SIB1 decoding time period shall not exceed the number of Y1 subframes specified in Table  7.32.2.8-1.

-	with interrupted subframes up to ratio of interruption length Y2 subframe specified in Table 7.32.2.8-1 over SMTC period of victim serving cell during SIB1 decoding time period TSIB1_interRAT specified in clause 8.17.4.3 and clause 8.17.4.4, for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3.

-	the length of each interruption during SIB1 decoding time period shall not exceed the number of Y2 subframes specified in Table  7.32.2.8-1.

Table 7.32.2.8-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | Subframe length (ms) | Interruption length X1 subframe | Interruption length Y1 subframe |  | Interruption length Y2 subframe |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sync | Async | Sync | Async |
| 0 | 1 | [2] | [3] | [4] | [2] | [3] |

< End of change #1 >

< Start of change #2 >

7.36.2.8	Interruptions during measurements with autonomous gaps

When a UE is identifying CGI of an NR cell with autonomous gaps, the UE is allowed interruptions on PSCell or any activated SCell:

-	with interrupted subframes up to ratio of interruption length X1 subframe specified in Table 7.36.2.8-1 over SMTC period of victim serving cell during MIB decoding time period TMIB_intra specified in clause 9.2.7 in TS 38.133 [50]  or TMIB_inter specified in clause 9.3.9 in TS 38.133 [50].

-	the length of each interruption during MIB decoding time period shall not exceed the number of X1 subframes specified in Table  7.36.2.8-1.

-	with interrupted subframes up to ratio of interruption length Y1 subframe specified in Table 7.36.2.8-1 over 20 ms during SIB1 decoding time period TSIB1_intra specified in clause 9.2.7 in TS 38.133 [50]  or TSIB1_inter specified in clause 9.3.9 in TS 38.133 [50], for SSB and CORESET for RMSI scheduling multiplexing patterns 1.

-	the length of each interruption during SIB1 decoding time period shall not exceed the number of Y1 subframes specified in Table  7.36.2.8-1.

-	with interrupted subframes up to ratio of interruption length Y2 subframe specified in Table 7.36.2.8-1 over SMTC period of victim serving cell during SIB1 decoding time period TSIB1_intra specified in clause 9.2.7 in TS 38.133 [50] or TSIB1_inter specified in clause 9.3.9 in TS 38.133 [50], for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3.

-	the length of each interruption during SIB1 decoding time period shall not exceed the number of Y2 subframes specified in Table  7.36.2.8-1.

Table 7.36.2.8-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | Subframe length (ms) | Interruption length X1 subframe | Interruption length Y1 subframe |  | Interruption length Y2 subframe |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sync | Async | Sync | Async |
| 0 | 1 | [2] | [3] | [4] | [2] | [3] |

< End of change #2 >