3GPP TSG RAN WG4 Meeting #93	R4-1915059

Reno, Nevada, US November 18 – 22, 2019

Agenda Item:	9.2.3.1

Source: 	Ericsson

Title:	On remaining CLI RRM open issues

Document for:	Discussion

# Introduction

At RAN4#92bis, good progress was made on CLI RRM core requirements, leaving only a few open issues as captured in the WF [1] for SRS-RSRP measurements:

  - Reference SNR point for side condition is SNR with zero timing error

    - Option 1: SNR 3dB

    - Option 2: SNR -1dB

    - Other higher SNR level is not precluded

  - The number of sample for measurement accuracy

    - Option 1: 1 samples

    - Option 2: 3 samples

In this contribution we state our preference on the options.

# Discussion

Reference SNR point for side condition is SNR with zero timing error

For reference SNR point for SNR with zero timing error, our preference is Option 2, i.e., SNR -1 dB, but we can also consider a somewhat higher SNR, e.g. SNR 0dB.

The number of sample for measurement accuracy

Our earlier simulation results [2] have indicated a large difference in measurement accuracy between single shot measurements (no time domain filtering) and measurements with time domain filtering. There is a significant improvement in measurement accuracy and reduction in variability when time-domain filtering is employed. Therefore it is our preference that measurement accuracy requirements shall be based on time-domain filtering, i.e., Option 2.

# Summary and Conclusion

In this contribution we have provided our preference on the options to choose for the remaining open issues in CLI RRM.

Our preference is:

  - Reference SNR point for side condition is SNR with zero timing error

    - Option 2: SNR -1dB 			(0dB would be acceptable too)

  - The number of sample for measurement accuracy

    - Option 2: 3 samples

# References

- R4-1912667	“Way Forward on CLI remaining RRM open issues”, LG Electronics

- R4-1907223 “Simulation results for SRS-RSRP measurements”, Ericsson