3GPP TSG-RAN4  #93								R4-1915088

Reno, USA, 18 - 22 November 2019

Source:	Qualcomm Incorporated

Title:	Consideration on environmental conditions for NR FR1 MIMO OTA

Agenda item: 	11.2.3.1

Document for:	Approval

# 1	Introduction

In RAN4#92bis meeting, the enviromental conditions for NR FR1 MIMO OTA were discussed in [1-4]. From the apporved  WF in [5], the envirometntal conditions will be decided between noise-limited and interference-limited in this meeting with the further analysis results or measurement results.

In this paper, we further provide our further considerations on test conditions for NR FR1 MIMO OTA testing.

# 2	Dicussion

In LTE MIMO OTA discussion, the tests aimed to find which MIMO testing method between 3GPP and CTIA can be used by the industry to identify MIMO OTA performance differences between devices was performed by operators. In that test, 10 commercial smartphones from 2016 and 2 commercial smartphones from 2013 have been selected. The study is to test MIMO OTA performance with all 10 devices using 3GPP (UMi) and CTIA (UMa) methods in anechoic chamber, the detail of test plan can be found in LS from GSMA to CTIA and the test plan below has been used [5]:

- Test MIMO OTA with all 10 devices using 3GPP (UMi) and CTIA (UMa) methods in DMP mode in Free Space by using anechoic chamber test environment

- Test outage points: 95%,75%,70%,65%,45%

- Test MIMO OTA by swapping the channel models i.e. 3GPP with UMa and CTIA with UMi

- Determine time duration of testing for each method per band

- Execute the tests in middle channel of LTE bands FDD 7 and FDD 20

From the test results, it can be observed that the CTIA method was not possible to distinguish between good and bad performing devices due to low variance of the results. The spread among the tested smartphones is within 2dB and the deviations were within measurement uncertainty. While with the 3GPP method we saw at least 10 dB difference between devices. Therefore, 3GPP method is feasible to identify the difference of  MIMO OTA performance. In order to understand if the channel model has any impact on MIMO performance of each method used, channel models, i.e. CTIA with UMi and 3GPP with UMa were swapped for further evaluation. The test results show that changing of the channel model is only equivalent to adding an offset.

However, [1] shows the performance utilizing the 3GPP methodology with TM2 are similar for two UEs while CTIA methodology shows that the device orientation in the channel can produce significant performance differences. This is opposite with the results in [6]. But we should note that the test results in [1] are based only two terminals on the lower frequency band 5. And the test results are based on 12 terminals including higher band 7 and lower band 20. Therefore, we have the following observation:

Observation 1: Interference-limited condition is not possible to distinguish between good and bad performing devices due to low variance of the results. While noise-limited method is feasible to distinguish the differences between devices and bands.

Furthermore, with interference-limited test condition, antenna efficiency will not directly impact the required SIR to reach the particular MIMO throughput since the signal and interference power are set tens of dB above the UE noise floor. Therefore, the controlled SIR at UE antenna side will be the same as SIR at the baseband. From the results in [6], we can figure out that interference-limited test condition is not sensitive to the antenna correlation since a UE with highly correlated received antenna will not be able to demodulate the rank 2 MIMO signal. Then, only MIMO performance difference, i.e. 2dB difference in the test results, can be verified under interference-limited test condition while it has been verified in the demodulation test cases with conducted mode.

Observation 2: Antenna imbalance will not directly impact the required SIR to reach the particular MIMO throughput in interference-limited test condition. Interference-limited test condition is not sensitive to the antenna correlation since a UE with highly correlated received antenna will not be able to demodulate the rank 2 MIMO signal.

Observation 3: MIMO performance can be verified under interference-limited test condition while  it has been tested in the demodulation test cases with conducted mode.

With above observations, it looks like noise-limited condition is a good choice and selecting this option can test the antenna efficiency, antenna imbalance, and MIMO performance for FR1 MIMO OTA.

Proposal 1: The option of noise-limited condition is preferred as the test condition for FR1 NR MIMO OTA.

# 3	Conclusion

In this paper, we provide the views on the three options of enviromental conditions for NR FR1 MIMO OTA testing. The following observations and proposal are given:

Observation 1: Interference-limited condition is not possible to distinguish between good and bad performing devices due to low variance of the results. While noise-limited method is feasible to distinguish the differences between devices and bands.

Observation 2: Antenna imbalance will not directly impact the required SIR to reach the particular MIMO throughput in interference-limited test condition. Interference-limited test condition is not sensitive to the antenna correlation since a UE with highly correlated received antenna will not be able to demodulate the rank 2 MIMO signal.

Observation 3: MIMO performance can be verified under interference-limited test condition while  it has been tested in the demodulation test cases with conducted mode.

Proposal 1: The option of noise-limited condition is preferred as the test condition for FR1 NR MIMO OTA.

# 4	Reference

- R4-1912033, “Evaluation of Low Band Performance using MARSS (SIR-Based Metric) versus TRMS”, PCTEST Engineering Lab, 3GPP  RAN4 #92bis meeting, Oct, 2019.

- R4-1911515, “Further views on the environmental conditions for FR1 MIMO OTA”, Apple Inc., 3GPP  RAN4 #92bis meeting, Oct, 2019.

- R4-1911682, “Environmental conditions for NR FR1 MIMO OTA”, Qualcomm Incorporated, 3GPP  RAN4 #92bis meeting, Oct, 2019.

- R4-1911717, “ Environmental condition for FR1 MIMO OTA”, DOCOMO, 3GPP  RAN4 #92 bis meeting, Oct, 2019.

- R4-1911637	, “WF on NR MIMO OTA”, CAICT, Samsung, Spirent, CMCC, SGS Wireless, vivo, 3GPP RAN4 #92bis meeting, Oct, 2019.

- MOSG171103, “LS from GSMA TSG: MIMO OTA Testing from Orange and Vodafone”, Oct. 2017.