3GPP TSG-RAN4  #93								R4-1915091

Reno, USA, 18 - 22 November 2019

Source:	Qualcomm Incorporated

Title:	Adding 64QAM RMC for NR FR2 MIMO OTA

Agenda item: 	11.2.3.2

Document for:	Approval

# 1	Introduction

In RAN4#92bis meeting, the RMC for NR MIMO OTA was discussed in [1-2]. The corresponding TP for RMC was agreed in [3] which specifies the R.PDSCH.5-2.2 TDD configurations considering the testable SNR for FR2 MIMO OTA. But during the discussion, we realized that the testing will be based on noise-limited test condition which will lead to higher maximum testable SNR. Therefore, the following action related with FR2 RMC was agreed in [4]:

- Further check on introducing 64QAM RMC in FR2 MIMO OTA based on the testability study

In this paper, we provide the views on the testability of 64QAM RMC in FR2 MIMO OTA testing.

# 2	Discussion

In FR2 demodulation methodology, the maximum testable SNR was derived in the attached calculator in TR38810. To achieve 1dB difference between Reference point SNR and Baseband SNR, no is sent with wanted signal from the test equipment.

The Noc values are based on REFSENS for the operating band and on the UE Power class and taking a baseline of UE Power Class 3 in band n260.

Noc = RESFENSPC3, n260, 50MHz -10log10(SCSREFSENS x PRBREFSENS x 12) - SNRREFSENS + ∆thermal

where:

-	REFSENSPC3, n260, 50MHz is the REFSENS value in dBm specified for Power Class 3 UE in band n260 for 50MHz channel bandwidth in TS 38.101-2 Table 7.3.2.3-1, [dBm/Hz]

-	SCSREFSENS is a subcarrier spacing associated with NRB for 50MHz in TS 38.101-2 Table 5.3.2-1, chosen as 120kHz.

-	PRBREFSENS is NRB associated with subcarrier spacing 120kHz for 50MHz in TS 38.101-2 Table 5.3.2-1 and is 32.

-	12 is the number of subcarriers in a PRB

-	SNRREFSENS = -1 dB is the SNR used for simulation of REFSENS

-	∆thermal is the amount of dB that the wanted noise is set above UE thermal noise, giving a rise in total noise of ∆BB. ∆thermal = 6dB, giving a rise in total noise of 1dB.

With that, the SNR upper bound values for Direct far field (DFF) which has the worst SNR range is given in Table 1. For 100MHz channel bandwidth UE even with the multi-band, the maximum SNR is 17.4dB

Table 1: SNR upper bound value for DFF with Noc

|  | Channel Bandwidth | Maximum SNR |
| --- | --- | --- |
| Multi-band UE (Note) | 100 MHz | [17.4 dB] |
| Note: For ∑MBp from TS 38.101-2Table 6.2.1.3-4 allow up to 2 dB |  |  |

While in FR2 MIMO OTA testing, the noise-limited test condition was agreed to be used which means only wanted signal will be transmitted from test equipment. Though the measurement distance is not defined yet for FR2 MIMO OTA, we can use the same assumption to calculation the SNR range based on Rel-15 testability SI. Then we re-calculate the maximum SNR without Noc level, and the following maximum SNR can be reached for 100MHz case (The calculator is attached).

Table 2: SNR upper bound value for DFF without Noc

|  | Channel Bandwidth | Maximum SNR |
| --- | --- | --- |
| Multi-band UE (Note) | 100 MHz | [24.7dB] |
| Note: For ∑MBp from TS 38.101-2Table 6.2.1.3-4 allow up to 2 dB |  |  |

Observation 1: The SNR upper bound value for DFF without Noc is 24.7dB for 100MHz channel bandwidth.

From the Table 3 which is defined in Table 7.2.2.2.1-4 of TS38.101-4, the reference value for baseband is 18.6dB for 70% maximum throughput.

Table 3: Minimum performance for Rank 2 (FRC)

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNRBB (dB) |
| 2-1 | R.PDSCH.5-4.1 TDD | 100 / 120 | QPSK, 0.30 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 70 | 4.1 |
| 2-2 | R.PDSCH.5-2.2 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 70 | 14.4 |
| 2-3 | R.PDSCH.5-5.2 TDD | 50 / 120 | 16QAM,0.48 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 70 | 14.0 |
| 2-4 | R.PDSCH.5-2.3 TDD | 200 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 70 | 14.2 |
| 2-5 | R.PDSCH.4-1.1 TDD | 50 / 60 | 16QAM, 0.48 | FR2.60-1 | TDLA30-75 | 2x2 ULA Low | 70 | 14.3 |
| 2-6 | R.PDSCH.5-6.1 TDD | 100 / 120 | 64QAM, 0.43 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 70 | 18.6 |

However, the fraction for maximum throughput has not been decided for NR MIMO OTA. But we can assume the same fraction as LTE MIMO OTA can be reused for FR2 MIMO OTA requirements which are 70% and 95%. Compared with 70% maximum throughput, to achieve 95% maximum throughput will need higher SNR. In TS37.144, E-UTRA TDD TRMS was defined shown in Table 4. We can observe that the requirements difference between 70% and 95% DL throughput with the reference measurement channel is 2dB.

Observation 2: For E-UTRA TDD, The TRMS minimum requirements difference between 70% and 95% DL throughput with the reference measurement channel is 2dB.

Therefore, even for 95% DL throughput, 64QAM RMC for FR2 is feasible with at least 4dB margin (24.7-18.6-2=4.1dB) based on the Rel-15 testability study. Moreover, we should note that the Rel-16 testability enhancement SI is ongoing in which the study on enhancing testable SNR is in the scope. Therefore, we can expect maximum SNR enhancement in the OTA chamber. In this case, we would have more margin.

Observation 3: For 95% DL throughput, 64QAM RMC for FR2 is feasible with at least 4dB margin based on the Rel-15 testability study. Moreover, we can expect the maximum SNR enhancement in the OTA chamber with Rel-16 testability enhancement SI output.

Table 4: Handheld UE TRMS minimum requirements for E-UTRA TDD roaming bands in free space and the primary mechanical mode

|  |  | Test 1 |  |
| --- | --- | --- | --- |
| Channel Model |  | SCME urban micro-cell |  |
| Operating band | Unit | <REFÎor> |  |
|  |  | Average, 70 | Average, 95 |
| 38 | dBm/15 kHz | -95.5 | -93.5 |
| 39 | dBm/15 kHz | TBD | TBD |
| 40 | dBm/15 kHz | TBD | TBD |
| 41 | dBm/15 kHz | -95.5 | -93.5 |
| 42 | dBm/15 kHz | TBD | TBD |
| 46 (1,2) | dBm/15 kHz | TBD | TBD |
| NOTE 1: This band is an unlicensed band restricted to licensed-assisted operation using Frame Structure Type 3 NOTE 2: In this version of the specification, restricted to E-UTRA DL operation when carrier aggregation is configured. NOTE 3: Applicability for devices supporting 4-receiver architectures is not confirmed. |  |  |  |

With above considerations, we have the following proposal:

Proposal: Adding below 64QAM test parameters to TR 38.827 for NR FR2 MIMO OTA.

Table 5: Test Parameters for FR2 TDD 2x2 (64QAM)

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Reference channel |  |  | R.PDSCH.5-6.1 TDD (Note 1) |
| Bandwidth |  | MHz | 100 |
| SCS |  | kHz | 120 |
| Modulation DL |  |  | 64QAM |
| Modulation UL |  |  | QPSK |
| Active DL BWP index |  |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Table 8.2-6 |
|  | CSI-RS offset | Slots | Table 8.2-6 |
| PDCCH configuration | Number of PDCCH candidates and aggregation levels |  | 1/AL8 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | Specific to each Reference channel as defined in A.3.2.2 of TS 38.101-4 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 1 |
| TDD UL-DL pattern |  |  | FR2.120-1 (Note2) |
| Note 1: “R.PDSCH.5-2.2 TDD” is defined in Table A.3.2.2.5-6 of TS 38.101-4 Note 2: “FR2.120-1” is defined in Annex A.1.3 of TS 38.101-4 |  |  |  |

# 3	Conclusion

In this paper, we provide the views on the 64QAM RMC for NR FR2 MIMO OTA testing. The following observations and proposal are given:

Observation 1: The SNR upper bound value for DFF without Noc is 24.7dB for 100MHz channel bandwidth.

Observation 2: For E-UTRA TDD, The TRMS minimum requirements difference between 70% and 95% DL throughput with the reference measurement channel is 2dB.

Observation 3: For 95% DL throughput, 64QAM RMC for FR2 is feasible with at least 4dB margin based on the Rel-15 testability study. Moreover, we can expect the maximum SNR enhancement in the OTA chamber with Rel-16 testability enhancement SI output.

Proposal: Adding below 64QAM test parameters to TR 38.827 for NR FR2 MIMO OTA.

Table 5: Test Parameters for FR2 TDD 2x2 (64QAM)

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Reference channel |  |  | R.PDSCH.5-6.1 TDD (Note 1) |
| Bandwidth |  | MHz | 100 |
| SCS |  | kHz | 120 |
| Modulation DL |  |  | 64QAM |
| Modulation UL |  |  | QPSK |
| Active DL BWP index |  |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Table 8.2-6 |
|  | CSI-RS offset | Slots | Table 8.2-6 |
| PDCCH configuration | Number of PDCCH candidates and aggregation levels |  | 1/AL8 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | Specific to each Reference channel as defined in A.3.2.2 of TS 38.101-4 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 1 |
| TDD UL-DL pattern |  |  | FR2.120-1 (Note2) |
| Note 1: “R.PDSCH.5-2.2 TDD” is defined in Table A.3.2.2.5-6 of TS 38.101-4 Note 2: “FR2.120-1” is defined in Annex A.1.3 of TS 38.101-4 |  |  |  |

# 4	Reference

- R4-1911410, “RMC for NR MIMO OTA”, Samsung, CAICT, 3GPP  RAN4 #92bis meeting, Oct, 2019.

- R4-1912098	, “Reference Measurement Channels for NR FR1 MIMO OTA”, Keysight Technologies UK Ltd, 3GPP  RAN4 #92bis meeting, Oct, 2019.

- R4-1912900, “TP to TR on RMC for MIMO OTA”, Samsung, CAICT, 3GPP  RAN4 #92bis meeting, Oct, 2019.

- R4-1911637	, “WF on NR MIMO OTA”, CAICT, Samsung, Spirent, CMCC, SGS Wireless, vivo, 3GPP RAN4 #92bis meeting, Oct, 2019.