3GPP TSG-RAN WG4 Meeting #93	R4-1915096

# Reno, Nevada, USA, 18 - 22 November, 2019

Title: 	Discussion and simulation results for LTE HST BS demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.13.4.1

Document for:	Discussion

Background

During RAN4#92bis meeting, WF [1] for LTE Rel-16 BS demodulation was approved, the following new scenarios and new test cases are introduced:

- Introduce the following HST scenarios Parameter; Value. Open space; Tunnel for multi-antennas. Bi-directional; Bi-directional. Ds; 1000 m; 300 m. Dmin; 50 m; 2 m. v; 500 km/h; 500 km/h. fd; 1944Hz If 70%ile of the max. throughput is not achievable with 1944Hz: Case 1: fallback to 1750Hz, or Case 2: accept %ile of max. throughput lower than 70%ile with 1944Hz. - Introduce the following new test cases Parameters; Value. Scenario 1 (Open space); Scenario 3 (Tunnel). CBW; 1.4/3/5/10/15/20 MHz; 1.4/3/5/10/15/20 MHz. CP; Normal; Normal. FRC; A.3-2 to A.3-7 (QPSK 1/3); A.3-2 to A.3-7 (QPSK 1/3). Antenna configuration; 1Tx2Rx; 1Tx1Rx. Reference receiver; MRC; MRC. Noise estimation; Practical; Practical. Time and frequency track; Practical; Practical. Test metric; [70%ile] of the max. throughput 30%ile of the max. throughput; [70%ile] of the max. throughput 30%ile of the max. throughput. NOTE: The configuration of PUCCH (format 2) is optional.

In this contribution, we share our views about the BS demodulation requirements for LTE Rel-16 HST.

Discussion

Maximum Doppler shift:

As per the approved WF [1], the following options for maximum Doppler shift are listed:

- 1944Hz - If 70%ile of the max. throughput is not achievable with 1944Hz: - Option 1: fallback to 1750Hz, or - Option 2: accept %ile of max. throughput lower than 70%ile with 1944Hz

For the different velocity at Band 1, by assuming the UL Doppler shift is twice of the DL Doppler shift, we derive the corresponding maximum Doppler shift as shown below in Table 2-1:

Table 2-1: Maximum Doppler frequency for high speed train conditions

| Velocity | Band | DL: Max Doppler shift (Hz) | UL: Max Doppler shift (Hz) |
| --- | --- | --- | --- |
| 450km/h | Band 1 (2.1GHz) | 875 | 1750 |
| 500km/h | Band 1 (2.1GHz) | 972 | 1944 |

Observation 1: Maximum Doppler shift 1944Hz is derived to support velocity 500km/h for Band 1.

Simulations

We provide our initial alignment simulations as per the simulation assumptions in [1] as following:

Table 3.1-1: Ideal Simulation results for LTE PUSCH under HST single-tap

| CBW [MHz] | CP | FRC | Antenna configuration | Propagation conditions | Fraction of maximum throughput | SNR[dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 1944Hz | 1750Hz |
| 1.4 | Normal | A3-2 | 1x1 | HST Scenario 3 | 30% | -4.97 | -5.16 |
|  |  |  |  |  | 70% | -0.85 | -0.85 |
|  |  |  | 1x2 | HST Scenario 1 Low | 30% | -7.21 | -7.11 |
|  |  |  |  |  | 70% | -3.86 | -3.86 |
| 3 | Normal | A3-3 | 1x1 | HST Scenario 3 | 30% | -5.11 | -5.07 |
|  |  |  |  |  | 70% | -0.79 | -0.80 |
|  |  |  | 1x2 | HST Scenario 1 Low | 30% | -7.80 | -7.80 |
|  |  |  |  |  | 70% | -3.85 | -3.85 |
| 5 | Normal | A3-4 | 1x1 | HST Scenario 3 | 30% | -6.16 | -6.22 |
|  |  |  |  |  | 70% | -1.57 | -1.57 |
|  |  |  | 1x2 | HST Scenario 1 Low | 30% | -9.09 | -9.16 |
|  |  |  |  |  | 70% | -4.61 | -4.61 |
| 10 | Normal | A3-5 | 1x1 | HST Scenario 3 | 30% | -5.29 | -5.30 |
|  |  |  |  |  | 70% | -0.89 | -0.87 |
|  |  |  | 1x2 | HST Scenario 1 Low | 30% | -8.32 | -8.24 |
|  |  |  |  |  | 70% | -3.97 | -3.98 |
| 15 | Normal | A3-6 | 1x1 | HST Scenario 3 | 30% | -6.24 | -6.24 |
|  |  |  |  |  | 70% | -1.55 | -1.56 |
|  |  |  | 1x2 | HST Scenario 1 Low | 30% | -9.20 | -9.26 |
|  |  |  |  |  | 70% | -4.60 | -4.60 |
| 20 | Normal | A3-7 | 1x1 | HST Scenario 3 | 30% | -5.27 | -5.27 |
|  |  |  |  |  | 70% | -0.67 | -0.67 |
|  |  |  | 1x2 | HST Scenario 1 Low | 30% | -8.40 | -8.37 |
|  |  |  |  |  | 70% | -3.74 | -3.74 |

From the simulation results we can get the following observation and proposal:

Observation 2: There is negligible difference for maximum Doppler shift 1944Hz and 1750Hz, and both of them can reach 70% maximum throughput.

Proposal: It is proposed to adopt the maximum Doppler shift 1944Hz for LTE HST BS demodulation requirements.

Proposals

In this contribution, we analyzed and shared our views on those open issues, provides our simulation results for alignment and discussions, our observations and proposals are:

Observation 1: Maximum Doppler shift 1944Hz is derived to support velocity 500km/h for Band 1.

Observation 2: There is negligible difference for maximum Doppler shift 1944Hz and 1750Hz.

Proposal: It is proposed to adopt the maximum Doppler shift 1944Hz for LTE HST BS demodulation requirements.

Reference

- R4-1912777, Way forward on LTE HST BS demodulation requirements, RAN4#92Bis, NTT DOCOMO, INC.