3GPP TSG-RAN4 WG4 Meeting # 93	R4-1915097

# Reno, Nevada, USA, 18 - 22 November, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 4885 | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for FeNB-IoT NPRACH TDD performance requirements for 36.104(Re-15) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Re-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The FeNB-IOT NPRACH TDD performance requirements is not clear. -The probability of detection for FeNB-IOT NPRACH TDD shall be equal to or exceed 99% for the SNR levels listed in table 8.5.3.2.1-2 is not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | According to the simulation results of each company,we update the probability of detection for FeNB-IOT NPRACH TDD shall be equal to or exceed 99% for the SNR levels listed in table 8.5.3.2.1-2. -Add the STD from ideal results to companies’ averaged values with impairments as the final FeNB-IoT NPRACH TDD format0 and format1 performance requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | We can not measure the performance of FeNB-IOT NPRACH TDD correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 8.5.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

8.5.3.2	NPRACH detection requirements

The probability of detection is the conditional probability of correct detection of the preamble when the signal is present. There are several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. A timing estimation error occurs if the estimation error of the timing of the strongest path is larger than 3.646us. The strongest path for the timing estimation error refers to the strongest path in the power delay profile.

The requirements for TDD are optional and only valid for base stations supporting TDD.

Table 8.5.3.2-1: Test preambles for NPRACH

| Parameter | Value |
| --- | --- |
| Narrowband physical layer cell identity | 0 |
| Initial subcarrier index | 0 |
| Uplink-downlink configuration for TDD | 1 |
| Special subframe configuration for TDD | 7 |

8.5.3.2.1	Minimum requirements

The probability of detection shall be equal to or exceed 99% for the SNR levels listed in table 8.5.3.2.1-1.

Table 8.5.3.2.1-1: NPRACH missed detection requirements for FDD

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR[dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Preamble format 0 | Preamble format 1 | Preamble format 2 |
| 1 | 2 | 8 | AWGN | 0 | -2.1 | -2.1 | -1.0 |
|  |  |  | EPA1 Low | 200 Hz | 6.1 | 6.1 | 9.0 |
|  |  | 32 | AWGN | 0 | -6.8 | -6.8 | -5.1 |
|  |  |  | EPA1 Low | 200 Hz | 0.5 | 0.5 | 2.2 |

Table 8.5.3.2.1-2: NPRACH missed detection requirements for TDD

| Number of TX antennas |  | Number of RX antennas |  | Repetition number |  | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR[dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Preamble format 0 | Preamble format 1 | Preamble format 0-a | Preamble format 1-a |
| 1 | 2 | 8 | AWGN | 0 | 6.1 | 3.3 | 4.8 | 4.4 |
|  |  |  | EPA1 Low | 200 Hz | 14.9 | 10.9 | 14.0 | 9.9 |
|  |  | 32 | AWGN | 0 | 1.9 | -1.1 | 0.5 | -2.5 |
|  |  |  | EPA1 Low | 200 Hz | 9.7 | 4.2 | 10.9 | 3.2 |

<End of change>