3GPP TSG-RAN WG4 Meeting #93	R4-1915122

# Reno, Nevada, USA, 18 - 22 November, 2019

Title: 	Discussion and simulation results on NR BS HST performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	9.17.2.3.1

Document for:	Discussion

Background

During RAN4#92bis meeting, way forward [1] for NR Rel-16 BS demodulation was approved. In this contribution, we share our views about the BS demodulation requirements for NR Rel-16 HST.

Discussion

Maximum Doppler shift

As per WF [1], the options for Maximum Doppler shift are shown below:

- Maximum Doppler shift - Single tap HST 350km/h - 15kHz SCS: 1340Hz - 30kHz SCS: 2334Hz - Single tap HST 500km/h - 15kHz SCS : - Option 1: 1944Hz - Option 2: 1750Hz - 30kHz SCS: - Option 1: 3334Hz - Option 2: 3000Hz

Doppler shift tracking capability depends on the distance between two closest column DMRS. The following Table 2-1 shows the maximum Doppler frequency tracking capability for different DMRS and SCS.

Table 2-1: NR PDSCH DMRS Doppler frequency tracking

| DMRS configuration | SCS (kHz) | The smallest interval of DMRS (ms) | Max Doppler frequency tracking (Hz) |
| --- | --- | --- | --- |
| 1+1+1 (2,7,11) | 15 | 0.285 | 1752 |
|  | 30 | 0.143 | 3504 |
| 1+1+1 (3,7,11) | 15 | 0.285 | 1752 |
|  | 30 | 0.143 | 3504 |

The following table shows the theoretical maximum Doppler shift value assuming UL is 2 times DL.

Table 2-2: Maximum Doppler frequency for high speed train conditions

| Velocity | Carrier frequency | DL: Max Doppler shift (Hz) | UL: Max Doppler shift (Hz) |
| --- | --- | --- | --- |
| 350km/h | Band n1 (2.1GHz) | 681 | 1361 |
|  | Band n77 (3.6GHz) | 1167 | 2333 |
| 450km/h | Band n1 (2.1GHz) | 875 | 1750 |
|  | Band n77 (3.6GHz) | 1500 | 3000 |
| 500km/h | Band n1 (2.1GHz) | 972 | 1944 |
|  | Band n77 (3.6GHz) | 1667 | 3333 |
|  | Band n2 (1.89GHz) | 875 | 1750 |

Maximum Doppler shift should not greater than Doppler shift tracking capability. Comparing above 2 tables, we can get the following observation and proposal:

Observation1: In theory, 500km/h with Band n1 (2.1GHz) is not feasible.

Proposal 1: To support 500km/h, carrier frequency can reduce to 1.89GHz, or UE velocity can reduce to 450km/h to support Band1 (2.1GHz).

Antenna configuration

- Antenna configuration - For tunnel - Option 1: 1x1 - Option 2: 1x2 (baseline for simulation alignment) - For open space - Option 1: 1x2 (baseline for simulation alignment) - Option 2: 1x8

For antenna configuration, more practical situation could be considered. It is more reasonable to set 1x2 for tunnel scenario, larger number of receiver antenna for open space scenario.

Proposal 2: For tunnel scenario, adopt 1x2 antenna configuration. For open space scenario, adopt 1x8 antenna configuration.

MCS

- MCS - Option 1: MCS#2 - Option 2: MCS#16 - Option 3: MCS#2 and MCS#16 MCS#2 and MCS#16 for simulation alignment

As per simulation results in Section 3, we can get the following observation and proposal:

Observation 2: Both MCS 2 and 16 are feasible. MCS 16 has better performance considering balance between throughput and SNR.

Proposal 3: MCS#16 is enough for HST performance requirements definition, it is proposed to adopt MCS#16.

l0 for PUSCH mapping type A

- l0 for PUSCH mapping type A - Provide the simulation results for 350km/h and 500km/h and evaluate the following configurations and make decision: - Option 1: l0 = 3 - Option 2: l0 = 2 - Same value for both 350km/h and 500km/h can be considered

As per simulation results in Section 3, we can get the following observation and proposal:

Observation 3: There is negligible difference between l0=2 and 3 from simulation results. However, it is more preferable to set l0=3 since that using l0=3 can demodulation faster than l0=2.

Proposal 4: It is proposed to set l0=3.

Simulation results

In this section, we give simulation results for alignment as per simulation assumption in WF [1].

Figure 2-1: Ideal Simulation results for NR BS HST

| Case Number | l0 position | Antenna configuration | CHBW/SCS | maximum Doppler shift(Hz) | Propagation condition | MCS | SNR@70% Max TP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 1Tx2Rx | 10MHz/15kHz | 1750 | Tunnel | 2 | -4.59 |
| 2 |  |  |  |  |  | 16 | 5.41 |
| 3 |  |  |  |  | Open space | 2 | -4.60 |
| 4 |  |  |  |  |  | 16 | 5.40 |
| 5 |  |  | 40MHz/30kHz | 3000 | Tunnel | 2 | -4.59 |
| 6 |  |  |  |  |  | 16 | 5.42 |
| 7 |  |  |  |  | Open space | 2 | -4.60 |
| 8 |  |  |  |  |  | 16 | 5.40 |
| 9 |  |  |  | 3334 | Tunnel | 2 | -4.58 |
| 10 |  |  |  |  |  | 16 | 5.42 |
| 11 |  |  |  |  | Open space | 2 | -4.60 |
| 12 |  |  |  |  |  | 16 | 5.40 |
| 13 |  | 1Tx8Rx | 10MHz/15kHz | 1750 | Open space | 16 | 0.40 |
| 14 |  |  | 40MHz/30kHz | 3000 |  | 16 | 0.40 |
| 15 |  |  |  | 3334 |  | 16 | 0.40 |
| 16 | 3 | 1Tx2Rx | 10MHz/15kHz | 1750 | Tunnel | 2 | -4.59 |
| 17 |  |  |  |  |  | 16 | 5.41 |
| 18 |  |  |  |  | Open space | 2 | -4.60 |
| 19 |  |  |  |  |  | 16 | 5.40 |
| 20 |  |  | 40MHz/30kHz | 3000 | Tunnel | 2 | -4.59 |
| 21 |  |  |  |  |  | 16 | 5.42 |
| 22 |  |  |  |  | Open space | 2 | -4.60 |
| 23 |  |  |  |  |  | 16 | 5.40 |
| 24 |  |  |  | 3334 | Tunnel | 2 | -4.58 |
| 25 |  |  |  |  |  | 16 | 5.42 |
| 26 |  |  |  |  | Open space | 2 | -4.60 |
| 27 |  |  |  |  |  | 16 | 5.40 |
| 28 |  | 1Tx8Rx | 10MHz/15kHz | 1750 | Open space | 16 | 0.40 |
| 29 |  |  | 40MHz/30kHz | 3000 |  | 16 | 0.40 |
| 30 |  |  |  | 3334 |  | 16 | 0.40 |

Figure 3-1: Ideal Simulation results for NR BS HST

Proposals

In this contribution, we analyses the BS performance requirement for NR HST, and our conclusions and proposals are:

Observation1: In theory, 500km/h with Band n1 (2.1GHz) is not feasible.

Observation 2: Both MCS 2 and 16 are feasible. MCS 16 has better performance considering balance between throughput and SNR.

Observation 3: There is negligible difference between l0=2 and 3 from simulation results. However, it is more preferable to set l0=3 since that using l0=3 can demodulation faster than l0=2.

Proposal 1: To support 500km/h, carrier frequency can reduce to 1.89GHz, or UE velocity can reduce to 450km/h to support Band1 (2.1GHz).

Proposal 2: For tunnel scenario, adopt 1x2 antenna configuration. For open space scenario, adopt 1x8 antenna configuration.

Proposal 3: MCS#16 is enough for HST performance requirements definition, it is proposed to adopt MCS#16.

Proposal 4: It is proposed to set l0=3.

Reference

- R4-1912808, WF on UE demodulation for NR HST, RAN4#92Bis, CMCC