3GPP TSG-RAN WG4 Meeting#93                	R4-1915149

Reno, Nevada, USA, November 18th – 22nd 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Measurement accuracy simulation results for Rel-16 NR high speed train

Agenda item:	9.17.1.2

Document for:	Discussion

Introduction

During the RAN4#92bis meeting, link level simulation assumptions on SS-RSRP/-RSRQ/-SINR measurement accuracy for NR HST [1] were approved.

Based on the link level assumptions, this document presents simulation results for the following:

- Delta SS-RSRP

- Delta SS-RSRQ

- Delta SS-SINR

Discussion

Simulation parameters

In our simulations, only 1 SSB beam was considered. Furthermore, an interfering cell was not simulated. Thus, SS-RSRQ, SS-RSRQ and SS-SINR estimations are affected by AWGN only.

The complete simulation parameters and assumptions are given in Tables 1 and 2.

Table 1: Link level simulation assumptions – general parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 3.6 GHz |
| DRX | No |
| BS transmit antennas for SS blocks | 1 tx or single layer transmissions |
| UE receive antennas | 2 rx |
| Data and control channel subcarrier spacing | The same as SS block subcarrier spacing |
| Measurement period (in number of measurement samples) | 1, 3, 5 |
| - Subcarrier spacing | 15 kHz; 30 kHz |
| - Number of SS blocks per SS burst set, K | 1 |
| - SS burst periodicity | 5 ms, 10 ms, 20 ms |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell |
| --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 |
| NR-PSS and NR-SSS sequences | - | NR-PSS: m-sequences and NR-SSS: Gold sequences |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 |
| RB Utilization | % | 100 |
| Data Modulation | - | QPSK |
| Slot length | - | 14 symbols |
| CP Length | - | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| Es/IoT | dB | SNR: -6dB; FFS: -8dB, -7dB) |
| Propagation conditions | - | AWGN |

Simulation results

The following section includes simulation results for:

- Delta SS-RSRP = (estimated SS-RSRP – ideal SS-RSRP) 	[dB]

- Delta SS-RSRQ = (estimated SS-RSRQ – ideal SS-RSRQ) 	[dB]

- Delta SS-SNR = (estimated SS-SINR – ideal SS-SNR) 	[dB]

Delta SS-RSRP

The following figures illustrate the CDF curves for delta SS-RSRP for different SNR, SCS values and SS burst periods.

| SNR | Samples | 5% | 95% | Max(5%, 95%) |
| --- | --- | --- | --- | --- |
| -6 | 1 | -1.1600 | 2.3700 | 2.3700 |
|  | 3 | -0.6900 | 1.5400 | 1.5400 |
|  | 5 | -0.5600 | 1.1600 | 1.1600 |
| -7 | 1 | -1.2600 | 2.7800 | 2.7800 |
|  | 3 | -0.7900 | 1.7800 | 1.7800 |
|  | 5 | -0.6000 | 1.4100 | 1.4100 |
| -8 | 1 | -1.4800 | 3.1200 | 3.1200 |
|  | 3 | -0.9500 | 2.0500 | 2.0500 |
|  | 5 | -0.6800 | 1.6600 | 1.6600 |

Figure 1 Delta SS-RSRP for SS burst period = 5 ms, SCS = 15 kHz.

Figure 2 Delta SS-RSRP for SS burst period = 20 ms, SCS = 15 kHz.

Figure 3 Delta SS-RSRP for SS burst period = 5 ms, SCS = 30 kHz.

Figure 4 Delta SS-RSRP for SS burst period = 20 ms, SCS = 30 kHz.

From the results we can observe the following:

- SS-RSRP accuracy is affected by the measurement period, i.e., the number of samples. As expected, 5 measurement samples achieve the highest accuracy followed by 3 and 1 measurement samples.

- SS-RSRP accuracy is not affected by different SCS values and SS burst periodicity. The simulation results show no significant difference in the SS-RSRP accuracy between SS burst periods 5 ms and 20 ms.

- SS-RSRP accuracy increases with higher SNR.

Delta SS-RSRQ

The following figures illustrate the CDF curves for delta SS-RSRQ for different SNR, SCS values and SS burst periods.

| SNR | Samples | 5% | 95% | Max(5%, 95%) |
| --- | --- | --- | --- | --- |
| -6 | 1 | -1.0200 | 2.1600 | 2.1600 |
|  | 3 | -0.6400 | 1.4100 | 1.4100 |
|  | 5 | -0.5600 | 1.0600 | 1.0600 |
| -7 | 1 | -1.1800 | 2.5700 | 2.5700 |
|  | 3 | -0.7500 | 1.6700 | 1.6700 |
|  | 5 | -0.5600 | 1.3300 | 1.3300 |
| -8 | 1 | -1.4400 | 2.9000 | 2.9000 |
|  | 3 | -0.8900 | 1.9300 | 1.9300 |
|  | 5 | -0.6300 | 1.5800 | 1.5800 |

Figure 5 Delta SS-RSRQ for SS burst period = 5 ms, SCS = 15 kHz.

Figure 6 Delta SS-RSRQ for SS burst period = 20 ms, SCS = 15 kHz.

Figure 7 Delta SS-RSRQ for SS burst period = 5 ms, SCS = 30 kHz.

Figure 8 Delta SS-RSRQ for SS burst period = 20 ms, SCS = 30 kHz.

From the results we can make similar observations as SS-RSRP, which are:

- SS-RSRQ accuracy is affected by the measurement period, i.e., the number of samples. As expected, 5 measurement samples achieve the highest accuracy followed by 3 and 1 measurement samples.

- SS-RSRQ accuracy is not affected by different SCS values and SS burst periodicity. The simulation results show no significant difference in the SS-RSRQ accuracy between SS burst periods 5 ms and 20 ms.

- SS-RSRQ accuracy increases with higher SNR.

Delta SS-SNR

The following figures illustrate the CDF curves for delta SS-SNR for different SNR, SCS values and SS burst periods.

| SNR | Samples | 5% | 95% | Max(5%, 95%) |
| --- | --- | --- | --- | --- |
| -6 | 1 | -1.3200 | 2.8800 | 2.8800 |
|  | 3 | -0.7900 | 1.8300 | 1.8300 |
|  | 5 | -0.6900 | 1.3800 | 1.3800 |
| -7 | 1 | -1.4100 | 3.3000 | 3.3000 |
|  | 3 | -0.9100 | 2.0800 | 2.0800 |
|  | 5 | -0.6800 | 1.6500 | 1.6500 |
| -8 | 1 | -1.6900 | 3.5600 | 3.5600 |
|  | 3 | -1.0500 | 2.3200 | 2.3200 |
|  | 5 | -0.7500 | 1.8900 | 1.8900 |

Figure 9 Delta SS-SNR for SS burst period = 5 ms, SCS = 15 kHz.

Figure 10 Delta SS-SNR for SS burst period = 20 ms, SCS = 15 kHz.

Figure 11 Delta SS-SNR for SS burst period = 5 ms, SCS = 30 kHz.

Figure 12 Delta SS-SNR for SS burst period = 20 ms, SCS = 30 kHz.

From the results, similar observations can be made as SS-RSRP and SS-RSRQ can be made.

Simulation conclusions

High SNR increases measurement accuracy (SS-RSRP/-RSRQ/-SINR) as expected.

Delta SS-RSRP/-RSRQ/-SINR are affected by the measurement period, i.e., the number of samples. As expected, 5 measurement samples achieve the highest accuracy. 1 measurement sample show accuracy degradation as compared with 3 and 5 samples.

Measurement accuracy (SS-RSRP/-RSRQ/-SINR) is not significantly affected by different SCS values and SS burst periodicity. The simulation results show no significant difference in the SS-RSRP accuracy between SS burst periods 5 ms and 20 ms.

Simulation results show that there is no significant difference between delta SS-RSRP, delta SS-RSRQ and delta SS-SINR.

Option 1 – no new measurement accuracy requirements for measurement samples ≥ 3 at SNR greater than -7 dB

Option 2 – new measurement accuracy requirements are needed for measurement samples = 1 at SNR ≤ -6 dB

Conclusions

The document has presented link level simulation results for SS-RSRP/-RSRQ/-SNR measurement accuracy in Rel-16 NR HST.

From the results, the following observations and proposals can be made:

- High SNR increases measurement accuracy (SS-RSRP/-RSRQ/-SINR) as expected.

Delta SS-RSRP/-RSRQ/-SINR are affected by the measurement period, i.e., the number of samples. As expected, 5 measurement samples achieve the highest accuracy. 1 measurement sample show accuracy degradation as compared with 3 and 5 samples.

Measurement accuracy (SS-RSRP/-RSRQ/-SINR) is not significantly affected by different SCS values and SS burst periodicity. The simulation results show no significant difference in the SS-RSRP accuracy between SS burst periods 5 ms and 20 ms.

Simulation results show that there is no significant difference between delta SS-RSRP, delta SS-RSRQ and delta SS-SINR.

- Option 1 – no new measurement accuracy requirements for measurement samples ≥ 3 at SNR greater than -7 dB

Option 2 – new measurement accuracy requirements are needed for measurement samples = 1 at SNR ≤ -6 dB

References

[1]   R4-1912842, Link level simulation assumption on measurement accuracy in NR HST, Huawei, HiSilicon

[2]