3GPP TSG-RAN WG4 Meeting #93	R4-1915227

# Reno, NV, USA,  18th – 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.171 | CR | 0008 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.171: Corrections to A-GNSS requirements with NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Editor’s note indicates that A-GPS/GNSS requirements with NR FR2, as well as the respective NR FR2 frequency are FFS/TBD. Since Rel-15 timeframe is ending, the requirements need to finalized or removed from Rel-15. However, there have been no contributions so far, identifying any issue with NR FR2, thus the same requirements and frequency offset can be applicable for both FR1 and FR2. - In Annex B.1.7 the NR FR1 frequency refers to TS 38.508-1 [20], clause 6.2.3, which implies testing in 3 different NR frequencies (Low/Mid/High), which for is redundant and does not bring any value for LBS perfornance testing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In sections 5 and 6 removed the Editor’s note on the restricted applicability, making the requirements applicable to both FR1 and FR2. - In Annex B.1.7 removed the statement on the undefined NR FR2 frequency and offset. As such frequencies from 38.508 apply. - In Annex B.1.7 changed the frequency reference from TS 38.508-1 [20], clause 6.2.3 to clauses 4.3.1.0A, 4.3.1.1 and 4.3.1.2 (incl. FR2). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - NR FR2 requirements will remain undefined in Rel-15. - NR testing frequencies will be unnecessarely redundant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5, 6, B.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.571 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

# 5	A-GNSS minimum performance requirements (UE supports A-GPS L1 C/A only)

## 5.0	Introduction

The minimum performance requirements specified in clause 5 apply for UEs that support A-GPS L1 C/A only. The requirements for UEs that support other or additional A-GNSSs are specified in clause 6.

The A-GNSS minimum performance requirements are defined by assuming that all relevant and valid assistance data is received by the UE in order to perform GPS L1 C/A measurements and/or position calculation. This clause does not include nor consider delays occurring in the various signalling interfaces of the network.

In the following subclauses the minimum performance requirements are based on availability of the assistance data information and messages defined in annexes D and E.

# < Unchanged sections omitted >

# 6	A-GNSS minimum performance requirements (UE supports other or additional GNSSs)

## 6.0	Introduction

The minimum performance requirements specified in clause 6 apply for UEs that support other A-GNSSs than GPS L1 C/A, or multiple A-GNSSs which may or may not include GPS L1 C/A. The requirements for UEs that support A-GPS L1 C/A only are specified in clause 5.

The A-GNSS minimum performance requirements are defined by assuming that all relevant and valid assistance data is received by the UE in order to perform GNSS measurements and/or position calculation. This clause does not include nor consider delays occurring in the various signalling interfaces of the network.

In the following subclauses the minimum performance requirements are based on availability of the assistance data information and messages defined in annexes D and E.

# < Unchanged sections omitted >

## B.1.7	E-UTRA or NR frequency and frequency error

In all test cases with E-UTRA frequency, the E-UTRA frequency used shall be the mid-range for the E-UTRA operating band. The E-UTRA frequency with respect to the GNSS carrier frequency shall be offset by +0.025 PPM.

In all test cases with NR, the NR frequency used shall be as specified in TS 38.508-1 [20], clauses4.3.1.0A, 4.3.1.1 and 4.3.1.2. The NR frequency with respect to the GNSS carrier frequency shall be offset by + 0.025 PPM.

< End of changes >