3GPP TR 38.887 V0.3.0 (2019-11)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

New frequency range for NR (39.5-43.5 GHz)

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Radio, LTE

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Introduction	0

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Background	6

4.1        Regulatory situation	6

5	NR Frequency band definition	6

6          System Parameters	6

7	Channel numbering and channel bandwidth	6

8	RF requirements for band n259	6

8.1	UE specific	6

8.1.1	Transmitter characteristics	6

8.1.2	Receiver characteristics	6

8.2	BS specific	6

8.2.1	Radiated transmitter characteristics	6

8.2.2	Radiated receiver characteristic	6

9	    Required changes to NR, E-UTRA, UTRA and MSR specifications	7

Annex <A> (normative): <Normative annex title>	8

Annex B: Change history	9

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for Work Item on New Radio (NR) Access Technology, covering the new frequency range between 39.5- 43.5 GHz for NR.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

BS	Base Station

BW	Bandwidth

EIRP	Effective Isotropic Radiated Power

EIS	Equivalent Isotropic Sensitivity

FR	Frequency Range

GSCN	Global Synchronization Channel Number

ICS	In-Channel Selectivity

ITU-R	Radiocommunication Sector of the International Telecommunication Union

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

OTA	Over The Air

RF	Radio Frequency

RX	Receiver

SCS	Sub-Carrier Spacing

TDD	Time division Duplex

# 4	Background

WRC-19 agenda item 1.13, Resolution 238 (WRC-15) identifies 37-40.5 GHz, 42.5-43.5 GHz, which have allocations to the mobile service on a primary basis; and 40.5-42.5 GHz which may require additional allocations to the mobile service on a primary basis, among other as IMT candidate bands. Consequently, after WRC-15 there have been extensive discussions and studies on the suitability of these bands for IMT allocation in different regulatory organizations. From the WRC-19 preparations, it is evident that many administrations across all ITU regions are interested in the IMT allocation of the whole or part of the 37-43.5 GHz frequency ranges. The position of regulatory bodies and regional forums regarding these ranges are summarized below:

Mobile Primary allocation

No Mobile allocation

APT (Asia-Pacific Telecommunity) members support identification of the frequency bands 37-40.5 GHz, 40.5-42.5 GHz and 42.5-43.5 GHz, or portions thereof, for IMT considering that protection of the incumbent services in these and adjacent frequency bands should be ensured. APT Members recognize that different administrations would implement IMT in different portions of the 37-43.5 GHz frequency range for IMT, and a global identification for IMT in the 37-43.5 GHz band, or portions thereof, would allow each country/region to implement IMT in different portions of the band in accordance with their national/regional considerations, while still facilitating the benefits of economies of scale.

ASMG (Arab Spectrum Management Group): supports IMT identification within the frequency bands 40.5-42.5GHz and 42.5-43.5GHz.

CEPT (European Conference of Postal and Telecommunications): proposes an IMT identification for 40.5-43.5 GHz. This is a priority band for CEPT and already specified for future harmonization in Europe. Whilst CEPT will not propose IMT identification at WRC-19 and has no intention of using 37-40.5 GHz for IMT, CEPT will not oppose a global IMT identification for the full 37-43.5 GHz range.

Some countries in CITEL including the US, Canada and Brazil support IMT identification for part of 37-43.5 GHz spectrum range.

Position of the RCC (Regional Commonwealth in the field of Communications) administrations on frequency bands 37.0−40.5 GHz and 40.5−42.5 GHz included in Resolution 238 (WRC-15): to be specified taking into account the need to protect both passive and active services.

ATU (African Telecommunications Union) supports the 40GHz band (37-40.5, 40.5-42.5 and 42.5-43.5 GHz frequency range) for IMT identification under Resolution 238 (WRC15).

## 4.1        Regulatory situation

Table 4.1-1 is the extract of the Radio Regulation (RR) table of allocations providing the services allocated in the 36-47 GHz frequency range.

Table 4.1.1 Allocation information in the 36-47GHz frequency range [2]

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 36-37 EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) 5.149 5.550A |  |  |
| 37-37.5 FIXED MOBILE except aeronautical mobile SPACE RESEARCH (space-to-Earth) 5.547 |  |  |
| 37.5-38 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile SPACE RESEARCH (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| 38-39.5 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| 39.5-40 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 | 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) 5.516B MOBILE BROADCASTING BROADCASTING-SATELLITE Mobile Mobile-satellite (space-to-Earth) 5.547 | 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 |
| 41-42.5 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 5.551F 5.551H 5.551I |  |  |
| 42.5-43.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 MOBILE except aeronautical mobile RADIO ASTRONOMY 5.149 5.547 |  |  |
| 43.5-47 MOBILE 5.553 MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE 5.554 |  |  |

# 5	NR Frequency band definition

The Band plan for 37-43.5 GHz frequency range is shown in Figure below. Two bands are defined in this range, existing band n260 (37-40 GHz) and the new band n259 (39.5-43.5 GHz).

Band definition in the frequency range 37- 43.5 GHz

Summary of the new NR band is provided in Table 5-1.

Table 5-1: New NR band in FR2

| Band number | UL | DL | Duplex mode |
| --- | --- | --- | --- |
| n259 | 39.5 – 43.5 GHz | 39.5 - 43.5 GHz | TDD |

# 6          System Parameters

<Text to be added>

# 7	Channel numbering and channel bandwidth

<Text to be added>

# 8	RF requirements for band n259

## 8.1	UE specific

<Text to be added>

### 8.1.1	Transmitter characteristics

<Text to be added>

### 8.1.2	Receiver characteristics

<Text to be added>

## 8.2	BS specific

Based on current BS specification, at least the following requirements needed to be considered for introduction of Band n259.

Table 8.2-1: Required changes in TS 38.104 for n259

| Section | Requirement | Required changes in TS 38.104 |
| --- | --- | --- |
| 5.2 | Operating bands | Operating band of n259 shall be added in to Table Table 5.2-2. |
| 5.3.5 | BS channel bandwidth per operating band | Channel bandwidths and corresponding SCS for n259 shall be added in Table 5.3.5-2. |
| 5.4.2.3 | Channel raster entries for each operating band | Applicable NR-ARFCN for n259 shall be added into Table 5.4.2.3-2 once the lower frequency limit for n259 is determined. |
| 5.4.3.3 | Synchronization raster entries for each operating band | Applicable SS raster entries for n259 shall be added into Table 5.4.3.3-2 once the lower frequency limit for n259 is determined. |

### 8.2.1	Radiated transmitter characteristics

<Text to be added>

### 8.2.2	Radiated receiver characteristic

<Text to be added>

# 9	    Required changes to NR, E-UTRA, UTRA and MSR specifications

The required changes to the 3GPP specifications for the NR band n259 are summarised in a Table 8-1.

Table 8-1: Overview of 3GPP specifications with required changes

| Affected existing specifications |  |  |  |
| --- | --- | --- | --- |
| Spec No. | Subject of the CR | Comments | CR/TP (Tdoc) |

The required changes to the 3GPP specifications for the NR band n259 are summarised in a Table 8-2.

Table 8-2: Overview of 3GPP specifications with required changes

| Affected existing specifications |  |  |  |
| --- | --- | --- | --- |
| Spec No. | Subject of the CR | Comments | CR/TP (Tdoc) |

Annex <A> (normative):
<Normative annex title>

Annexes are only to be used where appropriate:

Annex B:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 04/2019 | RAN4#90bis | R4-17xxxx |  |  | TR skeleton | N/A | 0.0.0 |
| 05/2019 | RAN4#91 | R4-1907101 |  |  | TP on General issues |  | 0.1.0 |
| 05/2019 | RAN4#91 | R4-1906861 |  |  | On frequency range for NR band n259 |  | 0.1.0 |
| 05/2019 | RAN4#91 | R4-1907786 |  |  | Regulatory situation in 37-43.5 GHz frequency range |  | 0.1.0 |
| 08/2019 | RAN4#92 | R4-1909915 |  |  | BS RF requirements |  | 0.2.0 |
| 11/2019 | RAN4#92bis | R4-1912187 |  |  | Band plan for NR band 259 |  | 0.3.0 |